3GPP TSG-RAN WG2 Meeting # 129bis 	                R2-2502490

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.6.2

Source: 	Sony

Title:	LTM Resource consumption for the target cells

Document for:	Discussion and Decision

# Introduction

In RAN#102, a new WI of mobility enhancements for NR has been agreed which contains the following bullet point [1]:

- Specify support for inter-CU Layer 2 Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured.

  - As secondary priority, support the case when NR-DC is configured, and CU is acting as SN and MCG is unchanged.

  - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released.

    - Note: The case that LTM is configured in both MCG and SCG is excluded

  - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

    - Coordination with SA3 needed with respect to security key handling.

  - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In RAN2#126, the following agreements were reached:

Agreements on inter-CU LTM:

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

In RAN2#127, the following agreements were reached:

Agreements on inter-CU LTM

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

In this contribution, we discuss how to reduce the resource consumption (e.g., configured grant) for the candidate target cells for both intra-CU and inter-CU scenarios.

## 2. LTM Resource consumption for the target cells

In Rel-18 LTM, resources (e.g., configured grant) for the target cells are allocated in the RRC reconfiguration messages. As the resources are pre-allocated for multiple candidate cells even if they are not used, there is an issue of resource consumption (i.e., allocated but not used at the end). Hence, how to minimize the resource consumption is an issue to be solved for both intra-CU and inter-CU deployment scenarios.

In the current LTM procedure, the resources (e.g., configured grant) for the target cell are allocated in the RRC reconfiguration message (Step 2 of Figure 4 below). For example, in order for a UE to switch to the target cell, the UE applies configured grant (CG) resources to transmit the first message (e.g., data) to indicate its arrival at the target cell where the CG resources are pre-allocated for each target cell in step 2 (Figure 4).

Figure 4. Rel-18 LTM procedure (Figure 9.2.3.5.2-1. Signalling procedure for LTM) [2]

In order to minimize the resource consumption, the resources that are allocated in step 2 can be deactivated by default, and then when the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM command to the UE for a target cell (for example index 1), i.e., the resources are activated only for that target cell (index 1). This means that activation message should be sent from the serving cell to the target cell only.

Proposal 1: RAN2 to discuss how minimize the resource consumption (e.g., configured grant) for the candidate target cells.

Proposal 2: CG resources of the target cell should be activated only when the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM command to the UE for the target cell. The activation message should be sent from the serving cell to the target cell only.

## Conclusions

In this contribution, we have discussed how to reduce the resource consumption (e.g., configured grant) for the candidate target cells for both intra-CU and inter-CU scenarios. We have the following proposals.

Proposal 1: RAN2 to discuss how minimize the resource consumption (e.g., configured grant) for the candidate target cells.

Proposal 2: CG resources of the target cell should be activated only when the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM command to the UE for the target cell. The activation message should be sent from the serving cell to the target cell only.

## References

[1] RP-234036, “New WID: NR mobility enhancements Phase 4,” RAN#102.

[2] TS 38.300 V18.0.0, Section 9.2.3.5 (Figure 9.2.3.5.2-1. Signalling procedure for LTM).