3GPP TSG-RAN WG2 Meeting # 129bis 	                R2-2502496

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.12.2

Source: 	Sony

Title:	Enhancement for Asymmetric DL sTRP/UL mTRP

Document for:	Discussion and Decision

# Introduction

In RAN#105, revised WI of NR MIMO Phase 5 has been agreed which contains the following bullet point from [1]:

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2:

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources.

RAN1#116bis Agreement:

Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1.

- Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification.

- Above is subject to a separate UE capability signaling.

RAN1#117 Agreement:

Introduce a new RRC parameter per BWP/CC to indicate that two separate SRS CLPC adjustment states are configured for SRS in a BWP/CC.

RAN1#117 Agreement:

For the association between PL offset and joint/UL TCI state, support the following:

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

RAN1#117 Agreement:

For the asymmetric DL sTRP/UL mTRP scenarios, study whether/how to consider PL offset in PHR calculation, including Type 1 PHR based on actual PUSCH transmission, Type 1 PHR based on reference PUSCH, Type 3 PHR based on actual SRS and Type 3 PHR based on reference SRS.

RAN1#118 Agreement:

For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH.

RAN1#118 Agreement:

Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states.

- Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

RAN1#118-bis Agreement:

The lower limit of the value range of PL offset is extended to -12 dB.

RAN1#118-bis Agreement:

Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

  - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

- The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA

- The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA.

- Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA

  - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped.

- FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

RAN1#118-bis Agreement:

For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0:

- This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset.

- When one joint/UL TCI state is indicated in Rel-17 unified TCI,

  - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation

  - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power.

- FFS: Whether the bit field can be used to indicate other information

- FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI

R1-2408997	Summary #3 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

RAN1#118-bis Agreement:

For indicating PL offset for PDCCH-order PRACH, when two joint/UL TCI states are indicated in Rel-18 unified TCI, down-select one from the following Alts for the 1-bit DCI field in DCI 1_0 which indicates the application of PL offset on PRACH transmission:

- Alt2:

  - the bit field index 0 of this field indicates that the PL offset associated in the first indicated joint/UL TCI state is included in the PRACH transmission power calculation

  - the bit field index 1 of this field indicates that the PL offset associated in the second indicated joint/UL TCI state is included in the PRACH transmission power calculation.

  - FFS: Whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed.

  - FFS: If other information can be indicated by this same 1-bit DCI field for the PDCCH-order PRACH transmission

RAN2#128 Agreement:

- New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

- Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB.

- In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

RAN2#129 Agreement:

- One PL offset value is indicated for each TCI state included in the new MAC CE.

- The new MAC CE contains one serving cell ID and one BWP ID

- TCI state ID is used to indicate a TCI state in the new MAC CE (i.e., no bitmap for TCI states is needed)

- The new MAC CE can include flexible number of PL offset values.

- Working assumption: UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found.

- RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

In this contribution, we discuss some remaing enhancements for asymmetric DL sTRP/UL mTRP deployment scenarios and provide some proposals at the end.

## Enhancement for Asymmetric DL sTRP/UL mTRP

In commercial NR deployment [2], it is identified that in some dense macro cell operation, UL traffic is a bottleneck. Nevertheless, it is well-known that a HetNet deployment may offload some UL traffic from the macro-gNB to the small cells. Hence, the small cells are also beneficial for UE in the cell edge performing uplink transmission as the UE can use a lower transmission power thereby reducing UL interferences. However, for cases where UL traffic is the issue rather than the DL, the legacy HetNet deployment will introduce DL interferences among the TRPs in a HetNet cell. A deployment scenario of asymmetric DL sTRP/UL mTRP could help in this case. An example of deployment scenario for asymmetric DL sTRP/UL mTRP is shown in Figure 1. It is assumed that UL TRP, which is capable of UL reception only and operates as UL-only cell, is deployed within the coverage of macro cell TRP which provides both DL and UL, and its coverage is overlapped with the macro cell TRP because the UL TRP does not provide any DL signals. A UE transmit to the UL TRP and receives DL signal from the macro cell TRP as shown in Figure 1.

Figure 1: Example of asymmetric DL sTRP/UL mTRP deployment scenario

DL sTRP should be considered in the deployment scenario because the UL TRP does not provide any DL signal/channel including CORESET(s) for a UE. To operate DL sTRP, an ideal fronthaul connection is established between the macro cell TRP and each of UL TRPs. In addition, to simplify the scenario, the UE should support and be configured with only one UL-only TRP at a given time (in addition to the legacy macro TRP), and from system perspective a UE should initially receive information about the existence of the UL-only TRPs in the cell from the network via system information (SIB) or RRC signalling.

Furthermore, if a UE is close to macro-cell, it is not reasonable to configure an UL-only TRP. Therefore, how to determine if a UE is close to anyone of the UL-only TRPs is important. In practice, if the UE transmits some signals (like SRS) and these signals are received on both macro TRP and UL-only TRP, the network can determine whether the UE is close to an UL-only TRP. Subsequently, the network can schedule PDCCH order to the UE to transmit PRACH towards the UL-only TRP to stablish synchronisation and pathloss.

Alternatively, it may be also reasonable that the UE discovers if it is close to the UL-only TRP coverage area by measuring DL RSRP of the downlink TRP and comparing against a pre-configured RSRP threshold. If the DL RSRP is below to the RSRP threshold, the UE assumes the signal toward macro TRP is very weak and must look for an UL-only TRP. In this case, the UE may try to transmit SRS or PRACH periodically with beam sweeping toward UL-only TRP.

Proposal 1: For asymmetric DL sTRP/UL mTRP, the existence of the UL-only TRPs in the cell should be configured from the network via system information or dedicated signalling.

Proposal 2: For asymmetric DL sTRP/UL mTRP, a UE discovers if it is close to the UL-only TRP coverage area by measuring DL RSRP of the downlink TRP and comparing against a pre-configured RSRP threshold.

## Initial Pathloss offset determination and PHR reporting

For initial pathloss offset determination based on network implementation, one option is to use SRS received power by single SRS transmission from the UE. As shown in Figure 2, since both DL TRP and UL TRP can receive the same SRS signal from the same resource (when FR1) and these TRPs can exchange information of the received power of the SRS through backhaul, the network can calculate DL TRP received power of the SRS – UL TRP received power of the SRS = pathloss offset. Then, network could indicate the calculated initial pathloss offset to a UE so that the UE can obtain estimated pathloss between UL TRP and the UE.

Figure 2: Pathloss offset calculation method by the network implementation.

The concern of this option is the PL offset calculation accuracy especially for FR2. In an environment such as FR2, it would be difficult the SRS to reach both DL TRP and UL TRP using the same resource and the same Tx beam (as transmissions are beamformed). Therefore, SRS resource has to be changed between DL TRP and UL TRP, but SRS transmit power may also be changed. In that case, the pathloss offset would not be accurate, which should be problematic.

Observation 1: Calculating initial pathloss offset by using single SRS reception from the same SRS resource may not be accurate especially for FR2 because SRS from the same resource may not reach both DL sTRP and UL mTRP due to different Tx beam, as a result the SRS resource needs to be separated between DL sTRP and UL mTRP which may result in changing SRS transmission power.

Type 3 PHR reporting would be useful to indicate SRS transmission power to gNB. By knowing actual SRS transmission power, network can calculate more accurate initial pathloss offset compared to a method employing a network implementation. In the detail, network can calculate SRS transmit power (indicated through Type 3 PHR) – SRS received power = pathloss for both DL TRP and UL TRP. Then, network can calculate initial pathloss offset = pathloss of the DL TRP - pathloss of the UL TRP.

In current specification, Type 3 PHR reporting is only used in a serving cell where PUSCH is not configured. Therefore, we prefer to support Type 3 PHR reporting for asymmetric DL sTRP/ UL mTRP deployment scenario as well. Considering the use case (indicating actual SRS transmission power to gNB), at least Type 3 PHR based on actual SRS should be supported.

Proposal 3: Support Type 3 PHR reporting in a serving cell configured with UL-only TRP.

## Conclusions

In this contribution, we have discussed some enhancements for asymmetric DL sTRP/UL mTRP deployment scenarios and we have the following observation and proposals.

Proposal 1: For asymmetric DL sTRP/UL mTRP, the existence of the UL-only TRPs in the cell should be configured from the network via system information or dedicated signalling.

Proposal 2: For asymmetric DL sTRP/UL mTRP, a UE discovers if it is close to the UL-only TRP coverage area by measuring DL RSRP of the downlink TRP and comparing against a pre-configured RSRP threshold.

Observation 1: Calculating initial pathloss offset by using single SRS reception from the same SRS resource may not be accurate especially for FR2 because SRS from the same resource may not reach both DL sTRP and UL mTRP due to different Tx beam, as a result the SRS resource needs to be separated between DL sTRP and UL mTRP which may result in changing SRS transmission power.

Proposal 3: Support Type 3 PHR reporting in a serving cell configured with UL-only TRP.

## References

[1] RP-242394, “NR MIMO Phase 5,” RAN#105.

[2] RWS-230248, “Views on Rel-19 MIMO/UL enhancements,” NTT DOCOMO, 3GPP TSG RAN Rel-19 workshop.