3GPP TSG-RAN2#129bis										                         R2-2502527

Wuhan, China, April 7th – 11st , 2025

Source: 			ZTE Corporation, Sanechips

Title: 	                  Consideration on IoT- NTN TDD mode

Agenda item:	      8.17

Document for:       Discussion and Decision

- Introduction

In RAN#106, the revised WID on introduction of IoT-NTN TDD mode[2] has been agreed to support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period[1].In the RAN2#129 meeting, some agreements has been made for supporting the IoT-NTN TDD mode.

In this contribution, we will further discuss the IoT-NTN TDD mode related RAN2 impacts and give our proposals.

- Discussion

Issue 1: Paging occasion determination

In RAN2#129 meeting, it has been agreed that RAN2 will continue studying paging aspects based on RAN1 progress[2].

In RAN1#119 meeting, the following agreements have been made[3]:

Agreement Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods.

And in RAN1#120 meeting, the following agreements have been made[4]:

Conclusion For determining the offset between the 90ms TDD structure and the 10240ms H-SFN: the UE may derive the 90ms TDD structure based on observations of downlink signals. No specification impact. Agreement The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be [48…50] ms at the ULSRP. - NOTE: This corresponds to the largest separation between DL and UL Iridium slot pairs. Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA. Agreement The downlink subframes within D=8 are fixed to: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames).

Based on the conclusion and the agreements, it can be assumed that the offset between the 90ms TDD structure and the 10240ms H-SFN start occasion(e.g. the start occasion of SFN 0) may change every H-SFN as shown in the following figure:

Figure 1. Example of the offset between the 90ms TDD structure and the 10240ms H-SFN

Considering that the downlink subframes within D=8 are fixed to [3 4 5 6 7 8 9 0] (across two consecutive radio frames) in the 90ms TDD structure, and only the downlink subframes can be used for paging occasion, the frames that can be used for paging may change every H-SHN due to the change of the offset between the 90ms TDD structure and the 10240ms H-SFN start occasion.

Observation 1: The radio frame that can be used as PF may change every H-SFN with a determined pattern for the change of the offset between the 90ms TDD structure and the 10240ms H-SFN start occasion.

To align the paging frame with the 90ms TDD structure, one of the following options can be used:

- Option 1: New paging DRX cycle with multiple 9 radio frames and radio frame offset for PF determination is defined. Based on which, with similar NR PF formula, UE and NW can determine the paging frame accurately. With this mechanism, the definition of PO and PF terminology remains the same as that defined in TS36.304, e.g.

One Paging Occasion (PO) is a subframe where there may be P-RNTI transmitted on PDCCH or MPDCCH or, for NB-IoT on NPDCCH addressing the paging message.

One Paging Frame (PF) is one Radio Frame, which may contain one or multiple Paging Occasion(s). When DRX is used the UE needs only to monitor one PO per DRX cycle.

- Option 2: Legacy NB-IoT paging parameters and PO formula are reused. When the determined paging subframe#n(e.g. PO) is not valid (i.e., is not mapped to downlink subframe), the PO is postponed to the nearest valid DL subframe#n.  Since the 90ms TDD frame pattern is fixed, both UE and NW can derive the used POs after postpone operation. The  only issue with this option 2 is that, the PF actually used is not the PF calculated in the legacy formula,  and  the PO actually used is not the PF calculated in the legacy formula, which may be not consistent with the PF/PO definition specified. However, it can be easily clarified in specs with additional explanations

- Based on above analysis, both option 1/2 works. Option 1 allows a paging mechanism that fits with 90 ms TDD frame structure but requires further discussion to handle PF/PO determination equation to comply with the shifted offset at each H-SFN.While opt2 can reuse most of existing PF/PO determination equations with further clarification on PF/PO definition. Which options to go depends on whether RAN2 intends to design a paging mechanism that fully adapted to 90ms TDD frame structure or not. It is proposed RAN2 to perform down-selection of the two options as below:

Proposal 1: RAN2 discusses and decides which of the following options is used for Paging occasion determination:

Option 1: New paging DRX cycle with multiple 9 radio frames and radio frame offset for PF determination are used to determine the PF and PO accurately.

Option 2: Legacy NB-IoT PO determination mechanism is used. When the determined paging subframe#n is not a valid downlink subframe, the PO is postponed to the nearest valid downlink subframe#n.

Issue 2: About PUR resource determination

In the current specification, the PUR resource is UE specific resource with periodicity in unit of of H-SFN duration (i.e., 10.24s) as follows:

– PUR-PeriodicityAndOffset-NB The IE PUR-PeriodicityAndOffset is used to indicate H-SFN of the first PUR occasion and periodicity of the subsequent PUR occasions. The value of periodicity is in the unit of H-SFN duration (i.e., 10.24s). Value periodicity8 corresponds to periodicity of 8 H-SFN, value periodicity16 corresponds to periodicity of 16 H-SFN and so on. The value of offset is in the unit of H-SFN duration (i.e., 10.24s). PUR-PeriodicityAndOffset-NB information element -- ASN1START PUR-PeriodicityAndOffset-NB-r16 ::= CHOICE { periodicity8 INTEGER (1..7), periodicity16 INTEGER (1..15), periodicity32 INTEGER (1..31), periodicity64 INTEGER (1..63), periodicity128 INTEGER (1..127), periodicity256 INTEGER (1..257), periodicity512 INTEGER (1..511), periodicity1024 INTEGER (1..1023), periodicity2048 INTEGER (1..2047), periodicity4096 INTEGER (1..4095), periodicity8192 INTEGER (1..8191) }

Based on the RAN1 agreements cited in previous section, it can be assumed that the offset between the 90ms TDD structure and the 10240ms H-SFN start occasion may change every H-SFN with a determined pattern. Based on existing configuration, PUR resource start subframe can align with the UL subframes every 9 H-SFNs, and there is an determined offset(with 1 radio frame, 2 radio frames,..., or 8 radio frames) between PUR resource start subframe and the UL subframes in other H-SFNs, in which case UE can postpone the PUR resource start subframe to the latest UL subframe for PUR transmission.

Proposal 2: When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the latest UL subframe for PUR transmission.

Issue 3: About cell access barring

In RAN2#129 meeting, the following agreement has been made[2]:

- Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

Although legacy UE cannot access the IoT NTN TDD cell because it does not support the IoT NTN TDD frequency band, it is still preferred to use NTN specific barred bit. Similar discussion happened when introducing cellBarred-NTN for legacy NB-IoT NTN. Considering that IoT NTN TDD is also a NTN system and all the NTN related enhancements will apply here, using cellBarred-NTN for IoT NTN TDD cell barring is more future proof. Moreover, it has been specified in RRC specs  that  EUTRA always includes cellBarred-NTN and sets cellBarred to 'barred' in an NTN cell, using cellBarred-NTN for IoT NTN TDD cell barring keep the same behaviour for NTN UE in barring bit check without more specs update. Therefore, it is preferred to use cellBarred-NTN for IoT NTN TDD cell barring.

Proposal 3: use cellBarred-NTN for IoT NTN TDD cell barring.

Issue 4: Impact on higher layer Timers

In the legacy NB-IoT TDD, only Uplink-downlink configuration 1 to 5 are supported[5], which means that in one system frame at least 4 subframes can be used for DL transmission (configuration 1), and at least 1 subframe can be used for UL transmission. That means at least 36 subframes can be used for DL transmission in 90ms, and at least 9 subframes can be used for UL transmission in 90ms.

Uplink-downlink configurations for legacy NB-IoT TDD

| Uplink-downlink configuration | Downlink-to-Uplink Switch-point periodicity | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D |

While in IoT NTN TDD with 90ms TDD structure, 8 subframes can be used for DL transmission and 8 subframes can be used for UL transmission in 90ms, the number of subframes used for DL transmission in IoT NTN TDD may be decreased to 1/4 of the number of subframes used for DL transmission in legacy NB-IoT TDD system. With the same coverage enhancement(e.g. at most 2048 repetitions for DL repetition), the DL transmission time in IoT NTN TDD may 4 times of the DL transmission time in IOT NTN TDD.

Observation 2: With the same coverage enhancement(e.g. at most 2048 repetitions for DL repetition), the DL transmission time in IoT NTN TDD may be 4 times of the DL transmission time in IOT NTN TDD.

Before Rel-15, only FDD is supported in NB-IoT. In Rel-15, TDD is introduced for NB-IoT, the timer extension for supporting TDD has been discussed, and the following agreement have been made[6][7]:

- T300 and T301 are extended to include 80s, 100s, 120s for TDD mode. - Reuse the value range of T310 in FDD mode for TDD mode. - T311 is extended to include 160s, and 200s for TDD mode. - Reuse the value range of discardTimer in FDD mode for TDD mode. - Extend T-PollRetransmit to 300s for TDD mode. - Reuse the value range of DataInactivityTimer in FDD mode for TDD mode. - ra-ResponseWindowSize and mac-ContentionResolutionTimer are extended to 20.48s for TDD mode. - For TDD mode, the value of ra-ResponseWindowSize and mac-ContentionResolutionTimer considered by the UE is Min (signaled value x PDCCH period, 20.48s). - Reuse the value range of timers defined with pp units in FDD mode for TDD mode, except for ra-ResponseWindowSize and mac-ContentionResolutionTimer.

In which, “ra-ResponseWindowSize and mac-ContentionResolutionTimer are extended to 20.48s for TDD mode.” is to extend the maximal absolute value limitation as following[8], the configured value range of ra-ResponseWindowSize and mac-ContentionResolutionTimer with unit of pp(PDCCH periods) is not extended.

mac-ContentionResolutionTimer Timer for contention resolution in TS 36.321 [6]. Value in PDCCH periods. Value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on. mac-ContentionResolutionTimer-r15 is only applicable for EDT. UE performing EDT shall use mac-ContentionResolutionTimer-r15, if present. For FDD: The value considered by the UE is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 10.24s). For TDD: The value considered by the UE is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 20.48s).

ra-ResponseWindowSize Duration of the RA response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods and so on. For FDD: The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s). For TDD: The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 20.48s).

Generally, in NB-IoT system, there are two type of timers as follows:

- Timer with unit of PP(PDCCH period): consider the PDCCH period is defined as “the interval between the start of two consecutive PDCCH occasions and depends on the currently used PDCCH search space, as specified in TS 36.213. A PDCCH occasion is the start of a search space and is defined by subframe k0 as specified in clause 16.6 of TS 36.213. The calculation of number of PDCCH-subframes for the timer configured in units of a PDCCH period is done by multiplying the number of PDCCH periods with npdcch-NumRepetitions-RA when the UE uses the common search space or by npdcch-NumRepetitions when the UE uses the UE specific search space”[7]. Since only downlink subframes will be calculated as PDCCH-subframes (For NB-IoT UE, all subframes that are part of the NPDCCH search space represent PDCCH-subframes among all NB-IoT downlink subframes, including those which the UE is not required to monitor as specified in clause 16.6 of TS 36.213)[5], e.g. the “G” subframes and “U” subframes will not be countered for the timer. Thus, the value range of timer with unit of PP is enough, e.g. not impacted by the IoT-NTN TDD pattern.

Observation 3: Only DL subframes will be counted towards timers with length unit of PP (PDCCH period), therefore the existing value range of timers with unit of pp are enough for IoT-NTN TDD, and no extension is needed.

Proposal 4: RAN 2 confirms that the existing value range of timers with unit of pp are enough for IoT NTN TDD.

Considering that ra-ResponseWindowSize and mac-ContentionResolutionTimer has the maximal absolute value limitation(e.g. 10.24s or 20.48s), if the IoT NTN TDD supports the same coverage enhancement level as legacy(e.g. at most 2048 repetitions for DL repetition), the maximal absolute value limitation(e.g. 10.24s for FDD or 20.48s for TDD) may need to be extended to 81.92s.

Observation 4: If the IoT NTN TDD supports the same coverage enhancement level as legacy, the maximal absolute value limitation of ra-ResponseWindowSize and mac-ContentionResolutionTimer (e.g. 10.24s for FDD or 20.48s for TDD) may need to be extended to 81.92s(e.g. 4 times of legacy TDD value).

- Timer with unit of ms or s(milliseconds or seconds): When determine the value range of rel-15 NB-IoT TDD timer with unit of ms or s, the maximal PUSCH repetition number(e.g. 128 repetitions) , the maximal PDCCH repetition number(e.g. 2048 repetitions) and the maximal PDSCH repetition number(e.g. 2048 repetitions) are considered, if coverage enhancement(e.g. transmission with repetitions) is not considered in IoT-NTN TDD pattern with a period of 9 radio frames(e.g. the repetition number of all channels will be 1), the existing value range of Timer with unit of ms or s is enough for IoT-NTN TDD; but if IoT NTN TDD supports same coverage enhancement level(e.g. UL with maximal 128 repetitions and DL with maximal 2048 repetitions), some of the existing value range of Timer with unit of ms or seconds(e.g. T300, T301, T311,T-PollRetransmit) will be insufficient and need to be extended(e.g. extended to 4 times of legacy TDD value).

Observation 5: If IoT NTN TDD supports same coverage enhancement level(e.g. UL with maximal 128 repetitions and DL with maximal 2048 repetitions), some of the existing value range of Timer with unit of ms or seconds(e.g. T300, T301, T311,T-PollRetransmit) will be insufficient and need to be extended(e.g. extended to 4 times of legacy TDD value).

In RAN2#129 meeting, the following agreement has been made[2]:

=>RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system.

Based on the current RRC specification[8], MIB transmission reliability has been considered for coverage enhancement case, e.g. MIB uses a fixed schedule with a periodicity of 640 ms and repetitions made within 640 ms, and the transmissions are arranged in 8 independently decodable blocks of 80 ms duration. UE can decode the MIB if it can receive any one of 64 MIB blocks in 640ms.

But in RAN1#102 meeting, the following agreement has been made[4],

Agreement NPSS/NSSS/NPBCH transmissions are dropped in non-D NB-IoT subframes - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

Dropped some NPBCH repetition may lead the MIB reliability impact in coverage enhancement case. With the limitation of NPBCH coverage, the legacy coverage enhancement level(e.g. UL with maximal 128 repetitions and DL with maximal 2048 repetitions) can not be provided in IoT NTN TDD.

Observation 6: Based on the RAN1 agreement, the legacy coverage enhancement level(e.g. UL with maximal 128 repetitions and DL with maximal 2048 repetitions) can not be provided in IoT NTN TDD.

Considering that whether to extend the timer (e.g. in observation 6 and 7) depends the coverage enhancement level(e.g. maximal repetitions) supported, it is suggested to ask RAN1 about more input on this aspect.

Proposal 4a: RAN2 send LS to consult RAN1 on coverage enhancement level(e.g. maximal repetitions) that can be supported in IoT-NTN TDD system.

Proposal 4b: RAN2 postpone the decision on extending the value range of timer with unit of milliseconds or seconds until receiving RAN1’s response on coverage enhancement level(e.g. maximal repetitions) that can be supported in IoT-NTN TDD system.

Issue 5: About SI scheduling impact

In RAN1#120 meeting[4], the following conclusion has been made:

Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

In the current specification, the SIs are scheduled by SIB1 based on the following time domain related parameters:

si-WindowLength-r13 ENUMERATED {ms160, ms320, ms480, ms640, ms960, ms1280, ms1600, spare1}, si-RadioFrameOffset-r13 INTEGER (1..15) OPTIONAL, -- Need OP SchedulingInfo-NB-r13::= SEQUENCE { si-Periodicity-r13 ENUMERATED {rf64, rf128, rf256, rf512, rf1024, rf2048, rf4096, spare}, si-RepetitionPattern-r13 ENUMERATED {every2ndRF, every4thRF, every8thRF, every16thRF}, ... }

Considering there is only 8 consecutive DL radio subframes in 90ms TDD structure, the si-RepetitionPattern with every2ndRF, every4thRF or every8thRF will lead the SI repetition useless(e.g. SI repetition in non-DL subframes), which need to be enhanced to include value range with multiple 90ms, e.g. with value range of every9thRF, every18thRF, every36thRF etc.

Proposal 5: Define value range with multiple 90ms for si-RepetitionPattern, e.g. with value range of every9thRF, every18thRF, every36thRF etc.

Based on the RAN1 agreements,  the offset between the 90ms TDD structure and the 10240ms H-SFN start occasion may change every H-SFN with a determined pattern, the si-RadioFrameOffset should also be adjusted dynamically so that the SI window starts at DL subframe. Since the change of offset between the 90ms TDD structure and the 10240ms H-SFN start occasion has fixed pattern(e.g. shown in figure 1), the change of si-RadioFrameOffset also has the fixed pattern, UE can deduce the si-RadioFrameOffset in the next H-SFN based on the si-RadioFrameOffset received in previous H-SFN. Similar as the hyperSFN-MSB-r13 in SIB 1, the change of the si-RadioFrameOffset due to SFN wraparound and/or H-SFN wraparound should not impact the value of systemInfoValueTag-r13 in MIB.

Proposal 6: The value of si-RadioFrameOffset updates dynamically every H-SFN with determined pattern, and the change of the offset due to SFN wraparound and/or H-SFN wraparound should not impact the value of systemInfoValueTag-r13 in MIB.

Except the si-RadioFrameOffset, si-Periodicity is also used to determine the SI window start occasion. So that the SI window can always start at DL subframe, new value range with multiple 90ms, e.g. rf63, rf126, rf252 etc should be defined for si-Periodicity.

Proposal 8: Define value range with multiple 90ms for si-Periodicity, e.g. rf63, rf126, rf252 etc.

When supporting TDD for NB-IoT in Rel-15, si-WindowLength has not been extended. Considering that maximal value of 1600ms is long enough to successfully decode one SI message, the si-WindowLength does not need be extended.

Observation 7: si-WindowLength extension is not necessary for IoT NTN TDD.

Issue 6: About SC-PTM impact

In SC-PTM, several timers and periodicity are used. But considering that SC-PTM are transmitted on common channel, e.g. SC-MCCH and SC-MTCH, and the control information associated with transmission of SC-PTM are transmitted in SIB-20, which can be transmitted periodically, UE can drop the non-DL subframes for SC-PTM reception .

Proposal 7: UE drops the non-DL subframes for SC-PTM reception.

Issue 7: Impact on UL SPS transmission

In NB-IoT, uplink SPS is only used for BSR transmission. In case SPS transmission is dropped, SR procedure and/or dynamic scheduling can still be used for BSR transmission, and the service performance will not be impacted too much. Therefore dropping the SPS transmission when the UL SPS overlaps with non-U NB-IoT subframes is feasible.

Proposal 8: UL SPS transmission is dropped when the UL SPS overlaps with non-U NB-IoT subframes.

Issue 8: About implications of Kmac extension

In RAN2#129 meeting, the following agreement has been made[2]:

2. RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed.

Considering that Kmac is extended by 1 bit to cover the case that the round trip time between RP (Reference Point) and eNB is over 512ms(e.g. from 513ms to 1024ms), the increasing of round trip time between RP and eNB may increase the delay between UE and gNB, similar as observation  9 and 10, value range of some timers may need be extended by 4 times if legacy coverage enhancement level(e.g. UL with maximal 128 repetitions and DL with maximal 2048 repetitions) is supported with increased K-mac. But based on observation 11, the coverage enhancement level(e.g. maximal repetitions) may be less than the legacy, with assumption that maximal repetitions will be decreased(e.g. decreased by half), the value range of timers need not be extended while increasing K-mac value range for IoT NTN TDD.

Proposal 9a: RAN2 assumes Kmac extension by 1 bit has no other implications (e.g. does not impact the value range of Uu timers).

Considering that k-Mac-r17 is an optional IE, similar as legacy for parameter value range extension, a new parameter with higher value range(e.g. k-Mac-r18) is introduced for k-Mac value range extension.

Proposal 9b: A new parameter with higher value range(e.g. k-Mac-r18) is introduced for k-Mac value range extension.

Issue 9: Other remaining issues

RAN1 has made the following agreements in RAN1#120 meeting[4]

Agreement NPSS/NSSS/NPBCH transmissions are dropped in non-D NB-IoT subframes - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - FFS: handling for SIB1-NB. Agreement The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications - NPUSCH is mapped as per current specifications. - NOTE: “non-U NB-IoT subframes” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the TDD structure. Agreement The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications. - FFS: additional NB-IoT TDD-specific rules for handling non “NB-IoT DL subframes” - FFS: Introduction of new periodicities for NPDCCH monitoring (e.g. by the introduction of scaling factor) - FFS: whether all the configuration values for NPDCCH are valid for NB-IoT NTN TDD. - FFS: handling of DL transmission gap - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. Agreement For NPRACH, further consider the two options until RAN1#120bis: - Option 1: New NPRACH periodicities (multiple of 90ms) are introduced for NB-IoT NTN TDD. - Option 2: NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s) - FFS: The unit of postponement (e.g. PRU) Agreement NPRACH format 2 is not supported in NB-IoT NTN TDD. Working assumption Uplink transmission gaps do not apply in NB-IoT NTN TDD.

Since these issues are more relevant to RAN1, RAN2 can wait for more progress for these issues in RAN1 to avoid duplicated work in RAN1 and RAN2..

Proposal 10: Below issues can wait for more progress in RAN1:

- Enhancements on NPSS/NSSS/NPBCH and SIB1-NB reception

- RACH adaptions caused due to ROs overlapped with non-U NB-IoT subframes (e.g., RA-RNTI calculation)

- NPDCCH monitoring.

- Conclusion

The following observations and proposals are made:

Observation 1: The radio frame that can be used as PF may change every H-SFN with a determined pattern for the change of the offset between the 90ms TDD structure and the 10240ms H-SFN start occasion.

Observation 2: With the same coverage enhancement(e.g. at most 2048 repetitions for DL repetition), the DL transmission time in IoT NTN TDD may be 4 times of the DL transmission time in IOT NTN TDD.

Observation 3: Only DL subframes will be counted towards timers with length unit of PP (PDCCH period), therefore the existing value range of timers with unit of pp are enough for IoT-NTN TDD, and no extension is needed.

Observation 4: If the IoT NTN TDD supports the same coverage enhancement level as legacy, the maximal absolute value limitation of ra-ResponseWindowSize and mac-ContentionResolutionTimer(e.g. 10.24s for FDD or 20.48s for TDD) may need to be extended to 81.92s(e.g. 4 times of legacy TDD value).

Observation 5: If IoT NTN TDD supports same coverage enhancement level(e.g. UL with maximal 128 repetitions and DL with maximal 2048 repetitions), some of the existing value range of Timer with unit of ms or seconds(e.g. T300, T301, T311,T-PollRetransmit) will be insufficient and need to be extended(e.g. extended to 4 times of legacy TDD value).

Observation 6: Based on the RAN1 agreement, the legacy coverage enhancement level(e.g. UL with maximal 128 repetitions and DL with maximal 2048 repetitions) can not be provided in IoT NTN TDD.

Observation 7: si-WindowLength extension is not necessary for IoT NTN TDD.

Proposal 1: RAN2 discusses and decides which of the following options is used for Paging occasion determination:

Option 1: New paging DRX cycle with multiple 9 radio frames and radio frame offset for PF determination are used to determine the PF and PO accurately.

Option 2: Legacy NB-IoT PO determination mechanism is used. When the determined paging subframe#n is not a valid downlink subframe, the PO is postponed to the nearest valid downlink subframe#n.

Proposal 2: When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the latest UL subframe for PUR transmission.

Proposal 3:  cellBarred-NTN is used for IoT NTN TDD cell barring.

Proposal 4: RAN 2 confirms that the existing value range of timers with unit of pp are enough for IoT NTN TDD.

Proposal 4a: RAN2 sends LS to consult RAN1 on coverage enhancement level (e.g. maximal repetitions) that can be supported in IoT-NTN TDD system.

Proposal 4b: RAN2 postpone the decision on extending the value range of timer with unit of milliseconds or seconds until receiving RAN1’s response on coverage enhancement level(e.g. maximal repetitions) that can be supported in IoT-NTN TDD system.

Proposal 5: Define value range with multiple 90ms for si-RepetitionPattern, e.g. with value range of every9thRF, every18thRF, every36thRF etc.

Proposal 6: The value of si-RadioFrameOffset updates dynamically every H-SFN with determined pattern, and the change of the offset due to SFN wraparound and/or H-SFN wraparound should not impact the value of systemInfoValueTag-r13 in MIB.

Proposal 8: Define value range with multiple 90ms for si-Periodicity, e.g. rf63, rf126, rf252 etc.

Proposal 7: UE drops the non-DL subframes for SC-PTM reception.

Proposal 8: UL SPS transmission is dropped when the UL SPS overlaps with non-U NB-IoT subframes.

Proposal 9a: RAN2 assumes Kmac extension by 1 bit has no other implications (e.g. does not impact the value range of Uu timers).

Proposal 9b: A new parameter with larger value range(e.g. k-Mac-r18) is introduced for k-Mac value range extension.

Proposal 10: Below issues can wait for more progress in RAN1:

- Enhancements on NPSS/NSSS/NPBCH and SIB1-NB reception

- RACH adaptions caused due to ROs overlapped with non-U NB-IoT subframes (e.g., RA-RNTI calculation)

- NPDCCH monitoring.

- References

- RP-243293 Revised WID on introduction of IoT-NTN TDD mode, RAN#106

- R2-129 NR-NTN-IoT-NTN (Sergio)_ EOM

- Draft_Minutes_report_RAN1#119_v030

- Draft_Minutes_report_RAN1#120_v010

- 3GPP TS 36.211, v18.0.1(2024-04), Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation(Release 18)

- Draft_RAN2#101bis_Report_v1

- Draft_RAN2#102_Report_v1

- 3GPP TS 36.331, V18.4.0 (2024-12), Evolved Universal Terrestrial Radio Access (E-UTRA);Radio Resource Control (RRC);Protocol specification(Release 18)

- 3GPP TS 36.304, V18.3.0 (2024-12), Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode(Release 18)

- Final_Minutes_report_RAN1#118b_v100

- 3GPP T3GPP TS 36.211 V18.0.1 (2024-04), Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation(Release 18)

- 3GPP TS 23.682 V19.1.0 (2024-12), Architecture enhancements to facilitate communications with packet data networks and applications

- 3GPP TS 24.301 V19.1.0 (2024-12), Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS);Stage 3(Release 19)

- 3GPP T3GPP TS 36.321 V18.3.0 (2024-09), Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification(Release 18)