3GPP TSG-RAN WG2 Meeting #129bis	R2-2502531

Wuhan, China, 7th – 11th April 2025

Agenda item:	8.10.2.1

Source:	Samsung

Title:	Discussion on MRO enhancements

Document for:	Discussion and Decision

# Introduction

As per [1], one of the objectives of R19 SON WI is to support the MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs and subsequent CPAC. From RAN2, this involves identification and specification of necessary UE reporting to enhance the mobility parameter tuning as given below. We discuss the remaining issues for MRO for LTM and CHO with candidate SCGs in this contribution.

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## MRO for LTM

In the running RRC CRs, there are two editor’s notes for the UE capability for LTM.

Editor’s Note: FFS of the UE capability of supporting RLF-Report for LTM cell switch.

Editor’s Note: FFS of the UE capability of supporting successful handover report for LTM cell switch.

Considering that the LTM is an entirely different mobility procedure than the L3 mobility, it would be beneficial to define separate capabilities for the reports for LTM and inform them to the network. It makes sense to have separate capabilities for RLF and SHR for LTM as well.

Proposal 1: Define separate UE capability parameters for RLF report and SHR for LTM cell switch.

There is also another editor’s note as below.

Editor’s note: For RACH-less LTM cell switch in MCG, the LTM cell switch execution is considered as successfully completed when successfully sending RRCReconfigurationComplete message, thus UE can not determine the successHO-InfoAvailable in this RRCReconfigurationComplete message. FFS whether and how to fix this.

We understand that the issue in the above EN may cause problems in some corner scenarios- If there is SHR logged in two consecutive mobility and there was no ReconfigurationComplete send in between, first SHR may be lost before the network can retrieve it. A similar issue was discussed in the context of SPR also, and it was decided not to do anything specific for this case.

The issue may not be very realistic as typically there will be some Reconfiguration between two handovers. We also note that this is not specific to RACH-less LTM, as ReconfigurationComplete could be carried over msg3 during a RACH based handover or LTM and the success of handover/LTM will be declared by the UE after receiving msg4. Therefore, it is better not to have any special handling for this case.

Proposal 2: No special handling is needed for reporting successHO-InfoAvailable in RACH less LTM cell switch.

In the running CR, L1 measurements related to LTM are logged and reported in RLF report/SHR for all the candidate cells. However, LTM can be based on L3 measurements or L1 measurements. When the LTM is based on L3 measurements, LTM-SSB-Config-r18 in the LTM-Candidate-r18 will not be configured, and UE will not report L1 measurements for the LTM, based on this configuration. However, the L1 measurements may still be available at the UE as they can be used for deriving L3 measurements after applying the relevant filtering.

As UE already reports beam level measurements (L3 filtered) in the current RLF report, L1 measurements for LTM need to be reported only when the network wants to trigger to LTM based on L1 measurements. So there is no need/benefit for reporting L1 measurement results when the network configures LTM based on L3 measurements.

Observation 1: L3 filtered beam measurements are included for all the candidates in RLF report and SHR.

Proposal 3: L1 measurements for LTM are logged only for the candidates for which LTM-SSB-Config-r18 is configured (i.e. only if configured for LTM based on L1 measurements).

## MRO for CHO with Candidate SCG(s)

There is an FFS in the running CR for MRO for CHO with Candidate SCG(s).

Editor’s note: FFS lastHO-Type for CHO with Candidate SCG

If there is a handover failure when the UE executes CHO with candidate SCG(s), network will be able to identify from the existing (i.e. including R19) IEs. One scenario were setting a separate value for          lastHO-Type is useful is when the network has configured the UE with both CHO and CHO with candidate SCG(s) and an RLF has occurred immediately after the handover. However, RAN2 has not specifically agreed to consider the case where both CHO and CHO with candidate SCG(s) are configured together. Even when this is considered, we think that the network can identify the type of handover which moved the UE to the cell where the RLF occurred. For e.g. it is a reasonable assumption that the SCG configuration of the UE will be different when the UE moved to a target PCell through CHO and through CHO with candidate SCG, and we think network can identify this. Thus we do not see that lastHO-Type need to be updated for CHO with Candidate SCG.

Proposal 4: lastHO-Type is not extended with “CHO with Candidate SCG”.

# Conclusion

Based on the discussion, we have the following observation and proposals.

Proposal 1: Define separate UE capability parameters for RLF report and SHR for LTM cell switch.

Proposal 2: No special handling is needed for reporting successHO-InfoAvailable in RACH less LTM cell switch.

Observation 1: L3 filtered beam measurements are included for all the candidates in RLF report and SHR.

Proposal 3: L1 measurements for LTM are logged only for the candidates for which LTM-SSB-Config-r18 is configured (i.e. only if configured for LTM based on L1 measurements).

Proposal 4: lastHO-Type is not extended with “CHO with Candidate SCG”.

# References

[1] RP-234038: New WID on Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4