3GPP TSG RAN WG2 Meeting #129bis                          	   R2-2502541

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.5.3

Source:	Qualcomm Incorporated

Title:	Discussion on On-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

At RAN #105, it was agreed that on-demand SIB1 is specified for Case #2. The updated objects are outlined with the follows [1].

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective. - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1. - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE. - RAN1 specification impact to support this feature should be minimized

In this contribution, we further discuss design considerations for on-demand SIB1.

# Discussion

## Indication of broadcasting on-demand SIB1

At RAN2 #127bis, broadcasting on-demand SIB1 was agreed with the following agreements [2].

Agreements 7. A cell for which SIB1 request configuration is available, can periodically broadcast SIB1. 8. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Based on the above agreements, a NES cell on-demand SIB1 transmission has two modes (similar to the on-demand SI currently supported):

- Broadcasting: on-demand SIB1 is periodically broadcasted after triggered by a UL WUS.

- Not-broadcasting: on-demand SIB1 is only transmitted via UL WUS triggering.

Additionally, a UE is required to check the on-demand SIB1 transmission mode (i.e., broadcasting or not-broadcasting) after acquires an UL WUS configuration for an on-demand SIB1 transmission. Thus, an indication is needed for a UE to check the on-demand SIB1 transmission mode.

There are multiple options to indicate if the on-demand SIB1 is broadcasted or not.

Option 1: Indication in UL WUS configuration

For example, following the on-demand SI currently specified, the indicated may be included in the IE SchedulingInfo in current SIB1 as shown below.

If this is adopted for on-demand SIB1, an indication (e.g., sib1-BroadcastStatus) may be included in UL WUS configuration (e.g., like SchedulingInfo with si-BroadcastStatus for on-demand SI configuration in current SIB1).

A UE may check the indication included in an UL WUS configuration and then decide to transmit the UL WUS or not for on-demand SIB1 transmission.

When a NES cell decides to switch between “broadcasting” and “not-broadcasting”, it needs to update this indication in its UL WUS configuration and needs to transmit the updated UL WUS configuration to Cell-As or other NES cells supporting this NES cell’s UL WUS configuration. If such switching is frequent, it may cause undesired traffic among CellAs and NES cells.

Observation 1. With the indication indicated in UL WUS configuration, a UE may get the indication in the UL WUS acquired by the UE. No extra UE monitoring is needed.

Observation 2. With the indication included in UL WUS configuration, frequently switching between broadcasting and not-broadcasting causes more traffic with updated UL WUS configurations among Cell-As and NES cells which should be avoid.

Option 2: Indication in MIB

For example, an indication may be supported in MIB such as using the K_SSB.

A UE may check the K_SSB value in MIB and then decide to transmit the UL WUS or not for on-demand SIB1 transmission.

When a NES cell decides to switch between “broadcasting” and “not-broadcasting”, it needs to update this indication in its MIB and does not need to update UL WUS configuration. Thus, this option avoids the traffic among CellAs and NES cells with such switching.

Observation 3. With the indication included in MIB, there is no need to update the UL WUS configuration and thus no traffic with updated UL WUS configurations among Cell-As and NES cells.

Observation 4. With the indication indicated in MIB, a UE needs to decode the MIB before decides to transmit the UL WUS or not for on-demand SIB1.

This subject has been discussed at RAN1 multiple times, and no conclusion could be made.

Observation 5. RAN1 could not make any conclusion for on-demand SIB1 transmission state switching between broadcast and on-demand.

With the observation of option 1 and option 2, we make the following proposal.

Proposal 1. RAN2 discuss how to support on-demand SIB1 switching between broadcasting and not-broadcasting.

## UE at RRC Connected state

At RAN #106, the following conclusion was made for UE in RRC Connected state [3].

Observation 6. It was concluded at RAN #106 that RAN2 would address UE in RRC Connected state specifically for RLF case, using the same procedure as UE in RRC Idle/Inactive state.

At RAN2 #129, the following agreements were made for UE in RRC Connected state with RLF [4].

Agreements.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

2.	The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

The following case was discussed without any conclusion.

Rel-19 NES UE in RRC_CONNECTED is configured with searchSpaceSIB1, but may not receive SIB1.

There are following two possible situations for miss reception of SIB1:

Situation 1: The active BWP does not contain the searchSpaceSIB1, thus the UE does not receive the SIB1.

Situation 2: The active BWP does contain the searchSpaceSIB1, but the UE does not receive the SIB1.

In our view, if a UE is configured with searchSpaceSIB1 for acquiring SIB1, the UE is expected to use the BWP containing the searchSpaceSIB1 for SIB1 acquization. In another words, for Situation 1, the UE may follow the legacy behavior as OD-SI. However, Situation 2, where the UE fails SIB1 reception with the active BWP containing searchSpaceSIB1 due to that SIB1 has not been transmitted, can be network configuration error and it can be up to network implementation to avoid this situation. Over all, no spec work is needed.

Proposal 2. RAN2 agrees that there is no spec impact for the case where Rel-19 NES UE in RRC_CONNECTED with searchSpaceSIB1 configured but no SIB1 transmission in its active BWP.

## Feature-specific cell barring

Currently, for a cell supporting periodic SIB1 transmission, a UE can be barred from accessing the cell via the cellBarred bit in MIB and/or via feature-specific barring parameters in SIB1 for various UE capabilities (e.g., cellBarred-eRedCap2Rx for eRedCap UEs supporting 2 Rx, cellBarredNES for UEs supporting nes-CellDTX-DRX, etc.). If this legacy procedure is reused for a NES cell supporting on-demand SIB1 without further enhancements, a Rel-19 UE with a specific feature, e.g. a Rel-19 UE with eRedCap capability finds that it is barred by the feature-specific barring parameter in the on-demand SIB1 triggered by its UL WUS transmission, and then needs to acquire another NES cell’s on-demand SIB1 to find if it’s barred or not. This may cause more latency and power consumtion to this UE for NES cell reselection. A better approach could be including the feature-specific barring information in UL-WUS configuration, so that a Rel-19 UE can determine its barring status for a NES cell in advance and avoid wasting its power to transmit UL-WUS and acquire on-demand SIB1 of a barred NES cell.

Proposal 3: RAN2 discusses the indication of feature-specific cell barring for a NES cell with on-demand SIB1.

# Conclusion

In this contribution, we discussed scenarios or options for on-demand SIB1 operation and concluded with the following observations and proposals.

Indication of broadcasting on-demand SIB1

Option 1: Indication in UL WUS configuration

Observation 1. With the indication indicated in UL WUS configuration, a UE may get the indication in the UL WUS acquired by the UE. No extra UE monitoring is needed.

Observation 2. With the indication included in UL WUS configuration, frequently switching between broadcasting and not-broadcasting causes more traffic with updated UL WUS configurations among Cell-As and NES cells which should be avoid.

Option 2: Indication in MIB

Observation 3. With the indication included in MIB, there is no need to update the UL WUS configuration and thus no traffic with updated UL WUS configurations among Cell-As and NES cells.

Observation 4. With the indication indicated in MIB, a UE needs to decode the MIB before decides to transmit the UL WUS or not for on-demand SIB1.

Observation 5. RAN1 could not make any conclusion for on-demand SIB1 transmission state switching between broadcast and on-demand.

Proposal 1. RAN2 discuss how to support on-demand SIB1 switching between broadcasting and not-broadcasting.

UE at RRC connected state

Observation 6. It was concluded at RAN #106 that RAN2 would address UE in RRC Connected state specifically for RLF case, using the same procedure as UE in RRC Idle/Inactive state.

Proposal 2. RAN2 agrees that there is no spec impact for the case where Rel-19 NES UE in RRC_CONNECTED with searchSpaceSIB1 configured but no SIB1 transmission in its active BWP.

Feature-specific cell barring

Proposal 3: RAN2 discusses the indication of feature-specific cell barring for a NES cell with on-demand SIB1.

# References

[1] RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

[2] “Report from session on V2X/SL, R18/19 MOB, and R19 NES” RAN2 #127bis, Hefei, China, October 14th – 18th, 2024.

[3] RP-243297 “Way forward on NES”, RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[4] “R2_129_R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM”, RAN2 #129, Athens, Greece, February 17th – 21st, 2025