3GPP TSG RAN WG2 Meeting #129bis                          	   R2-2502544

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.12.3

Source:	Qualcomm Incorporated

Title:	Discussion on UE initiated beam reporting

WID/SID:	NR_MIMO_Ph5-Core – Release 19

Document for:	Discussion

# Introduction

For UE-initiated/event-driven beam management, the following subject was outlined in the WID of NR MIMO Phase 5” [1].

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we discuss potential RAN2 work for supporting UE initiated beam reporting.

# Discussion

## Event evaluation

RAN1 has made progress and decisions on the following design subjects [2] [3].

Beam reporting modes - Mode A: dynamically scheduling UCI by gNB - Mode B: UCI in pre-configured resource(s) for second UL channel #### UL signaling content(s) of L1-RSRP report - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB Beam reporting trigger events - Event-1: Quality of the current beam is worse than a certain threshold. - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. RS measurement for the current beam for Event-2 - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. RS measurement for the new beam for Event-2 - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE

However, RAN1 did decide if the triggering event determination is handled at PHY or MAC layer.

At RAN1 #119, the following was proposed from the offline email discussion, but failed to reach an agreement during the online session [4].

At RAN2 #129, the following working assumption was made for event evaluation [7].

- Working assumption: RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision. - No need to send a LS to R1 on this.

Furthermore, with RAN1 #120 post meeting email discussion, no conclusion can be converged regarding RAN2’s working assumption.

Observation 1. RAN1 still cannot conclude whether PHY layer or MAC layer handles the event evaluation.

Based on the above observation, we propose to confirm the working assumption.

Proposal 1. RAN2 confirms the following working assumption:

“RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision.”

If event evaluation is handled by MAC layer, the event evaluation may include the follows:

- PHY send indication per new beam at each evaluation occasion

- MAC decides whether the event is triggered

- If event is triggered, MAC indicates PHY to send the UCI and a CSI report on PUSCH

Proposal 2. If event evaluation is handled by MAC layer, it contains the following steps:

- PHY send indication per new beam at each evaluation occasion

- MAC decides whether the event is triggered

- If event is triggered, MAC indicates PHY to send the UCI and a CSI report on PUSCH.

At RAN1 #120, the following agreements were made for event evaluation [6].

Agreement Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point. Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, further study the following options for possible down-selection - Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured. - Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured. - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s). - Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW. - Option-4: If an Event-2 instance for a new beam is obtained at the time t, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: T_window is the agreed time window parameter for measurement. - Note: Other options are not precluded.

Observation 2. RAN1 has not concluded the design details for counting and measurement window.

Proposal 3. RAN2 waits for RAN1's final decision on design details of counter and measurement window.

# Conclusion

In this contribution, we discussed potential RAN2 work for supporting UE initiated beam reporting and concluded with the following observations and proposals.

Event evaluation

Observation 1. RAN1 still cannot conclude whether PHY layer or MAC layer handles the event evaluation.

Proposal 1. RAN2 confirms the following working assumption:

“RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision.”

Proposal 2. If event evaluation is handled by MAC layer, it contains the following steps:

- PHY send indication per new beam at each evaluation occasion

- MAC decides whether the event is triggered

- If event is triggered, MAC indicates PHY to send the UCI and a CSI report on PUSCH.

Observation 2. RAN1 has not concluded the design details for counting and measurement window.

Proposal 3. RAN2 waits for RAN1's final decision on design details of counter and measurement window.

# References

[1] RP-242394, “WID revision: NR MIMO Phase 5”, RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.

[2] RP-241775, Status Report of NR MIMO Phase 5, RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.

[3] RP-241051, Status Report of NR MIMO Phase 5, RAN Meeting #104, Shanghai, China, June 17-20, 2024.

[4] R1-2410117, “Moderator Summary #1 on UE-initiated/event-driven beam management”, RAN1 #119, Orlando, US, November 18th – 22nd, 2024

[5] “Chair notes RAN1#119 eom0”, RAN1 #119, Orlando, US, November 18th – 22nd, 2024

[6] “Chair notes RAN1#120 eom0”, RAN1 #120, Athens, Greece, February 17th – 21st, 2025

[7] “R2_129 R18 MIMO R19 MIMO LPWUS SBFD (Erlin) 2025-02-21 1025 eom” RAN2 #129, Athens, Greece, February 17th – 21st, 2025