3GPP TSG-RAN WG2 Meeting #129bis	R2-2502564

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.7.5

Source:	China Telecom

Title:	Consideration on RLC AM Enhancements

Document for:	Discussion and Decision

- Introduction

In the last RAN2#129 meeting, the following agreements were made for RLC AM enhancements.

Autonomous retransmissions and polling enhancements Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC Only a single autonomous retransmission will be triggered per RLC SDU. There is no dynamic activation/deactivation of the autonomous retransmission mechanism. We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. The new RLC timer at the RX is per RLC entity The duration of the new RLC timer is not lower than that of t-reassembly Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

The paper aims to discuss the remaining issues on autonomous retransmission and polling enhancements.

2 Discussion

To ensure the timely retransmission of delay-critical data, it has agreed that the autonomous retransmission and/or polling can be triggered when the remaining time of an RLC SDU falls below a specified threshold. However, it is undetermined whether the remaining time is determined based on the discardTimer at PDCP or new timer at RLC.

Per TS38.323, the discardTimer is used to determine the delay-critical PDCP SDU. Specifically, if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold is defined as delay-critical PDCP SDU.

In addition, the PDCP will perform the SDU discard operation as follows.

TS38.323

5.3	SDU discard

When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU.

When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall:

-	if pdu-SetDiscard is configured:

-	discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs;

NOTE 1:	PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set.

-	else:

-	discard the PDCP SDU along with the corresponding PDCP Data PDU.

If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

It can be observed that the PDCP discards SDU upon the discardTimer or discardTimerForLowImportance expires for a PDCP SDU. If the corresponding PDCP data PDU has already been transmitted to a lower layer, the PDCP layer will also indicate the RLC layer to discard it. Therefore, the existing PDCP discardTimer can be used to determine which RLC SDUs becomes useless or needs to be discarded.

Based on the above description, it would be straightforward to use the discardTimer at PDCP to determine the remaining time that triggers autonomous retransmission and/or polling.

Proposal 1: Reuse the existing discardTimer at PDCP to determine the remaining time that triggers autonomous retransmission and/or polling.

In Rel-18 XR study, a PDU set containing one or more PDUs is introduced to carry the payload of one unit of information generated at the application level. Per TS38.322, the UE polls the gNB after a configured number of PDUs or bytes have been transmitted. It is likely that the UE may trigger a poll in the middle of a PDU set. As a result, the status report may only contain the reception status of a part of PDUs in a PDU set. If the remaining part of the PDUs has not been retransmitted promptly, the entire PDU set including previously successfully transmitted PDUs may be discarded, resulting in resource wastage.

Therefore, for XR traffic, the polling mechanism may need to be enhanced to allow the UE to trigger polling based on the PDU set. For instance, the polling bit could be set based on the PDU set traffic pattern. In this way, the UE can receive the status report of one or more PDU sets.

Proposal 2: Polling enhancement is needed to enable the UE to poll the RLC status based on the PDU set.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: Reuse the existing discardTimer at PDCP to determine the remaining time that triggers autonomous retransmission and/or polling.

Proposal 2: Polling enhancement is needed to enable the UE to poll the RLC status based on the PDU set.

- References

- R2-25xxxxx, Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece