3GPP TSG-RAN WG2 #129bis	R2-2502565

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.11.2

Source:	Ericsson

Title:	SBFD RA aspects

Document for:	Discussion

# 1	Introduction

In the WID [1], the following objective regarding random access operation has been defined for RAN2

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

We discuss the RAN2 aspects for the above objective in this paper. In the below, we use the term SBFD RA operation to stand for random access operation in SBFD symbols unless otherwise stated. We use the term legacy RA operation to stand for random access operation during non SBFD symbols.

In this paper, a SBFD aware UE means a UE which is capable of operating in a cell configured with SBFD feature, i.e., the cell/the gNB transmits DL and receive UL simultaneously in SBFD slots and symbols within a carrier. The UE can be aware of SBFD configurations so that the UE knows which slots/symbols are SBFD capable, which are also referred to as SBFD slots/symbols. This doesn’t mean that the UE needs to support full duplex operation.

# 2	Discussion

RAN2 has made the below agreements at RAN2#129

- When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell.

- When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration.

- For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS.

- If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

- FFS whether RO type selection is performed before or after the RA type selection.

- FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

## 2.1	Modelling aspects in MAC spec

Regardless of whether RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2 is applied, it is beneficial to aim for a unified design/framework between RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2 to simplify specification complexity. In an example, we can avoid specifying different UE behaviours and procedures for different RACH configuration options.

To aim for a unified RACH model in MAC for RACH configuration Option 1 and RACH configuration Option 2.

How to model SBFD RA operation in the MAC spec is a relevant issue. In the MAC spec, the MAC entity performs the following steps/procedures to conduct a random access procedure:

- Carrier selection, e.g., selection between NUL and SUL (as specified in clause 5.1.1)

- BWP selection (as specified in clause 5.1.1)

- selection of the set of RACH resource (as specified in clause 5.1.1b)

- determination of the RA type (as specified in clause 5.1.1)

- RACH resource selection according to the determined RA type (as specified in clauses 5.1.2 and 5.1.2a)

- transmission/handling the random access procedure (including Msg1/MsgA transmission as specified in clauses 5.1.3 and 5.1.3a, random access response/MsgB reception as specified in clauses 5.1.4 and 5.1.4a, contention resolution as specified in clause 5.1.5 and completion of the random access procedure as specified in clause 5.1.6) .

Modelling of SBFD RA operation in MAC spec may needs to consider the below aspects

- selection of the SBFD RACH configuration

- SBFD RACH occasion selection

- RO fallback

Regarding 1), we think it is unnecessary to model selection of the SBFD RACH configuration in MAC spec, due to below reasons

- There was no selection of RACH configuration modelled in the legacy. As an example, in legacy, the UE can perform fallback from 2-step RA to 4-step RA where 2-step RA and 4-step RA are configured with different RACH configurations. During the RACH procedure, there is no modelling/reference to the selection of either the 4-step RA configuration or the 2-step RA configuration.

No need to model with respect to selection of the SBFD RACH configuration in MAC spec.

In RACH configuration Option 1, SBFD ROs/additional ROs are configured/provided by the legacy RACH configuration. While for RACH configuration Option 2, SBFD ROs/additional ROs are configured/provided by the additional RACH configuration. Based on the current RAN2 discussion progress, it is assumed that the conditions will be adopted based on which the UE selects a proper RO type for initial PRACH transmission. As soon as the RO type is determined, the MAC shall apply the RA parameters (e.g., power control, preamble format, SSB to RO mapping if configured etc) associated with the RO type for the PRACH transmission. Such behaviours (i.e., the MAC shall apply the RA parameters associated with the RO type) need to be captured in the MAC spec.

As soon as the MAC selects a suitable RO type for a PRACH transmission, the MAC shall apply the RA parameters associated with the RO type when conducting the PRACH transmission.

In RAN2#129, RAN2 has discussed whether whether RO type selection is performed before or after the RA type selection, however without consensus. The discussed options are shown in the below

R2-2500298	Random Access for SBFD Operation	NEC	discussion	Rel-19	NR_duplex_evo-Core

Proposal-2: The RO type selection is performed after the RA type selection.

R2-2501244	Discussion on Random Access in SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

Proposal 6. The SBFD-aware UE selects between SBFD RO and legacy RO before selecting a set of Random Access resources.

It is reasonable to place RO type selection before selecting a set of random access resources due to the below reason

- The UE can determine whether to perform Msg1 repetition and corresponding Msg 1 repetition number according to the selected RO type

During the last RAN2 meeting, some companies had a concern that the fallback between legacy ROs and SBFD ROs mechanism will require the MAC entity to use RACH resources across the sets, i.e., fallback from set1 (e.g., associated with SBFD RO) to set2 (e.g., associated with legacy ROs). In an example, the UE is configured with one legacy RACH configuration and one SBFD/additional RACH configuration, during a random access procedure, the UE first selects SBFD ROs for the RACH procedure, the UE determines set1 of RACH resources (which is taken from SBFD RACH configuration). During the RACH procedure, the UE is triggered to fallback from SBFD ROs to legacy ROs. Then the UE is required to determine set2 of RACH resources (i.e., taken from the legacy RACH configuration), after that, the UE uses set2 RACH resources to perform Msg1 repetition until the UE has succeeded to transmit Msg1 and complete the RACH procedure. An example of the procedure is illustrated in Figure 1.

Figure 1. An example of the fallback procedure combined with Msg1 repetition.

We think this can be easily implemented in MAC spec.

RO type selection is performed before selection of a set of RACH resources.

## 2.2	RO Prioritization

Regarding RO prioritization, RAN2 has made the below agreement

- For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS.

- If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

If there is no cell specific indication on the RO type, a UE selects ROs based on measured SSB RSRP. There are two scenarios raised by companies for usage of SSB RSRP during RAN2#128, wherein one scenario is to limit the use of additional ROs to cell centre UEs (for mitigating CLI purpose), and the second scenario is to limit application of additional ROs to UEs at the cell edge (for increasing coverage purpose). 
To ensure both scenarios to be covered, the network can signal a SSB RSRP threshold to UEs. In addition, the network also indicates to UEs how to use the SSB RSRP threshold. As an example, the network configures a high SSB RSRP threshold to UEs. The UEs whose measured SSB RSRP above the threshold can choose additional ROs for mitigating CLI purpose. As another example, the network configures a low SSB RSRP threshold to UEs. The UEs whose measured SSB RSRP below the threshold can choose additional ROs (together with a long preamble format) for coverage extension purpose.

For the cell specific indications on the RO type prioritization for CBRA, the indication can be included in SIB1, ServingCellConfigComm. We think such cell specific signaling alternative is sufficient given the use cases which the SBFD RA operation intends are deployment dependent, i.e., latency reduction, capacity increase, range extension or coverage enhancement. There is no need for the gNB/cell to often update the indication of the RO type.

For initial RA transmission, the network only indicates the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA via cell specific signaling in SIB1 and ServingCellConfigComm. No other signaling means on the RO type is needed.

If there is no cell specific indication on the RO type, a UE selects ROs based on measured SSB RSRP. There are two scenarios raised by companies for usage of SSB RSRP during RAN2#128, wherein one scenario is to limit the use of additional ROs to cell centre UEs (for mitigating CLI purpose), and the second scenario is to limit application of additional ROs to UEs at the cell edge (for increasing coverage purpose). 
To ensure both scenarios to be covered, the network can signal a SSB RSRP threshold to UEs. In addition, the network also indicates to UEs how to use the SSB RSRP threshold. As an example, the network configures a high SSB RSRP threshold to UEs. The UEs whose measured SSB RSRP above the threshold can choose additional ROs for mitigating CLI purpose. As another example, the network configures a low SSB RSRP threshold to UEs. The UEs whose measured SSB RSRP below the threshold can choose additional ROs (together with a long preamble format) for coverage extension purpose.

If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold

For initial RA transmission using CBRA, if no RO type indication is provided by the NW, in addition to signal a SSB RSRP threshold, the network also signals an indicator indicating whether to select the additional RO type below or above this SSB RSRP threshold.

## 2.4	RO Fallback

An SBFD aware UE can use SBFD ROs and/or legacy ROs. It may be necessary to discuss if the UE can mix ROs of different RO types when performing RA operation. Given the UE may apply different power control parameters/settings for RACH operation in SBFD symbols and non SBFD symbols, it may be not preferred for the UE to mix ROs of different RO types when performing RA operation.

It may be also necessary for RAN2 to study whether the SBFD aware UE to use both SBFD ROs and legacy ROs at the same time especially when the UE has detected congestion or RA problem for the selected ROs. In an example, the UE first uses legacy ROs, since the triggered RA procedure is not delay sensitive. However, the UE detects RA failures/problem after a while, then there is risk that the UE cannot fulfil the latency requirement for the RA procedure, it would be helpful if the UE is allowed to fall back to SBFD ROs. It may be also possible that the UE initially selects SBFD ROs, however the UE later experiences RA problem due to interference e.g., CLI introduced by other UEs or gNBs, thus it may be beneficial to allow the UE to fall back to legacy ROs.

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempts (i.e., RACH problem is detected) in legacy RACH occasions, UE is allowed to switch to SBFD RACH occasions.

## 2.5	SBFD awareness early indication

Regarding SBFD awareness early indication, RAN2 has made the below agreement:

- From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

The remaining issue is whether a SBFD aware UE can indicate SBFD awareness to the gNB via Msg3 or Msg5. We think this is not needed either due to below reasons:

- This would require an indicator to be introduced in either MAC layer or RRC signaling, which would increase unnecessary design efforts for RAN2.

- With early indication via Msg3 or Msg5, the gNB may be only feasible to schedule SBFD resources to subsequent signalling transmissions. The achieved latency reduction would be marginal given that the UE will anyway provide its capability report to the gNB after the RACH procedure.

- The UE doesn’t choose SBFD RO for Msg1 transmission, meaning that the UE sees no need to apply SBFD RA (to exploit latency reduction or coverage extension). In this sense, it is unnecessary for the gNB to schedule SBFD resources to the UE before the gNB receives the UE  capability from the UE or CN.

Support SBFD awareness early indication via Msg3 or Msg5 would require RAN2 to introduce an indicator in either MAC layer or RRC signalling, which would increase unnecessary design efforts for RAN2.

With early indication via Msg3 or Msg5, the gNB may be only feasible to schedule SBFD resources to subsequent signalling transmissions. The achieved latency reduction or other performance enhancements would be marginal given that the gNB will anyway receive the UE capability from UE or CN after the RACH procedure.

If the UE sees no need to apply SBFD RA (to exploit latency reduction or coverage extension), the UE would not choose SBFD RO for Msg1 transmission.

Therefore, in our view, it is unnecessary to introduce early indication for SBFD awareness. Anyway, an SBFD aware UE can be identified by the gNB at an early stage when the UE uses SBFD RO, which is sufficient.

SBFD awareness early indication via Msg3 or Msg5 is not pursued for SBFD aware UE.

## 2.6	SBFD Indication for CFRA

RAN2 has made agreement regarding CFRA

The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

Regarding the FFS, it is reasonable to also include the RO type for the SI request. The network can decide whether to apply a common RO type for all SBFD aware UEs for their RACH based SI request, based on the cell level measurements, i.e., RACH load/congestion, gNB to gNB CLI etc.

The network can decide whether to apply a common RO type for all SBFD aware UEs for their RACH based SI request, based on the cell level measurements, i.e., RACH load/congestion, gNB to gNB CLI etc.

The RO type is indicated by NW for the CFRA based SI request case.

## 2.7	RA-RNTI collision

At RAN2#127bis, some companies have proposed to study the issue on RA-RNTI collision. They declare that:

For PRACH configuration Option 2 (i.e., an SBFD-aware UE is configured with a legacy and additional PRACH configurations), there is a possibility that the SBFD ROs configured by the additional configurations may overlap with the legacy ROs. This could happen in flexible symbols (as both legacy and SBFD ROs can be configured in flexible symbols). When this overlapping happens, there is a non-negligible possibility that at least s_id and t_id of the overlapping legacy RO and SBFD RO are the same (and of course, the ul-carrier_id is the same since we are talking about the same carrier). Now the tricky part is the f_id. If the legacy specs is kept as is, then at the point when there is such overlapping, there is a mismatch between the frequency indices being used by legacy UEs and the ones being used by SBFD UEs. This leads to the same RA-RNTI and confuses these UE when receiving the same RA-RNTI.

More specifically, the UE calculates the RA-RANTI when sending the Msg 1 of the RA procedure. According to TS38.321, the RA-RNTI is computed as

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

- s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

- t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

- f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

- ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

In our view, we think the issue is invalid due to the below reasons

- According to RAN1 agreements,

a RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default. A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE:

- The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH.

- UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols

- There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH.

- Other assumptions are not precluded.

NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB.

For a SBFD RO across SBFD symbols and non-SBFD symbols, if the SBFD RO is overlapping with the legacy RO on a 2nd symbol or beyond, there is no issue, in such case, at least the s_id would be different, which would be sufficient to distinguish RA-RNTIs between the SBFD RO and the legacy RO which are overlapping.

Regarding whether a SBFD RO across SBFD symbols and non-SBFD symbols, may be overlapping with a legacy RO in the first symbol, we think it can be either avoided by NW configuration, or RA-RNTI collision can addressed by allocating different preamble list to the legacy RACH configuration and additional RACH configuration. o

- In addition, a configured RO starting from non SBFD symbols and ending in SBFD symbols, is invalid.

For a RO across SBFD symbols and non-SBFD symbols, the RO may be overlapping with a legacy RO on 2nd symbol or beyond, in such case, at least the s_id would be different, which would be sufficient to distinguish RA-RNTIs between the SBFD RO and the legacy RO which are overlapping.

A configured RO starting from non SBFD symbols and ending in SBFD symbols, is invalid.

All in all, we see no issue for RA-RNTI.

No need to update the RA-RNTI formula for SBFD RA configuration.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	Support SBFD awareness early indication via Msg3 or Msg5 would require RAN2 to introduce an indicator in either MAC layer or RRC signalling, which would increase unnecessary design efforts for RAN2.

Observation 2	With early indication via Msg3 or Msg5, the gNB may be only feasible to schedule SBFD resources to subsequent signalling transmissions. The achieved latency reduction or other performance enhancements would be marginal given that the gNB will anyway receive the UE capability from UE or CN after the RACH procedure.

Observation 3	If the UE sees no need to apply SBFD RA (to exploit latency reduction or coverage extension), the UE would not choose SBFD RO for Msg1 transmission.

Observation 4	The network can decide whether to apply a common RO type for all SBFD aware UEs for their RACH based SI request, based on the cell level measurements, i.e., RACH load/congestion, gNB to gNB CLI etc.

Observation 5	For a RO across SBFD symbols and non-SBFD symbols, the RO may be overlapping with a legacy RO on 2nd symbol or beyond, in such case, at least the s_id would be different, which would be sufficient to distinguish RA-RNTIs between the SBFD RO and the legacy RO which are overlapping.

Observation 6	A configured RO starting from non SBFD symbols and ending in SBFD symbols, is invalid.

Based on the discussion in the previous sections we propose the following:

Proposal 1	To aim for a unified RACH model in MAC for RACH configuration Option 1 and RACH configuration Option 2.

Proposal 2	No need to model with respect to selection of the SBFD RACH configuration in MAC spec.

Proposal 3	As soon as the MAC selects a suitable RO type for a PRACH transmission, the MAC shall apply the RA parameters associated with the RO type when conducting the PRACH transmission.

Proposal 4	RO type selection is performed before selection of a set of RACH resources.

Proposal 5	For initial RA transmission, the network only indicates the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA via cell specific signaling in SIB1 and ServingCellConfigComm. No other signaling means on the RO type is needed.

Proposal 6	For initial RA transmission using CBRA, if no RO type indication is provided by the NW, in addition to signal a SSB RSRP threshold, the network also signals an indicator indicating whether to select the additional RO type below or above this SSB RSRP threshold.

Proposal 7	For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempts (i.e., RACH problem is detected) in legacy RACH occasions, UE is allowed to switch to SBFD RACH occasions.

Proposal 8	SBFD awareness early indication via Msg3 or Msg5 is not pursued for SBFD aware UE.

Proposal 9	The RO type is indicated by NW for the CFRA based SI request case.

Proposal 10	No need to update the RA-RNTI formula for SBFD RA configuration.

# 4	References

- [1] 	RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei,

- Shanghai, China, June 17-20, 2024, RAN#104.