3GPP TSG-RAN WG2 #129bis	R2-2502566

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.11.3

Source:	Ericsson

Title:	CSI-RS measurements and SBFD operation in CA and DC

Document for:	Discussion

# 1	Introduction

In this paper, we discuss CSI-RS measurements for RLM/BFD/CBD in SBFD.

In this paper, a SBFD aware UE means a UE which is capable of operating in a cell configured with SBFD feature, i.e., the cell/the gNB transmits DL and receive UL simultaneously in SBFD slots and symbols within a carrier. The UE can be aware of SBFD configurations so that the UE knows which slots/symbols are SBFD capable, which are also referred to as SBFD slots/symbols. This doesn’t mean that the UE needs to support full duplex operation.

# 2	Discussion

## 2.1	CSI-RS measurements for RLM/BFD/CBD

RAN1 has sent a LS reply in “R1-2501560 - Reply LS on CSI-RS measurement with SBFD operation”. The relevant content is:

Issue#2: Behaviour for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD measurements For resource configuration and/or reporting configuration for CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement, RAN1 would like to provide the following response: - According to the following RAN1 agreements, Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement. Agreement (RAN1#120) The configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP. Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only. - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols. - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability. Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.. - The following RAN1 agreements for CSI reporting are applicable to CSI-RS based L1-RSRP/SINR measurement/reporting. Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement (RAN1#120) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig..

Based on the LS reply, the below observations can be made:

Configuration 1 and Configuration 2 on UL transmissions and DL reception across SBFD symbols and non-SBFD symbols in different slots are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement.

UE can provide separate CSI reports to the gNB according to the configured valid symbol type in the corresponding CSI-ReportConfig.

RAN1 doesn’t indicate whether the valid symbol type can be included in RadioLinkMonitoringConfig and BeamFailureRecoveryConfig.

Since RAN1 has not indicated whether the valid symbol type can be included in RadioLinkMonitoringConfig and BeamFailureRecoveryConfig, RAN2 can assume that there is no enhancement needed for RLM/BFD/CBD measurements unless RAN1/RAN4 indicates to RAN2 for further actions. Even without any enhancement for RLM/BFD/CBD measurements, the gNB can provide proper settings (e.g., beamFailureDetectionTimer and/or beamFailureInstanceMaxCount) to the UE to accommodate differences between CSI-RS measurement results in legacy symbols and in SBFD symbols.

RAN2 can assume that there is no enhancement needed for RLM/BFD/CBD measurements unless RAN1/RAN4 indicates to RAN2 for further actions.

In the MAC spec, the MAC entity monitors BFIs. When the number of consecutively occurred BFIs reaches the maximum number, the UE triggers BFR. There is no need for the MAC entity to trigger BFR for non SBFD symbols and SBFD symbols separately.

Keep the legacy RLM, BFD and BFR procedure in the MAC layer, i.e., no need for the MAC entity to trigger RLF, BFD and BFR for non SBFD symbols and SBFD symbols separately.

## 2.2	SBFD in CA and DC

RAN1 made the following agreement with respect to SBFD in a CA scenario:

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

In addition, RAN4 has previously made the following agreement:

Agreement: - Rel-19 requirement shall consider both (1) single carrier operation for SBFD (2) SBFD operates in only one BS carrier, and legacy TDD operates in other intra-band BS carrier(s) contiguous or non-contiguous to the SBFD carrier.

During the RAN1 discussions leading to the above agreement, it was argued that RAN4, based on the above RAN4 agreement, had already agreed to support CA. In our view, that is an incorrect interpretation of the RAN4 agreement since it does not even mention CA. Nevertheless, the RAN1 agreement requires clarification in what it really means. In short, the agreement can be viewed from two perspectives: the UE perspective and the gNB perspective.

The gNB perspective is straightforward, then the agreement implies that one out of multiple carriers may support SBFD but it says nothing about multi-carrier operation with respect to a specific UE. In our view, this interpretation requires no additional specification work. In our opinion, the above RAN4 agreement supports this view since it clearly refers to BS carriers, i.e., from the gNB’s perspective.

The UE perspective, on the other hand, becomes complicated since it would imply that carrier aggregation (CA) is supported. In our view, CA should not be combined with SBFD for several reasons. First, according to the WID [1], the justification for SBFD is predominantly to increase UL coverage and UL capacity and/or to decrease latency. While CA may be helpful for capacity, albeit not efficiently so in combination with SBFD, it will not be useful in a low latency scenario considering it takes approximately 20-50 ms to activate CA. Additionally, performance gains in SBFD and CA are opposing in the sense that any gains from additional carriers will be severely restricted by the increased UL need from SBFD as is shown in Figure 1. Furthermore, we don’t see the benefit for CA in the coverage aspect, leading us to conclude that the CA and SBFD features are at best orthogonal in their respective use cases but sometimes even opposing.

Figure 1: Example of contradiction between CA and SBFD.

In order to support SBFD together with CA, RAN1 would need to also specify collision rules among carriers, significantly adding to the specification effort with hardly any time left in Rel-19. The need to resolve collisions among carriers is valid for both intra- and inter-band CA. For inter-band CA, the existing UE capability SimultaneousTxRx eliminates the need for further collision rules but there is no corresponding capability for intra-band CA (that would in practice be an SBFD-capable UE). This leads us to conclude that SBFD with CA should only be supported for inter-band CA and for UEs supporting SimultaneousTxRx, unless the network operates with synchronized and aligned TDD UL/DL patterns among carriers.

In our view, any attempt to combining SBFD and CA should proceed with caution, if it should be undertaken at all. SBFD can be supported in PCells with feasible additional specification work (assuming it is restricted to inter-band and for UEs supporting Simultaneous TxRx such that inter-carrier collisions can be disregarded by RAN1) whereas supporting SBFD in SCells would require additional efforts. Moreover, even if RAN1 impact is limited, in addition to the RAN1 specification impacts as described in the above, support SBFD with CA has also RAN4 impacts, i.e., RAN4 needs to study requirements for SCell activation/deactivation. That will be difficult to finalize in Rel-19.

Therefore, it is necessary to trigger further discussions and clarification on whether SBFD supported in CA in RAN1 and RAN4. It is therefore recommended to send a LS to RAN1 and RAN4 for querying if SBFD is supported in CA. The same question is also valid for DC.

Send a LS to RAN1 and RAN4 for querying if SBFD is supported in CA and DC scenarios.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	Configuration 1 and Configuration 2 on UL transmissions and DL reception across SBFD symbols and non-SBFD symbols in different slots are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement .

Observation 2	UE can provide separate CSI reports to the gNB according to the configured valid symbol type in the corresponding CSI-ReportConfig.

Observation 3	RAN1 doesn’t indicate whether the valid symbol type can be included in RadioLinkMonitoringConfig and BeamFailureRecoveryConfig.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 can assume that there is no enhancement needed for RLM/BFD/CBD measurements unless RAN1/RAN4 indicates to RAN2 for further actions.

Proposal 2	Keep the legacy RLM, BFD and BFR procedure in the MAC layer, i.e., no need for the MAC entity to trigger RLF, BFD and BFR for non SBFD symbols and SBFD symbols separately.

Proposal 3	Send a LS to RAN1 and RAN4 for querying if SBFD is supported in CA and DC scenarios.

# 4	References

- [1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Shanghai, China, June 17-20, 2024, RAN#104.

[2] R1-2501560, Reply LS on CSI-RS measurement with SBFD operation, MediaTek, Athens, Greece, February 17th – 21st, 2025, RAN1#120.