3GPP TSG-RAN WG2 #129bis	R2-2502575

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.18

Source:	Ericsson

Title:	MPS at gNB overload

Document for:	Discussion

# 1	Introduction

At RAN2#128, draft CRs on modifying resume cause used at RNA Update by MPS-registered UE (from rna-Update to mps-PriorityAccess) for NR and LTE was discussed [1], [2]. RAN2 concluded:

The CR is not pursued for Rel-18

This will be addressed in Rel-19 in TEI19 with optional feature without capability signaling

In this document, we motivate why this (optional) UE change is not essentially needed.

# 2	Discussion

A gNB should at high load ensure prioritised traffic is executed, without having to reject UEs. Rejecting UEs is not a very wise strategy by network, since those UE will anyway come back soon, and even build up a bulk of UEs that will trigger new accesses.

Our preferred way to handle overload situations on network side is to use the UAC (Unified Access Control) feature. With UAC information provided in SIB1, the gNB can selectively bar UEs from triggering accesses (establishment and resume requests). Barred UEs will not trigger any access attempt during a certain time period and the network can decide when to stop the UAC barring behaviour. E.g., a simple network strategy could be to bar connection requests from “normal UEs” (e.g. at a certain percentage in a fair manner), such that sufficient capacity is ensured e.g. for accesses from MPS-registered UEs, for emergency services etc. This means that the cases where the gNB need to explicitly reject UEs will be very few, if any at all.

For RNA Update, it is e.g. possible to set UAC information such that RNA updates (Access Class 8) is barred for “normal UEs”, but MPS-registered UEs (Access Identity 1) may bypass this barring and trigger RNA Update.

With UAC, the cases where the gNB need to explicitly reject UEs will be very few, if any at all.

There are also some drawbacks on network side by changing the resume cause from rnaUpdate to mps-PriorityAccess for MPS-registered UE. By this change, gNB will always for MPS-registered UE act as if this is a resume of the RRC connection, for the purpose of communication. The gNB will then retrieve the UE context from other gNB and activate user plane connections with the UE. This was not the intention by UE that triggered RNA update. Furthermore, today the gNB can (in case the UE context is not present in the gNB that receives the resume request from the UE) select to execute the RNA update with or without UE context retrieval.  With the changed resume cause, RAN Update without UE context retrieval is no longer possible.

Changed resume cause from rnaUpdate to mps-PriorityAccess for MPS-registered UE results in un-intended gNB actions.

Above we motivated why here is no critical need to change the UE-setting of resume cause used at RNA update for MPS-registered UE. We ask RAN2 to agree on the following:

RAN2 to consider the observations above in potential further discussions on this topic.

# 3	Conclusion

Based on the discussion in the previous sections we observe the following:

Observation 1	With UAC, the cases where the gNB need to explicitly reject UEs will be very few, if any at all.

Observation 2	Changed resume cause from rnaUpdate to mps-PriorityAccess for MPS-registered UE results in un-intended gNB actions.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to consider the observations above in potential further discussions on this topic.

# 4	References

[1]	R2-2501339	RAN Notification Area Update (RNA) for Multimedia Priority Service (MPS)

[2]	R2-2501340	RAN Notification Area Update (RNA) for Multimedia Priority Service (MPS)