3GPP TSG RAN WG2 #129bis	                             R2- 2502577

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.5.3

Source:         Fainity

Title:	SIB1 acquisition for Rel-19 NES UE in RRC_CONNECTED state

Document for:	Decision

Introduction

In RAN2#129 meeting, OD-SIB1 for Rel-19 NES UE in RRC_CONNECTED and RRC_IDEL/INACTIVE were discussed. Accordingly, the following agreements were achieved [1].

Agreement - There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1. 2. Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running: - When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid. - The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition. - The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs. 3. The UE follows the legacy validity principle of stored SIB. 4. For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected. 5. SIB-x can be cell specific or area specific, as legacy. 6. Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

However, the behavior for Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, can’t have consensus. In this contribution, we share our opinion and have proposals.

Discussion

There are three options proposed in RAN2#129 meeting to resolve the issue for CONNECTED Rel-19 NES UE to acquire SIB1 if searchSpaceSIB1 is configured and SIB1 is not broadcasting:

Option 1: Rely on UE to acquire SIB1 via OD-SIB1 procedure as RRC IDLE/INACTIVE state

Option 2: Rely on UE to acquire SIB1 via a dedicated-RRC request

Option 3: Rely on the network dedicated RRC to deliver SIB1.

The Pros/Cons analysis and our opinion is provided as below session.

Option 1: Rely on UE to acquire SIB1 via OD-SIB1 procedure

If Connected UE has valid WUS configuration, it is natural the UE can trigger the WUS if trigger conditions satisfied. To simplify the design, the validity checking and trigger conditions are supposed to be the same as RRC IDEL/INACTIVE state. With option 1, the benefit is to allow Connected UE to acquire SIB1 based on its requirement which may not be known by the network. However, it is still up to network implementation to provide SIB1 even the OD-SIB1 request was successfully received. In addition, option 1 may not be useful since whether every Connected Rel-19 NES UE has valid WUS configuration is problematic to guarantee the proceeding.

Observation #1: Rely on UE to acquire SIB1 via OD-SIB1 procedure is an optimization and can’t be used for every scenarios.

Option 2: Rely on UE to acquire SIB1 via a dedicated-RRC request

To enable option 2, it is required to add a new code point for SIB1 in DedicatedSIBRequest. The consequence of the update is whether we only allow Rel-19 NES UE to have this capability, or whether the feature is generally allowed for all Rel-19 UE. If we go to the 1st choice, it implies option 2 still can’t resolve all scenarios. On the contrary, if we go with the 2nd choice, it is expected UEs may aggressively request the SIB1 transmission and may enforce the SIB-less Cell lost the power saving gain if the concerned cell agrees to either broadcast SIB1 or transmit the SIB1 via dedicated RRC. Meanwhile, it seems like no difference with option 3.

Observation#2: Rely on UE to acquire SIB1 via a dedicated-RRC request doesn’t have extra performance gain as comparing with option 3, but the enable of option 2 add specification impacts.

Option 3: Rely on the network dedicated RRC to deliver SIB1.

The argument on option 3 is network may not know when the UE wants to request SIB1, but this issue already existed in legacy specifications and fully rely on network implementation. From network perspective, option 3 can aim to sustain the network energy saving and align with the original intention that searchSpaceSIB1 is configured and SIB1 is not broadcasting.

Observation#3: Rely on the network dedicated RRC to deliver SIB1 could align with the purpose of NES and no significant issues to be resolved.

Therefore, based on the analysis and observations, we prefer option3 as the solution for CONNECTED Rel-19 NES UE to acquire SIB1 if searchSpaceSIB1 is configured and SIB1 is not broadcasting.

Proposal 1: Support rely on the network dedicated RRC to deliver SIB1 for CONNECTED Rel-19 UE and no further optimization is needed.

Conclusion

In this contribution, we provide our views on NES SIB1 acquisition for Rel-19 CONNECTED UE and we have the following observations:

Observation #1: Rely on UE to acquire SIB1 via OD-SIB1 procedure is an optimization and can’t be used for every scenarios.

Observation#2: Rely on UE to acquire SIB1 via a dedicated-RRC request doesn’t have extra performance gain as comparing with option 3, but the enable of option 2 add specification impacts.

Observation#3: Rely on the network dedicated RRC to deliver SIB1 could align with the purpose of NES and no significant issues to be resolved.

We further propose:

Proposal 1: Support rely on the network dedicated RRC to deliver SIB1 for CONNECTED Rel-19 UE and no further optimization is needed.

Reference

  - RAN2#129 meeting chairman notes.