3GPP TSG-RAN WG2 Meeting #129bis	R2-2502578

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.2.3

Source:	Panasonic

Title: 	Discussion on re-access mechanism for D2R failures

Document for:	Discussion

# Introduction

RAN2 started work item phase for A-IoT in Rel-19 as per WID in [1], to bring some refinements for the conclusions made in TR 38.769. In the previous meetings, RAN2 has discussed high level aspects about failure mechanism for devices to trigger re-access during next paging occasion, some of the agreements are listed below.

Agreements RAN2#127bis - In case of D2R data transmission failure, device follows the reader instruction: - it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and - reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R). - Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data. Agreements RAN2#128 - The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose. - Capture the following option in TR: The devices can be triggered by R2D message to re-access before subsequent paging retransmissions. Agreements RAN2#129 - NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback.

This document discusses re-access mechanism to handle failure scenarios of D2R transmissions and provides more details on the requirements of a timer and feedback message form the reader, also discusses multiple approaches to handle the issue.

# Discussion

Baseline procedure for A-IoT is agreed in RAN2 as seen in the figure below. The procedure flow and the steps involved for the data transmission between A-IoT reader and A-IoT device depends on the type of service required to be executed, either ‘inventory only’ or ‘inventory and command’. If the reader triggers inventory only then the procedure ends at step B, otherwise if the reader intends to perform command operations to the device, then the procedure continues to step C.

Figure 1: Overall AS procedures between A-IoT device and reader (source; TR 38.769 v19.0)

From the device point of view, the type of service triggered (i.e., ‘inventory only’ or ‘inventory and command’) for the ongoing procedure may not be aware, also it need not be indicated when the paging is triggered by the reader. Therefore, when A-IoT device does not receive a follow-up message from the reader in R2D after sending the data on the D2R resource, it could be either due to the end of a procedure (e.g., end of A-IoT procedure at step B as part of inventory only) or due to failure of the transmitted data over the A-IoT air interface.

According to the current agreements, failure handling is seen possible using a feedback message sent by the reader to indicate as success or failure of the expected D2R transmission. Also, such explicit feedback message to indicate success or failure maybe important if reader does not intend to continue with the procedure any further, i.e., no follow-up message after D2R. But it is not clear from the agreements, what is the behaviour of the device if there is no feedback message (to indicate success or failure) or a follow-up message in R2D from the reader, possibly due to failure of the R2D as illustrated in the issue below.

Issue with explicit feedback indication for re-access triggering

When device does not receive feedback (or a follow-up) message in R2D after the data transmission in D2R, could mean that either the reader has received the message in D2R successfully and does not intend to trigger messages further as the inventory or command procedure has been completed, or it could also mean that the D2R or R2D message has failed to be received (possibly R2D was sent with a NACK feedback to trigger re-access for the detected D2R failure, which could also fail to be received), figure 2 illustrates one such scenario.

Figure 2: D2R/R2D failure illustration

Here the duration for which devices must wait for the feedback message in R2D could be proportional to the [D2R to R2D] maximum time specified for A-IoT devices, which indicates the time duration where devices can expect the next corresponding R2D message followed by the D2R response. This could also be applicable to the Msg3 transmissions which are sent in D2R as part of the CBRA random access procedure.

Note that [D2R to R2D] maximum time specified for A-IoT devices could be just power shortage of the device if explicit failure indication (NACK) with implicit success detection is taken. In this case, there can be no explicit D2R to R2D maximum time.

Observation 1: A-IoT devices cannot completely rely on explicit feedback message from the reader to perform re-access for all the failure scenarios occurred for the D2R transmission failures and not received R2D feedback.

Proposal 1: Support implicit success or failure detection of the D2R transmission (including Msg3 failure) using D2R to R2D maximum time (T[D2R-R2D]max) defined for the device, when the follow-up message in R2D is not received from the reader. In case of implicit success detection, D2R to R2D maximum time may be realized by the power drain of the device i.e. without timer.

Further, the re-access mechanism using implicit failure detection at the device must be accompanied by an explicit success indication from the reader for the received D2R, or conversely the implicit success detection at the device must be accompanied by an explicit failure indication from the reader for the detected failure of the D2R. The later sections explain these mechanisms in detail to compare the benefits of each of the alternative approaches, and RAN2 can consider the pros and cons to down select from one of the mechanisms.

Implicit failure detection with explicit success indication (ACK)

Figure 3: Misleading contention resolution and Msg3 transmission failure

If device receives a positive acknowledgment from the reader for each D2R transmission either in the form of ACK message or a follow-up message in R2D (when D2R and R2D messages are triggered consequentially and executed sequentially), it could mean that the last transmitted D2R is successfully received. Conversely, when devices do not receive any acknowledgment after D2R transmission and within the anticipated time window, it means that the transmission or reception has failed.

This approach makes failure detection and re-access mechanism more effective and reliable since the device can detect all the failure cases unlike the reader. Though the reader can detect the failure, if the same is not indicated to the device within the time window dedicated for the failure indication it still results in failure of the procedure, as seen in figure 2 above. Also, some misleading failure indication from the reader can also be corrected by using implicit failure detection at the device, e.g., when Msg3 encounters collision due to unresolved contention of using same random ID by multiple devices, as shown in figure 3.

Consider the case where the reader detects random access response (Msg1) sent from multiple A-IoT devices using same random ID value. i.e., when the transmitted signal is same from multiple devices, then the receiver can happen to detect the random ID as from a single device and respond with Msg2 in R2D. Consequentially, these devices can consider contention was resolved by the used random ID value (see figure 3) although contention is not resolved in reality. Therefore, these devices transmitting Msg3 in D2R (with individual device IDs) is most likely to undergo contention again and not decoded by the reader. At this point, re-transmission attempts could be possible, but the re-transmitted Msg3 data is most likely to fail again because of the contention of the different device ID. Alternatively, it may seem suitable to send NACK message to trigger re-access for not decoding Msg3 correctly, but sending this trigger within the time window maintained for feedback is crucial which may be missed because the reader may have to attempt re-transmission first and then trigger re-access. Therefore, if reader fails to send the feedback on time, it is best if failure detection is performed by the device which ensures re-access at least during the next occasion.

Observation 2: A-IoT device can detect all the failure scenarios occurred due to D2R transmission failures, if the R2D feedback with positive acknowledgment (ACK) is always sent in R2D for every D2R.

Observation 3: Implicit failure detection mechanism for re-access is more reliable which ensures completion of the procedure started by the device, at least during the subsequent paging triggered for re-access.

Explicit failure indication (NACK) with implicit success detection

Figure 4: Implicit success detection mechanism

In contrast to the first approach as described above, unexpected behaviour of the device due to failure of R2D messages, as seen in figure 2, can be avoided when implemented with proper precautionary measures and by reducing the probability of the transmission failures of at least the R2D messages received at the device.

- The time window to receive R2D feedback at the device could be configured large enough to incorporate repeated attempts of re-transmission requests and a feedback (like negative acknowledgement) message to follow when needed.

- If R2D failure probability is ensured to be very low, then reader sending negative acknowledgment (NACK) to trigger devices for re-access could be helpful when re-transmission attempts fail to receive the expected D2R.

On the other hand, failures due to misleading feedback indication from the reader as shown in figure 3 can be very low, considering that the code space for the random ID size of 16-bits reduces the probability of multiple devices using the same random ID, to a factor of approximately: 1/ (2^16) ≈ 10^-5. Furthermore, those devices where Msg3 is failed due to rarely occurring contention can also follow the same precautionary steps as above, to explicitly trigger re-access to all such devices and to re-generate a new random ID during the next paging occasion.

Overall, since the reader sends feedback messages either in the form of re-transmission requests or re-access triggers to the devices, only upon failure detection of the D2R. The utilization of R2D resources to send feedback message would be low, considering the occurrence of failures in A-IoT procedure is also low.

Observation 4: Implicit success detection mechanism for re-access, though maybe less reliable but it could be sufficient when the probability of transmission failures in R2D is sufficiently lower than D2R failures.

Observation 5: It improves resources utilization in R2D, since the feedback messages are sent only upon failure detection and because the number of transmissions to indicated failure (NACK) is comparatively less than the transmissions required to indicate success (ACK).

Finally, the two approaches to be discussed are proposed below, but the suitable options can differ for the failure handling during Msg1 sent in D2R for the CBRA case and that for the remaining D2R messages (for e.g., Msg3 sent in D2R for CBRA, or messages sent in D2R as part of step-C).

For the case of Msg1 failure handling during CBRA, the subsequent R2D is always required to determine the successful operation, like Msg 2 corresponding to Msg 1 to resolve contention in CBRA. Hence option 1 is more suitable as the overhead concern of option 2 is not valid. And for the remaining cases, option 2 can be used.

Proposal 2: Support implicit failure detection for Msg1 transmission in D2R during CBRA procedure, if no explicit success indication is sent from the reader within a time window.

Proposal 3: RAN2 to discuss if device should perform implicit success detection or implicit failure detection for Msg3 failure handling (in CBRA) and the subsequent D2R transmissions, considering the following options. And option-2 is more suitable since subsequent R2D is not always required for the successful operation.

Option-1: Implicit failure detection at the device with explicit success indication (ACK) from the reader, for higher reliability to recover or re-access upon all the possible failure scenarios.

Option-2: Implicit success detection at the device with explicit failure indication (NACK) from the reader, for efficient resource utilization in R2D and with an assumption that the failures in R2D is sufficiently lower than the failures in D2R.

# Conclusions

Time window for implicit success or failure detection

Observation 1: A-IoT devices cannot completely rely on explicit feedback message from the reader to perform re-access for all the failure scenarios occurred for the D2R transmission failures and not received R2D feedback.

Proposal 1: Support implicit success or failure detection of the D2R transmission (including Msg3 failure) using D2R to R2D maximum time (T[D2R-R2D]max) defined for the device, when the follow-up message in R2D is not received from the reader. In case of implicit success detection, D2R to R2D maximum time may be realized by the power drain of the device i.e. without timer.

Implicit success or failure detection for NACK based mechanism

Observation 2: A-IoT device can detect all the failure scenarios occurred due to D2R transmission failures, if the R2D feedback with positive acknowledgment (ACK) is always sent in R2D for every D2R.

Observation 3: Implicit failure detection mechanism for re-access is more reliable which ensures completion of the procedure started by the device, at least during the subsequent paging triggered for re-access.

Observation 4: Implicit success detection mechanism for re-access, though maybe less reliable but it could be sufficient when the probability of transmission failures in R2D is sufficiently lower than D2R failures.

Observation 5: It improves resources utilization in R2D, since the feedback messages are sent only upon failure detection and because the number of transmissions to indicated failure (NACK) is comparatively less than the transmissions required to indicate success (ACK).

Proposal 2: Support implicit failure detection for Msg1 transmission in D2R during CBRA procedure, if no explicit success indication is sent from the reader within a time window.

Proposal 3: RAN2 to discuss if device should perform implicit success detection or implicit failure detection for Msg3 failure handling (in CBRA) and the subsequent D2R transmissions, considering the following options. And option-2 is more suitable since subsequent R2D is not always required for the successful operation.

Option-1: Implicit failure detection at the device with explicit success indication (ACK) from the reader, for higher reliability to recover or re-access upon all the possible failure scenarios.

Option-2: Implicit success detection at the device with explicit failure indication (NACK) from the reader, for efficient resource utilization in R2D and with an assumption that the failures in R2D is sufficiently lower than the failures in D2R.

# References

[1]		RP-250796; New Work Item: Solutions for Ambient IoT (Internet of Things) in NR.