3GPP TSG-RAN WG2 Meeting #129bis	R2-2502579

Wuhan, China, 7th – 11th April, 2025

Agenda item:	8.7.4.1

Source:		Lenovo

Title:			Remaining details on Intra-UE prioritization/LCP enhancements

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss enhancements to the uplink scheduling for XR traffic. A detailed look is taking at the  remainig details of the intra-UE prioritization and the logical channel prioritization (LCP) procedure.

# Discussion

In RAN2#129 the following agreements were reached:

1. Only one additional priority is configured to an LCH for LCP enhancement. 2. We keep an existing agreement (remaining time threshold is configured per LCH) 3. There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. 4. Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact 5. FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination 6. No additional PBR is needed for priority adjusted data 7. FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

Intra-UE prioritization

Similar to the considerations for the LCP procedure, also for the intra-UE prioritization mechanism which was introduced in Rel-16 the remaining time/delay of the data should be considered as a decisive parameter determining the priority of an UL grant. According to the current specification the priority of an overlapping UL grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU is yet to be generated) in the MAC PDU. Essentially up to Rel-19 only the configured “default” logical channel priority is considered to determine which UL grant is prioritized/deprioritized.

However, as agreed during RAN2#129, UE shall also use the additional priority when determining the priority of an UL grant. It is FFS whether such agreed behaviour for the intra-UE prioritization case should be explicitly captured in the specifications.

In our view, we think it is better to explicitly capture the intended behaviour in specifications, e.g. specify which priority is used for the determination of the priority of an UL grant, because now a LCH may be configured with two LCH priorities. Furthermore, we think that RAN2 needs to further discuss which of the potentially two priorities configured for a LCH should be used for determining the priority of an UL grant. It should be noted that a UE may apparently use both configured LCH priorities during LCP procedure. As agreed in RAN2#128 and captured in the draft MAC CE, UE may fall back to the default priority for the second round of LCP procedure when having applied the additional priority during the first round of LCP. Question is now, which priority UE should use for the determination of the corresponding UL grant.

For the retransmission case, e.g. LCP procedure has been already executed by UE, UE could essentially either use the additional priority (since it was used in the first round during LCP) or the default priority, when it was used during the second round, for determining the priority of the UL grant. Since UE has already performed LCP and is aware of the used priorities, both alternatives are same from UE complexity point of view. Here, we think that UE should rather use the additional priority for the determination of the UL grant priority only if UE has used the additional priority during the complete LCP (e.g. first and second round of LCP).

For the initial transmission case, e.g. LCP has not been executed yet, it will be easier from UE implementation/complexity perspective to use the additional priority of an LCH for determining the priority of the corresponding UL when UE uses the additional priority in the first round of LCP, e.g. regardless of which priority is used during second round of LCP. Otherwise, UE would need to estimate/be aware of the result of LCP in order to know whether additional priority is also used in the second round of LCP.

Proposal 1: RAN2 to agree that it should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant.

Proposal 2: RAN2 to agree that UE should use for the retransmission case the additional priority of a LCH for determining the priority of a corresponding UL grant only if the additional priority was used during the complete LCP, i.e. first and second round of LCP.

Proposal 3:  RAN2 to agree that UE should use for the initial transmission case the priority that will be used for a LCH during first round of LCP for determining the priority of the corresponding UL grant.

For the HARQ process prioritization case, we think similiar behaviour should be applied as for the UL grant prioritization case. It should be noted that for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions. Therefore, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant.

Proposal 4: RAN2 to agree that UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant.

For determining the priority of an SR, we don’t think that an additional priority configured for a LCH should be considered, since for SR/BSR triggering no LCP procedure is involved. It will be anyway difficult for UE to predict which LCH priority is used for the corresponding UL grant/LCP in response to a triggered SR. However, when comparing the priority of an SR against a priority of an UL grant, the behavior as described above should be applied for determining the priority of the UL grant.

Proposal 5: RAN2 to agree that an additional priority configured for a LCH triggered a SR is not considered for determining the priority of an SR. Additional priority is only considered for determining the priority of an UL grant, e.g. for the case where SR and UL grant collide.

Enhanced LCP procedure

The current LCP procedure has two stages, UE allocates in the first round uplink resources based on Bj in a decreasing logical channel priority order. If any UL resources remain after the first round, UE allocates the remaining resources regardless of Bj in a decreasing priority order during the second round.  RAN2 discussed whether to reconsider Bj when a higher LCH priority is applied during first round of LCP for delay-critical data. It was argued that if the data volume of the delay critical data is larger than Bj, it may happen that not the complete delay-critical data may be multiplexed into the TB during the first round of LCP which would diminish the benefits of the enhanced delay-aware LCP procedure. One possibility is to leave it up to UE implementation to use a negative value for Bj. Another alternative is to specify the behaviour in specifications so that it is easy to control and test. For example, to allow considering a LCH using an additional priority during first round of LCP regardless of whether the corresponding Bj is negative while the remaining time is less the configured threshold, in order to maximise the transmission of delay-critical data.

Proposal 6: RAN2 to agree to explicitly specify that a LCH using an additional priority during LCP is considered for the first round of LCP regardless of whether Bj is negative while the remaining time is less the configured threshold.

We think that NW should control/configure if and for which LCHs the remaining time should be considered when executing the LCP. For example, the remaining time doesn’t need to be considered during LCP for every kind of service/LCH, e.g. only for specific LCHs carrying XR traffic. As another example, the enhanced LCP procedure may be only used when DSR has been reported to the gNB in order to ensure that delay-critical data – as reported in the DSR – is transmitted in the corresponding UL grant, e.g. received in response to the DSR. It should be also further discussed whether the enhanced LCP procedure is explicitly activated/deactivated by NW, e.g. similar to the PSI-based discarding procedure by a MAC CE.

Proposal 7: NW should control/configure whether the enhanced LCP procedure taking remaining delay/time into account is used or not by the UE. RAN2 to further discuss whether enhanced LCP procedure is explicitly activated/deactivated by NW.

The PDU set importance based PDCP discard has been introduced for Rel-18 XR. In case of UL congestion, PSI based discard can be activated, and a lower discard timer value, e.g. discardTimerForLowImportance is applied to the low importance PDCP SDUs.  When PSI based discarding is activated, UE only supports DSR reporting for high importance data. Following the same principle, when LCP procedure considers the remaining time/delay of data prioritization, the enhancement may also only be applied to high importance data, e.g. delay-critical LCH that has high importance data in its buffers.

Proposal 8: RAN2 to discuss whether the LCP enhancements are only applicable for high importance packets in case of UL congestion, e.g., PSI-based discard is activated.

# Conclusion

In this contribution, we discuss enhancements to the uplink scheduling. We have the following proposals:

Proposal 1: RAN2 to agree that it should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant.

Proposal 2: RAN2 to agree that UE should use for the retransmission case the additional priority of a LCH for determining the priority of a corresponding UL grant only if the additional priority was used during the complete LCP, i.e. first and second round of LCP.

Proposal 3:  RAN2 to agree that UE should use for the initial transmission case the priority that will be used for a LCH during first round of LCP for determining the priority of the corresponding UL grant.

Proposal 4: RAN2 to agree that UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant.

Proposal 5: RAN2 to agree that an additional priority configured for a LCH triggered a SR is not considered for determining the priority of an SR. Additional priority is only considered for determining the priority of an UL grant, e.g. for the case where SR and UL grant collide.

Proposal 6: RAN2 to agree to explicitly specify that a LCH using an additional priority during LCP is considered for the first round of LCP regardless of whether Bj is negative while the remaining time is less the configured threshold.

Proposal 7: NW should control/configure whether the enhanced LCP procedure taking remaining delay/time into account is used or not by the UE. RAN2 to further discuss whether enhanced LCP procedure is explicitly activated/deactivated by NW.

Proposal 8: RAN2 to discuss whether the LCP enhancements are only applicable for high importance packets in case of UL congestion, e.g., PSI-based discard is activated.

# References

### TS 38.321 v18.0.0