3GPP TSG-RAN WG2 Meeting #129bis	R2-2502583
Wuhan, China, April 7 – 11, 2025

Agenda Item:	8.7.4

Source:	InterDigital

Title:	LCP enhancement for XR

Document for:	Discussion, Decision

# Introduction

The Rel-19 WID on XR the following objective on LCP enhancements:

Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

- Specify additional Logical Channel priority handling using delay/deadline information of packets;

This contribution address remaining open issues on how to realize the additional LCH priority in LCP when the UE has delay critical data to transmit.

# Discussion

The following agreements on LCP enhancements were made in RAN2#127bis, 128, and 129 [2]:

Agreements on LCP enhancements - Only one additional priority is configured to an LCH for LCP enhancement. - We keep an existing agreement (remaining time threshold is configured per LCH) - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact - FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination - No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold. Agreements on LCP enhancements - As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold. Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

One remaining FFS whether/how additional priority impacts intra-UE. It was agreed that Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact. Intra-UE prioritization was introduced in R16 URLLC for handling overlapping grants and UCI/SR resources. Intra-UE prioritization can be a separate capability from XR.

Another FFS is whether Intra-UE prioritization shall use the additional LCP priority for SR priority determination. For an SR-PUSCH overlap with intra-UE prioritization, the priority of the SR is determined by the LCH that triggered the BSR. When the overlap happens, the priority of the SR should not be changed as it competes with priority of other PUSCH data. For such reason, the additional priority introduced in LCP for the LCH should not be used outside the LCP procedure.

Proposal 1:	“Additional priority” for the LCH is not used outside the LCP procedure. MAC is not optimized for applying the configured additional priority for LCP in intra-UE prioritization, including SR priority determination.

Another open issue is to whether to also enhance PBR and Bj when the LCH’s priority is upgraded per above. One scenario that is possible is Bj can be negative for the logical channel, which results in not having the LCH served in LCP. in the current design, if Bj is negative, the delay-critical LCH will not be served/selected regardless of how the new value of priority, which can happen for bursty traffic with limited BSD. One proposed option is to temporarily increase the PBR while the remaining time is less than the threshold, to help more delay-critical data to be transmitted. However, it is ambiguous how Bj or PBR can be temporarily raised and how to configure a value it should be raised to. Increasing the PBR to infinity, also proposed by other companies, creates a fairness problem to other LCHs and is thus undesirable. Another option is to allow an LCH with an upgraded priority to be transmitted even if Bj is negative, while the remaining time is less the configured threshold. This can guarantee the delivery of delay critical data when the priority is upgraded.

Another alternative is to rely on the network to configure PBR to be large enough to consider bursty traffic from the get-go. If PBR is configured properly, this case may be viewed as corner case, as LCH has not been allocated much UL resources with the default/downgraded priority, and likely the Bj level has accumulated to a required rate once the priority is upgraded. Further, there is always the possibility for the network to allocate a larger grant, where allocating more resources in the round 2 of LCP is possible regardless of Bj, especially given RAN2 agreed to also use the upgraded priority in the second round of LCP.

Proposal 2:	Allow an LCH with an upgraded priority to be transmitted even if Bj is negative, while the remaining time is less the configured threshold.

# Conclusion

The following proposals are provided on LCP enhancement for XR:

Proposal 1:	“Additional priority” for the LCH is not used outside the LCP procedure. MAC is not optimized for applying the configured additional priority for LCP in intra-UE prioritization, including SR priority determination.

Proposal 2:	Allow an LCH with an upgraded priority to be transmitted even if Bj is negative, while the remaining time is less the configured threshold.