3GPP TSG-RAN2 Meeting #129bis	R2-2502585

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.2.3

Source:	Qualcomm Incorporated

Title:	Views on Random Access Aspects of Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

The Rel-19 Ambient IoT WI was approved in [1] and the RAN2 objectives were described as follows:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included. - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

RAN2 #129 made following agreements regarding the A-IoT random access.

Agreements

- For Rel-19, only 3-step CBRA is supported for A-IoT

- We will specify both CBRA and CFRA.

- Re-use the subsequent paging message to trigger re-access.  There is no need to differentiate msg1 resource for initial access vs re-access.

Agreements

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

- RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

In this paper, we will discuss the random-access aspects of Ambient IoT (A-IoT).

# Discussion

## Contention-Based A-IoT Random Access

RAN2 #129 has agreed that only 3-step contention-based A-IoT random access is supported in Rel-19. For contention-based A-IoT random access, RAN2 assumes that the device randomly selects among FDMA occasions as the baseline. Specifically, the A-IoT device should randomly select one specific time-frequency resource as the A-IoT access occasion configured from the initial A-IoT paging message. If the A-IoT device is short of energy or the device is not ready for responding to this A-IoT paging, the A-IoT device may select the access occasion from a subsequent A-IoT paging message with the same service request (transaction ID) to perform A-IoT random access.

Proposal 1: The A-IoT device randomly selects the access resource from the A-IoT paging message (initial or a subsequent A-IoT paging message corresponding to the same service request).

## Contention-Free A-IoT Access

For contention-free A-IoT access, the scheduled A-IoT device should be configured with a dedicated D2R resource. From A-IoT device perspective, if the resource is dedicatedly configured or the resource configured can be one-to-one mapped to the device, the A-IoT device selects the indicated D2R resource and performs contention-free A-IoT access. When A-IoT paging indicates a device specific identifier and the paging message includes an A-IoT access resource (i.e., D2R resource) for that device, contention-free A-IoT access is performed.

Proposal 2: When A-IoT paging includes a device specific identifier and provides the A-IoT access resource for that A-IoT device, contention-free A-IoT access is performed.

In contention-free A-IoT access, the device ID and/or A-IoT upper layer data is sent in the first message via the D2R by using the dedicated D2R resource. The A-IoT device should use the identifier indicated in A-IoT paging as a response to the Reader which sends this paging message. The random ID is neither needed nor sufficient in the contention-free case. If random ID is used in contention-free, it will cause ambiguity between two Readers which may provide the same D2R resource to different A-IoT devices in the same vicinity (see our companion paper [2] for detailed illustration). The “unification” between contention-free and contention-based A-IoT random access is therefore neither beneficial nor necessary.

Proposal 3: In contention-free A-IoT access, the device ID is sent in the first D2R message. Random ID in the first D2R message is neither needed nor sufficient for contention-free A-IoT access.

## Failure Indication and A-IoT Re-access

In the A-IoT SI phase, it was agreed that the reader can use an optional explicit R2D failure/success feedback indication to trigger the re-access. For the R2D failure indication, it can be used to address the A-IoT Msg3 transmission failure. Since the A-IoT Msg3 is scheduled by A-IoT Msg2, Reader may retransmit the same A-IoT Msg2 including the random ID, and A-IoT device associated with this random ID may consider the previous A-IoT Msg3 was unsuccessfully (re)transmitted and should follow the A-IoT Msg2 to retransmit A-IoT Msg3. Thus, the R2D failure indication of A-IoT Msg3 is the retransmission of A-IoT Msg2 including the random ID or AS ID (if assigned in A-IoT Msg2).

Proposal 4: The R2D failure indication of A-IoT Msg3 is indicated by the A-IoT Msg2 retransmission including the previously used identity in Msg2 (i.e., random ID or AS ID, if assigned in A-IoT Msg2).

In different scenarios, the A-IoT Msg2 may echo the received random ID or may assign AS ID which is pending on the AS ID discussion. The A-IoT device will retransmit the A-IoT Msg3 if A-IoT Msg2 is received again.

Proposal 5: The A-IoT device retransmits the A-IoT Msg3 if the A-IoT Msg2 including the random ID or AS ID is received again.

RAN2 #129 agreed the NACK based solution to trigger the A-IoT re-access. It is FFS on the details on whether the NACK is one explicit feedback indication or a timer-based solution. For timer-based solution, the disadvantage is that the A-IoT device has to maintain a timer and some additional resources have to be reserved for the timer. Besides that, when the timer is configured, the start or stop is another issue. It is difficult to configure a suitable length for the timer considering the tradeoff between the latency and efficiency. Thus, the timer-based solution is not preferred considering the complexity for A-IoT device.

Observation 1: The timer-based NACK indication solution causes complexity to the A-IoT device.

For the explicit NACK feedback solution, one simple solution is that one additional flag is introduced in the retransmitted A-IoT Msg2 message. If A-IoT device receives the retransmitted A-IoT Msg2 with the new flag (e.g., set to 1), it implies the previous A-IoT Msg3 retransmissions are still unsuccessful. Reader does not want the A-IoT device to retransmit A-IoT Msg3 anymore and requires the A-IoT device to prepare for the A-IoT re-access in the next A-IoT paging message. If the new flag is set to 0, the A-IoT Msg3 retransmission is triggered based on the retransmitted A-IoT Msg2.

The A-IoT devices not receiving the NACK indication may assume the current A-IoT random access is successful and will not respond to the next paging message.

Proposal 6: One flag is introduced in the retransmitted A-IoT Msg2 as the NACK indication to trigger A-IoT device to perform A-IoT re-access in the subsequent paging message with the same transaction ID.

Proposal 7: If the A-IoT device does not receive any additional A-IoT Msg2 (for trigger A-IoT Msg3 retransmission) before the next A-IoT paging message, the device will assume the current A-IoT random access procedure is successful and will not respond to the next A-IoT paging message with the same transaction ID.

## A-IoT Random Access Message

RAN1 #120 made the following agreements regarding the A-IoT Msg2.

Agreement Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

According to RAN1 agreements, A-IoT Msg2 supports to include one or multiple echoed random ID received from A-IoT Msg1. It should be possible that A-IoT Msg2 responds to multiple A-IoT devices whose A-IoT Msg1 are successfully received.

Proposal 8: A-IoT Msg2 may contain one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices.

Besides the echoed random ID, the A-IoT Msg2 may include additional information. For example, it may include the A-IoT Msg3 resource allocation but that needs RAN1 further discussion. The AS ID may be assigned by Reader in A-IoT Msg2. Thus, A-IoT Msg2 format can be designed to include multiple MAC subPDUs for multiple A-IoT devices. The format may be like NR MSGB.

Proposal 9: A-IoT Msg2 format can be designed to include multiple MAC subPDUs.

Similar to A-IoT Msg2, the NACK indication message can include ID information for more than one failed A-IoT device(s).

Proposal 10: The NACK indication can include ID information of multiple failed A-IoT device(s).

When A-IoT device receives A-IoT Msg2, the A-IoT device will transmit one A-IoT device ID and AS ID (if already assigned) or same random ID used in A-IoT Msg1 (if AS ID not already assigned).

Proposal 11: The A-IoT device ID along with AS ID (or same random ID used in A-IoT Msg1 if AS-ID not already assigned) is included in A-IoT Msg3 message.

## Summary

Figure 1 gives an example of the A-IoT random access triggered by A-IoT paging.

- When A-IoT device receives initial A-IoT paging message, the A-IoT device randomly selects one access resource/occasion configured by the initial A-IoT paging message and sends A-IoT Msg1. It is possible that the A-IoT device may select the other access/occasion from a subsequent A-IoT paging message corresponding to the same service request due to short of energy status or device not ready to respond to the initial paging message.

- After A-IoT Msg3 transmission, R2D failure indication may carry the retransmitted A-IoT Msg2 which includes the echoed random ID to trigger the (same) A-IoT device to re-send the same D2R message, i.e., A-IoT Msg3.

- The R2D failure indication could be a NACK indication to indicate the failed devices to respond to the subsequent A-IoT paging for re-access.

- If A-IoT device receives the NACK indication and also receives a subsequent A-IoT paging message for the same service request, A-IoT device should perform re-access (starting from A-IoT Msg1) by using the newly assigned access resource/occasion.

- The A-IoT devices which have not responded to the initial A-IoT paging can select from the access resource/occasions assigned by subsequent A-IoT paging to perform A-IoT random access.

- Reader may send the R2D data message including the upper layer command to the inventoried A-IoT device to trigger data communication.

Figure 1 A-IoT random access triggered by A-IoT paging

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1: The timer-based NACK indication solution causes complexity to the A-IoT device.

Proposal 1: The A-IoT device randomly selects the access resource from the A-IoT paging message (initial or a subsequent A-IoT paging message corresponding to the same service request) .

Proposal 2: When A-IoT paging includes a device specific identifier and provides the A-IoT access resource for that A-IoT device, contention-free A-IoT access is performed.

Proposal 3: In contention-free A-IoT access, the device ID is sent in the first D2R message. Random ID in the first D2R message is neither needed nor sufficient for contention-free A-IoT access.

Proposal 4: The R2D failure indication of A-IoT Msg3 is indicated by the A-IoT Msg2 retransmission including the previously used identity in Msg2 (i.e., random ID or AS ID, if assigned in A-IoT Msg2).

Proposal 5: The A-IoT device retransmits the A-IoT Msg3 if the A-IoT Msg2 including the random ID or AS ID is received again.

Proposal 6: One flag is introduced in the retransmitted A-IoT Msg2 as the NACK indication to trigger A-IoT device to perform A-IoT re-access in the subsequent paging message with the same transaction ID.

Proposal 7: If the A-IoT device does not receive any additional A-IoT Msg2 (for trigger A-IoT Msg3 retransmission) before the next A-IoT paging message, the device will assume the current A-IoT random access procedure is successful and will not respond to the next A-IoT paging message with the same transaction ID.

Proposal 8: A-IoT Msg2 may contain one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices.

Proposal 9: A-IoT Msg2 format can be designed to include multiple MAC subPDUs.

Proposal 10: The NACK indication can include ID information of multiple failed A-IoT device(s).

Proposal 11: The A-IoT device ID along with AS ID (or same random ID used in A-IoT Msg1 if AS-ID not already assigned) is included in A-IoT Msg3 message.

# References

- RP-250796, Solutions for Ambient IoT (Internet of Things) in NR.

- R2-2502607, Ambient IoT Paging, Qualcomm.