3GPP TSG RAN WG2 Meeting #129bis	 R2-2502592

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.1.3

Source:	Futurewei

Title:	Discussion on Data Collection for NW-side Model Training

Document for:	Discussion

# Introduction

In meeting #129[8], RAN2 discussion focused on mechanisms and principles for data collection for network side model training. The discussions covered the types of data required for both network-sided and UE-sided models, and identify questions for RAN1.

The discussions were around the following aspects.

- Logging events

- Availability indication

- Content of collected data

- Sending of large data

- Handling of configuration/data during mobility/state transition

- Data collection configuration

The agreements are listed below.

Agreements

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log.

- Low power bit indication is supported

- Data availability indication is supported.  FFS when this would be triggered.

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- FFS how to handle idle/inactive and RLF cases and whether we have a unified.

In this contribution, our discussion will focus on the mechanisms and principles identified for data collection for network side model training during R18, as indicated by the Chair.

# Background

In the TR [2], Table 7.3.1.2-1 lists existing data collection mechanisms available in current RAN specifications for the UE to report measurements to another entity acting as termination point for this data. As indicated by the TR, the analysis/selection of the data collection frameworks should focus on the RRC CONNECTED state for both data generation and reporting.

Table 7.3.1.2-1. Existing data collection methods identified.

| Involved network entity (termination point) | RRC state to generate data | Max payload size per reporting* | Contents to be collected | - End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- |
| Method: Logged MDT |  |  |  |  |  |  |
| TCE/OAM (Data can be utilized by gNB) | IDLE / INACTIVE | <9kbyte | - L3 cell/beam measurements - location information - sensor information - timing information | - Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency****: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message Privacy via user consent |
| Method: Immediate MDT |  |  |  |  |  |  |
| TCE/OAM (Data can be utilized by gNB) | CONNECTED | <9kbyte | - L3 cell/beam measurements - location information - sensor information | - Procedure latency: - Report interval: - 120ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | - Event triggered - Periodic reporting | AS security via RRC message Privacy via user consent |
| Method: L3 measurements |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - 20ms (RRC) | - Event triggered report - Periodic reporting | AS security via RRC message |
| Method: L1 measurement (CSI reporting) |  |  |  |  |  |  |
| gNB | CONNECTED | <1706bit in PUCCH <3840bit in PUSCH | L1 CSI measurement | - Procedure latency: - Report interval: - 4-320 slot for periodic and semi-persistent report - 0-32 slot after reception of DCI for aperiodic report - Air interface signaling latency: - 1 TTI (PUCCH) | - Aperiodic report - Semi-persistent report - Periodic report | No AS security |
| Method: UE Assistance Information (UAI) |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | Assistance information to show UE preference | - Procedure latency: - Upon generation of UE's preference - Air interface signaling latency: - ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Method: Early measurements |  |  |  |  |  |  |
| gNB | IDLE / INACTIVE | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency: - ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| Method: LPP |  |  |  |  |  |  |
| LMF | CONNECTED | <9kbyte | Location information | - Procedure latency: - Latency to get upper layer trigger (for UE triggered) - Or latency to receive network request message (~20ms) - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and LMF | - UE-triggered - Network-triggered | AS security via RRC message |

* The payload size doesn't consider signalling overhead.
** The End-to-End report latency is the latency from availability of the measurement report at the UE side to the availability of the measurement report at the terminated network entity. The time to generate data or perform measurements depends on RAN1/RAN4 specification.
*** Procedure latency is the latency caused by procedures, including procedure to ready for reporting (e.g., entering CONNECTED state, report interval).
**** Air interface signalling latency is the latency to transmit one report, e.g., RRC signalling latency or PUCCH signalling latency.

The TR [2] also captured the following considerations for network-side data collection (Section 7.2.1.3.1).

In addition, A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

Furthermore, and regarding the use cases in this study, the following is considered.

For CSI and beam management use cases, the training of network-side models can consider both gNB and OAM-centric data collection mechanisms.

- The gNB-centric data collection implies that the gNB can configure the UE to initiate/terminate the data collection procedure. The potential impact of L3 signalling for the reporting of collected data should be assessed.

- On the other hand, OAM-centric data collection implies that the OAM provides the configuration (via the gNB) needed for the UE to initiate/terminate the data collection procedure. MDT framework can be considered to achieve this. The potential impact on MDT for RRC_CONNECTED state should be assessed.

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

# Discussion

## Data Logging

### On-Demand Data Logging

In RAN2 meeting #125bis [3], the group reached the following agreements for Data Collection for NW-side Model Training.

- RAN2 supports enhancements to MDT for data collection framework for training.

  - FSS Whether to enhance logged or immediate MDT.

In RAN2 meeting #126 [4], the group agreed to take immediate MDT as the baseline for OAM-centric data collection (see the agreement below).

- Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

In RAN2 meeting #127 [5], the group further agreed the following.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE.

- FFS on event-based data collection/logging.

In RAN2 meeting #127bis [6], the group has agreed the following on data logging.

- Periodic logging is supported for training data collection procedure in R19.

- Event-triggered data logging will be supported.

  - At least radio condition based event triggered logging will be supported.

  - FFS the details of radio condition based event.

  - FFS if other events are supported.

During RAN2 meeting #128 [7],  no agreements on data logging mechanism was made. Agreement achieved so far can be summarized in the following table.

| Mechanism | Periodic | On-Demand | Event-Triggered |
| --- | --- | --- | --- |
| Data Logging | Yes |  | Yes |

As we can see from the table, the support of on-demand logging has not been determined. It is our understanding that on-demand data logging can be useful in some situations. For example, during the training, we may find it necessary to collect data for a specific configuration or a specific time frame during which that has not been collected before. In this example, the task won’t be event-based, and it may not need to be collected periodically. In this case the on-demand approach may be the most suitable one. We therefore think on-demand data logging should be supported.

Proposal 1: Support on-demand data logging and enhancement to logged MDT for data collection for NW-side models.

### Data Logging after Event-triggered Logging

In the post-meeting email discussion after meeting #128 ([POST128][019][AI PHY] NW side data collection), Question #6 asks the following,

- Question 6: Should periodical logging after a measurement event is triggered be supported?

It is our understanding that after a measurement event is triggered, in most cases, the measurement should continuously be done as long as the triggering event conditions are met, for example, L1/L3 measured levels continuously higher/lower than certain preset values. When the measured values go back to normal, the logging will stop. However, there may also be events in which the UE will not change back to its original state before the event was triggered. For example, in beam based events,  a beam may change to become the top-1 beam and never change back again to non-top-1 beam. In this case, the event is a one-time event and the periodic measurement after the event may not have a stopping trigger. It is necessary to define a stopping trigger for these one-time events after being triggered, for example, the duration of measurement after being triggered.

Proposal 2: Further periodic measurement logging after a one-time triggering event should have a predetermined stopping trigger.

## Data reporting

### Event/trigger-based reporting

In RAN2 meeting #126 [4], one of the agreements we achieved was the following, on reporting mechanisms.

Enhance the immediate MDT framework to support periodical reporting.

- FFS whether and what event-based reporting is supported, and

- FFS on network request reporting.

In RAN2 meeting #127[5], we further achieved the following agreement, on the support of on-demand reporting.

- On-demand request from the network is supported.   FFS details on signaling.

Therefore, up to meeting #127, we have agreed to support both enhanced periodical and on-demand (network request) reporting mechanisms. Meanwhile, the support of event-based reporting is still FFS, as agreed below.

- FFS whether AS buffer event-based reporting is supported.

In RAN2 meeting #127bis [6], the group has agreed the following on data logging.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

During RAN2 meeting #128 [7],  no new agreements on data reporting mechanism was made. As a summary, after Meeting #128, the supported methods for logged data reporting are shown in the following table.

|  | Periodic | On-Demand | Event/Trigger-based |
| --- | --- | --- | --- |
| Logged Data Reporting | No | Yes | undecided |

From the table, we can see that whether event-based/triggered reporting is supported has not been decided.

However, the TR [2]  clearly says event/trigger-based reporting approach is part of the “general data collections principles”, as shown below.

A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

In addition, if we looked at the data collection methods identified in R18 (Table 7.3.1.2-1 in Section 2 of this TDoc), we can see event-based reporting mechanisms is identified as one of the reporting types. For example, event-triggered reporting is identified for

- Immediate MDT: collected by TEC/OAM

- L3 measurement: collected by gNB

- LPP: collected by LMF

In fact, for some use cases, such as positioning, event-triggered reporting can be more efficient and up-to-date, because measurements only needs to be done when the UE starts to move, which saves the traffic generated by periodical reporting. In addition, allowing event-based reporting provides flexibility of control and reporting.

Based on this consideration, we propose to support event-based reporting mechanisms.

Proposal 3: Support event/trigger-based reporting mechanisms in addition to on-demand request reporting for NW-side data collection.

### SRB Selection

In RAN2 meeting #127bis [6], it was agreed that low-priority SRB will be used for NW-side data collection.  However, it was not decided whether a new SRB or SRB4 should be used.

According to the standard (TS 38.331), SRB4 is used for transmitting RRC messages containing application layer measurement reporting information on the DCCH. From the usage it was designed for, it matches the data collection purpose very well. To determine whether SRB4 can be used for data collection for NW-side model training, we should look at its limitations and, if there are any, see if these limitations will impact the performance of data collection for AI/ML functionalities/models.

The first thing to consider would be the SRB priority. Currently, while there isn't a strict, fixed priority order for SRBs in 5G NR, the general priority can be inferred from their roles and the criticality of the messages they carry, from SRB0 to SRB4. SRB4 carries RRC messages containing application layer measurement reports. While important for network optimization, it has a lower priority compared to the other SRBs, as its traffic is not as time-critical as control signaling or user data. We think the data collection for AI/ML applications should be at the same level as what SRB4 was designed for (or even higher if later on other types of AI/ML approaches are introduced, e.g., reinforcement learning). Therefore, from the priority aspect, data collection should at least use SRB4; an SRB lower than SRB4 (e.g., if we choose a new SRB5) may impact the performance of data collection.

We then look at the limitations of SRB4 and see if it is suitable for data collection for model training. SRB4 has the following limitations.

- Security Dependency: SRB4 can only be configured after security activation. This means it cannot be used for initial setup or in scenarios where security is not yet established.

- Not Applicable to NB-IoT: SRB4 is not applicable for Narrowband IoT (NB-IoT) devices, which have specific requirements and limitations.

- Resource Consumption: Using SRB4 for measurement reporting can consume additional network resources, potentially impacting overall system performance.

- Delay Sensitivity: Timely delivery of measurement reports is crucial for network optimization. Due to its lower priority, SRB4 may not be suitable for some delay-sensitive measurements.

Based on our understanding, none of these limitations will impact the performance of SRB4 for the purpose of data collection. We therefore propose to use SRB4 for data collection for NW-side model training.

Proposal 4: SRB4 can be used for data collection for NW-side model training.

## UE capability/capacity for data processing and storage

In the TR [2], the “general data collections principles” include principles regarding UE’s capability and capacity for data collection for NW-side models.

A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

We consider this a practical concern because the resources at the UE side are limited, for example, the computational power and memory (the collected data will be in the memory for the BM and Positioning use cases, unlike some of the application-level use cases). We believe it is necessary to provide a mechanism to report a UE’s capability/capacity for storing measured data temporarily for NW-side model training. Logged MDT may only be used when the UE has enough available resources to support it. Otherwise, immediate MDT should be used for data collection.

Proposal 5: Design a mechanism for a UE to report its capability and/or capacity to the NW, such as computational power, available memory and storage space. NW decides on whether immediate MDT or logged MDT to be used based on the UE’s capability and capacity.

# Conclusions

In this contribution, we continued to present our observations and views on topics related to NW-side data collection. Based on the discussions in the previous sections, our proposals are as follows.

Proposal 1: Support on-demand data logging and enhancement to logged MDT for data collection for NW-side models.

Proposal 2: Further periodic measurement logging after a one-time triggering event should have a predetermined stopping trigger.

Proposal 3: Support event/trigger-based reporting mechanisms in addition to on-demand request reporting for NW-side data collection.

Proposal 4: SRB4 can be used for data collection for NW-side model training.

Proposal 5: Design a mechanism for a UE to report its capability and/or capacity to the NW, such as computational power, available memory and storage space. NW decides on whether immediate MDT or logged MDT to be used based on the UE’s capability and capacity.

# References

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2004

Chair’s notes, R2_126_ChairNotes_24-05-24_final, RAN2 meeting #126, Fukuoka, May 2024

Chair’s notes, R2_127_ChairNotes_ 24-08-23_17-00_eom, RAN2 meeting #127, Maastricht, Aug 2024

Chair’s notes, R2_127bis_ChairNotes_ 24-10-18_15-45-_eom_clean, RAN2 meeting #127bis, Hefei, Oct 2024

Chair’s notes, R2_128_ChairNotes_24-11-22_17-00_eom, RAN2 meeting #128, Orlando, Nov 2024

Chair’s notes, R2_129_ChairNotes_25-02-21_final.docx, RAN2 meeting #129, Athens, Feb 2025