3GPP TSG RAN WG2 Meeting #129bis	 R2-2502593

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.1.4

Source:	Futurewei

Title:	Discussion on Data Collection for UE-side Model Training

Document for:	Discussion

# Introduction

During meeting #129 [8] (Feb 2025), the discussions on data collection for UE-side model training resulted in the agreements below.

- Agreements

- Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration:

- Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop).  FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network.

- The network can provide or release the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

  - The network can decide when to start/stop the data collection and send configuration.

  - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

Before the last meeting, the discussion on data collection for UE-side model training has been paused for two meeting (Oct and Nov 2024 meetings) as RAN2 was waiting for SA responses.

During RANP meeting #106 (Sep 2024), the following were agreed.

- Transfer of data over UP for Solution 2 (CN) in SA2 should be studied.

- Transfer of data over UP for Solution 3 (OAM) can be studied by SA5.

- RAN no longer expects SA2 and SA5 to study CP-based solutions.

- RAN2 continues the studies on AI/ML UE sided data collection in R19

- RAN will make a final decision on way forward taking into account SA inputs when available.

- RAN will have a checkpoint in Q2, 2025

The last time this agenda item was discussed before RANP meeting #106 was back in RAN2 meeting #127 (Aug 2024) [5], during which the following agreements were made.

Agreements

- Update note above table to add controllability can be achieved via SLA:

Note: RAN2 discussed that, except for the case of standardized data content, the data content visibility and any level controllability could be achieved via SLA (Service Level Agreement). However, SLA is out of RAN2 scope.

- Update note1 in table as follows:

Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection without the need of SLA. This includes at least initiating, terminating, and fully managing data transfer.

- Update Note 2

Note 2: Visibility of data content signifies that the MNO can at least, be aware of, access, and comprehend the data without the need of SLA.

- Table modified by chair during online time uploaded in R2-2407785 is agreeable and will be reviewed over email for editorial corrections

The CR to TR 38.843 related to the agreements made was captured in [7].

##### 7.2.1.3.2	Data collection for UE-side model training

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

The options listed above were analysed to understand potential specification impact. The analysis included aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved, MNO controllability and visibility over the collected data, etc. The result of this analysis can be found below in Table 7.2.1.3.2-1.

It is worth noting that different options for the data content visibility were discussed. The different levels of data content visibility are captured in the Note 3 in the Table 7.2.1.3.2-1.

Note:	RAN2 discussed that, except for the case of standardized data content, the data content visibility and any level of controllability could be achieved via SLA (Service Level Agreement). However, SLA is out of RAN2 scope.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | FFS: level of controllability (Note 5) (Note 1) | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure, FFS other procedures | Via RRC procedure |
| Possible Options for data content visibility in MNO (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. UE proprietary information can only be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of possible MNO controllability and visibility via Option 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 cannot reach consensus on the feasibility of UP tunnel in Options 2 and 3 |  |  |  |  |

Related to privacy, it has been stressed in RAN2 the importance that any potential mechanism to collect UE side data for model training purposes (including the options 1a, 1b, 2, 3 listed above) must comply with privacy protection regulations, requirements, laws and/or policies. An informative Annex is included at the end of this document capturing examples of privacy concerns for different stakeholders participating in the discussion.

In this contribution, we discuss remaining issues of data collection for UE-side model training.

# Discussion

## Definitions of Terminologies

### Termination Entities

So far, we have been using the term “first termination entity” but we don’t have a definition of it. The rapporteur suggested initial definition as below.

The "(First)termination entity" refers to the entity that receives and stores data transmitted from the UE, which possesses the authority to oversee the subsequent handling of this data, such as data cleaning, forwarding, sharing, and analysis, among others, in compliance with privacy policies, security protocols, and any regulatory compliance requirements. The purpose of introducing the term ‘(first) termination entity’ is to emphasize the entity’s role in management and possession of the collected data.

As the result of the agreement, first termination entity for each option was documented in Table 7.2.1.3.2-1 (quoted below).

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |

During the discussion, ambiguity arose between the terms “termination entity” and “first termination entity”. In early discussions, the term used was “termination entity”. Many companies pointed out that we should only discuss the first termination entity, which handles data collection. In other words, there may exist the second or final termination entity for further data transfer and processing. It is our opinion that these two terms need to be clearly defined to avoid ambiguity in future discussions.

Here are our proposed definitions.

Proposal 1: The first termination entity refers to the first network element/function that receives and stores data transmitted from the UE.

- A first termination entity possesses the authority to oversee the subsequent handling of this data, such as data cleaning, forwarding, sharing, and analysis, among others, in compliance with privacy policies, security protocols, and any regulatory compliance requirements. I

- A first termination entity may not be the place where UE-side model is trained, in which case the entity is a final termination entity.

Proposal 2: The (final) termination entity refers to the place where the UE-side model training is done.

- Depending on the data collection procedure, a final termination can also be the first termination entity and vice versa.

## Configuration of data collection

With UP for Option 2 and Option 3 data collection, the configuration of data collection should be done by the network, CN or OAM, depending on which entity collects the data. Initially, the request for data collection and transfer will come from the server for data collection for UE-side model training. The requests will be communicated to either CN or OAM for them to configure and schedule the data collection and transfer. The CN or OAM will then establish the UP tunnel between the UE and the CN/OAM, and configure and schedule the UE for data collection and transfer.

Proposal 3: RAN2 request SA to study the mechanism for the CN/OAM to interact with the server for data collection for UE-side model training in order to understand the request for data collection and transfer.

## Data Transfer over UP

RAN plenary meeting #106 decided that RAN2 continues the studies on AI/ML UE sided data collection in R19. While originally determined as alternative solutions to CP-based solutions, UP-based solutions are now in the centre of discussion for data collection for UE-side model training.

The UP is able to handle large amount of data comparing to the CP which has scalability issues. However, there are also some common concerns about transferring collected data over the UP, for Option 2 (via CN) and Option 3 (via OAM). Some of these concerns are listed below.

- The feasibility of supporting UP tunnels in options 2 (i.e., to the CN) and 3 (i.e., to the OAM) with current architecture/system.

- The standardization effort required to make it work, if Point 1) is valid.

- The mechanisms enabling CN and/or OAM to transfer the collected data to the final termination entities are not clear, if CN and OAM are not the final termination entities (the training servers are not inside CN or OAM).

- The protocol layer to be used for transferring collected data.

We discuss issues/concerns common to both Option 2 and Option 3 in Section 2.2 and issues specific to Option 2 and Option 3 in their own sub-sections respectively, i.e., Section 2.2.1 for Option 2 and Section 2.2.2 for Option 3.

### The architecture issues for UP support

In 5G networks, user data flows from the UE to the gNB and then to the UPF over the N3 interface. The UPF is responsible for forwarding this data to external data networks through the N6 interface.

The protocol used for tunneling user data between the gNB and UPF is typically GTP-U (GPRS Tunneling Protocol – User Plane).

For Option 2, the UE does not establish a direct GTP-U tunnel with the Core Network. Instead, user plane tunnels are set up between the gNB and UPF. This architecture ensures proper routing, QoS enforcement, and mobility management. However, it does not support direct UP tunneling between the UE and the CN.

Option 3 has the similar situation; there is no direct UP tunnel between a UE and the OAM system in 3GPP-defined networks. UP tunnels are strictly for data transmission between the UE and external data networks via intermediate network functions like gNB and UPF, while OAM operates at a management level to configure and monitor these tunnels.

In summary, if UP is supported and the server for data collection for UE-side model training is not in CN or OAM, the data transfer path would be:

  - For option 2: UE  UPF (CN)  (local transfer to) CN  Server for data collection

  - For option 3: UE  UPF (CN)  OAM  Server for data collection

We note that most of these issues involve SA investigations and revisions to the standards.

Observation 1: Current 3GPP architecture does not support AI/ML data collection for UE-side model training in the UP. Resolving these issues involves SA investigations and revisions to the current standards.

### The necessity of a new protocol layer

As we see it, the collected data from UE for model training is not different than other user data. For example, it does not have more stringent QoS requirements such as lower latency (based on RAN1 evaluation). In this case, we don’t think a new protocol layer needs to be introduced for collected data transfer.

Proposal 4: It is not necessary to introduce a new protocol layer for collected data transfer.

### Logging of collected data at the UE

In Agenda Item 8.1.3, RAN2 has agreed to support both periodic and event-triggered logging of measured data. It is our understanding that on-demand data logging can also be useful in some situations. For example, during the training, we may find it necessary to collect data for a specific configuration or a specific time frame during which that has not been collected before. In this example, the task won’t be event-based, and it may not need to be collected periodically. In this case the on-demand approach may be the most suitable one. We therefore think on-demand data logging should also be supported.

Proposal 5: Support periodic, event-triggered, and on-demand data logging for data collection for UE-side model training.

### Transfer/reporting of collected data to the first termination entity

Once logged, the collected data will need to be transferred/reported to the configured first termination entity. We suggest supporting the following data reporting mechanisms.

- Periodic reporting

- Immediate reporting

- On-demand reporting

- Event-trigger reporting

We believe each method has its scenario. For example, immediate reporting allows a UE to transfer the data immediately after it is collected to the first termination entity. This releases the memory/storage used immediately so that more data can be logged, which is especially useful for UEs running short for memory/storage. As another example, periodic reporting is a good match for periodic data logging; when the periods are set to the same, the logged data can be transferred out from the UE right after it is collected.

Proposal 6: Support periodic, immediate, event-triggered, and on-demand data transfer/reporting for data collection for UE-side model training.

### Standardization effort: CN domain vs OAM domain

As mentioned in Section 2.2, in the context of 3GPP standards, there is no direct UP tunnel established between the UE and the OAM system for data transmission. The OAM system primarily operates at the management level and does not directly interact with the UE in the user plane.

In 5G networks, UP data is transmitted between the UE and the CN through intermediary nodes such as the gNB and UPF. This data is encapsulated using GTP-U over interfaces like N3 between gNB and UPF. As UPF is part of the CN, it is easier for the collected data to reached CN than the OAM which is not involved in the data flow at all. Therefore, we prefer Option 2 over Option 3 if we can only pick one from these two options to standardize.

Proposal 7: Priority is given to Option 2 over Option 3 if only one of these two options can be standardized.

### Data transfer from CN/OAM to the data collection/training server

So far in RAN2 we have not had much discussion on the data transfer from CN/OAM to the server for data collection for UE-side model training (i.e., the final termination entity), which could be outside the 3GPP network. When the training server is outside the CN or OAM, the CN or OAM node needs to communicate with and configure the data collection/training server in order to activate/disactivate the data collection procedure and to start/stop/schedule the transfer of the collected data. While most of these activities need to be considered by the SA work groups, we think RAN2 needs to provide service requirements to the NG-RAN and SA groups.

Proposal 8: RAN2 to request SA to study the interaction between CN/OAM and the server for data collection for UE-side model training to support the configuration for data collection and transfer.

## NG-RAN Involvement in Data Collection/Transfer

### MNO’s Visibility of Data Content

MNO’s visibility of data content refers to the extent to which the MNO understands what the data is or is about. Many companies agreed that 'visibility' of data content entails the MNO's ability to at least be aware of, access, and comprehend the data being transferred to some degree. However, it is our understanding that for some data, the MNO may not have the right to access and comprehend (for example, those related to UE’s privacy and security).  In other words, there are concerns related to regulatory compliance for these types of activities. In this case, some of the UE data needs to be pre-processed, for example, anonymized, before being sent to the data collection server.

Proposal 9: When collecting data for UE-side model training, data needs to be pre-processed to remove sensitive and privacy information associated with the UE.

# Conclusions

In this contribution, we continued to present our observations and views on data collection for UE-side model training. Based on the discussions in the previous sections, our proposals are as follows.

Observation 1: Current 3GPP architecture does not support AI/ML data collection for UE-side model training in the UP. Resolving these issues involves SA investigations and revisions to the current standards.

Proposal 1: The first termination entity refers to the first network element/function that receives and stores data transmitted from the UE.

- A first termination entity possesses the authority to oversee the subsequent handling of this data, such as data cleaning, forwarding, sharing, and analysis, among others, in compliance with privacy policies, security protocols, and any regulatory compliance requirements. I

- A first termination entity may not be the place where UE-side model is trained, in which case the entity is a final termination entity.

Proposal 2: The (final) termination entity refers to the place where the UE-side model training is done.

- Depending on the data collection procedure, a final termination can also be the first termination entity and vice versa.

Proposal 3: RAN2 request SA to study the mechanism for the CN/OAM to interact with the server for data collection for UE-side model training in order to understand the request for data collection and transfer.

Proposal 4: It is not necessary to introduce a new protocol layer for collected data transfer.

Proposal 5: Support periodic, event-triggered, and on-demand data logging for data collection for UE-side model training.

Proposal 6: Support periodic, immediate, event-triggered, and on-demand data transfer/reporting for data collection for UE-side model training.

Proposal 7: Priority is given to Option 2 over Option 3 if only one of these two options can be standardized.

Proposal 8: RAN2 to request SA to study the interaction between CN/OAM and the server for data collection for UE-side model training to support the configuration for data collection and transfer.

Proposal 9: When collecting data for UE-side model training, data needs to be pre-processed to remove sensitive and privacy information associated with the UE.

# References Intelligence

RP-234039, New WID on Artificial (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2024

Chair’s notes, R2_126_ChairNotes_24-05-24_final, RAN2 meeting #126, Fukuoka, May 2024

Chair’s notes, R2_127_ChairNotes_24-08-23_17-00_eom, RAN2 meeting #127, Maastricht, Aug 2024

Chair’s notes, R2_128_ChairNotes_24-11-22_17-00_eom, RAN2 meeting #128, Orlando, Nov 2024

R2-2407807, RAN2 inputs to TR38.843, Maastricht, Aug 2024

Chair’s notes, R2_129_ChairNotes_25-02-21_final.docx, RAN2 meeting #129, Athens, Feb 2025