3GPP TSG-RAN WG2 Meeting #129bis	R2-2502595

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.1.4

Source:	Nokia

Title:	Data collection for UE side models

WID/SID:	FS_NR_AIML_Air-Ph2 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

SID for AI/ML for NR air interface in [1] involves objective to examinate UE-sided model training data as follows:

Study CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

- For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

- Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

In accordance with [2], RAN#106 down selected studied options in TR 38.843, to focus study for UE side data collection to Options 2 and 3, as they fulfil agreed criteria for UE-side model training, outlined in [3].

This contribution includes observations and proposals on the data collection for UE-side models, focusing on AI/ML-based beam management, in particular, to align with the choice of gathering and transferring the data from the UE under Option 2 and Option 3. Moreover, the contribution also highlights the concerned related to AI/ML based positioning.

# 2	Discussion

## 2.1	Scope for Data Collection for UE side models

Data collection for UE side models focuses on gathering raw information from the device to train and refine ML models in offline training process. Based on the narrowed down options by RAN#106 [2], consideration includes different endpoints for the data being sourced from the UE. Potential termination points are: gNB or OAM or CN entity, depending on the Rel-19 use case applicability (destinations being either gNB or OAM for Beam Management or LMF for Positioning). In any case, the data source for the data collection for training UE-side models was supposed to be the UE.

However, we observe differences may exist in terms of the primary data source. Specifically, regarding Positioning, an agreement reached by RAN1 states that label data can be produced by LMF and sent to the UE:

RAN1 #120: Agreement For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. - From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

Consequently, the data might be originally produced in the LMF and sent to the UE for further delivery onwards (pending data transfer option selection). This alternative creates a possible scenario, that not all information might be produced at the UE itself.

Observation 1: For positioning case 1, RAN1 agreements consider that data sourced from the UE for data collection does not have to be entirely generated in the UE.

Alternatively, depending on a data transfer realization for case 1, the UE data might be combined with LMF data, without LMF data (label) delivery to the UE (e.g. CP-based data transfer). Considering numerous options and variety of realizations that are dependent on SA WGS studies outcome, we suggest that Study Item efforts in RAN2 should preliminarily focus on obtaining data in the UE that are to be transferred from UE to a termination point.

Proposal 1: RAN2 study for Data Collection for UE-side models focus on obtaining the training data in the UE and corresponding measurement configuration through RRC for beam prediction and LPP for positioning.

## 2.2	Configuration Plane vs. Data transfer plane

Considering the way of data transfer, TSG RAN has agreed to the following:

From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfillment of visibility and controllability requirements.  Transfer of data over UP for Solution 3 can be studied by SA5

RAN has already concluded that transfer of data over CP for Solution 2 and 3 can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer.  RAN is not expecting SA2 and SA5 to study the transfer of data over CP for now.

UP-based solutions came into play for transferring AI/ML data during Rel-19 studies [4], to overcome a burden to the physical layer or control planes of the gNB and the LMF and to avoid potential AI/ML impacts to traffic essential for normal network operation. UP has been recognized as a candidate for the transmission of larger volumes of data and measurements that can be sent without latency requirements, i.e., those used for data collection for offline training.

In the current stage, before SA study’s conclusions on the options feasibility [7], it is too premature to develop any alternative for data transfer in RAN. Therefore, we suggest waiting for SA2 and SA5 progress on that matter:

Proposal 2: Data collection for UE-side models is always configured to the UE through Control Plane (RRC or LPP) regardless of a use case and which transfer method is decided.

Proposal 3: RAN2 do not progress data transfer aspects until SA WG studies are concluded.

## 2.3	Data Collection Configuration

### 2.3.1	Data Collection Agreements Generalization

Regarding aspects on generalizing Data Collection Configuration, RAN2#129 agreed:

- Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

It becomes ambiguous whether the agreement was meant to generalize for any Rel-19 radio use case, including positioning use case, or if it was meant to generalize the beam management agreements for different data collection solution options that concern gNB and RRC-configured data collection. We note that for Positioning case 1, the existing MO-LR LCS procedure could be reused for UEs to request data collection-specific assistance data from the LMF. Also the existing on-demand positioning SI/SIB and on-demand PRS mechanisms could be reused. However, while for Beam Management the agreements will become applicable for the UE in active RRC connection, it is not clear for Positioning, if the start/stop request would apply also to the UEs without ongoing LPP session. Therefore, to identify most suitable matching option, RAN2 studies should exercise mapping of the agreements to existing procedures for AI/ML positioning case 1.

Proposal 4: RAN2 to clarify whether the RAN2#129 agreed data collection (configuration) principles generalization:

- concern gNB and RRC-involved data collection

- concern LMF and LPP-involved data collection, or

- require mapping to Positioning use case before concluding all the steps apply to LPP-involved data collection.

### 2.3.2	Data Collection Configuration through RRC

To enable the key content for data collection for Beam Management, i.e. the standardized UE measurements, the gNB has to configure signals for measurements, i.e., CSI-RS for the training data collection activation. Although, RAN2#128 already agreed that for data collection for UE side models, the UE can send a request for data collection, the network always needs to enquiry UE capabilities before activation and generate appropriate configuration accordingly. That is, regardless of the pre-configured UE request, the same pre-requisite for activation of data collection for UE-side models can be assumed as for data collection for network-side models. Resources and signals configuration for measurement needs to be reused for both cases: data collection for UE-side and network-side models.

Proposal 5: For Beam Management, the same RRC configuration framework is used to configure measurement resources for data collection for UE-side models as for network-side models

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: For positioning case 1, RAN1 agreements consider that data sourced from the UE for data collection does not have to be entirely generated in the UE.

Proposal 1: RAN2 study for Data Collection for UE-side models focus on obtaining the training data in the UE and corresponding measurement configuration through RRC for beam prediction and LPP for positioning.

Proposal 2: Data collection for UE-side models is always configured to the UE through Control Plane (RRC or LPP) regardless of a use case and which transfer method is decided.

Proposal 3: RAN2 do not progress data transfer aspects until SA WG studies are concluded.

Proposal 4: RAN2 to clarify whether the RAN2#129 agreed data collection (configuration) principles generalization:

- concern gNB and RRC-involved data collection

- concern LMF and LPP-involved data collection, or

- require mapping to Positioning use case before concluding all the steps apply to LPP-involved data collection.

Proposal 5: For Beam Management, the same RRC configuration framework is used to configure measurement resources for data collection for UE-side models as for network-side models

# 4	References

[1] RP-243245, “New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm, December 2024

[2] RP-243316, LS on AI/ML UE sided data collection, RAN, December 2024

[3] RP-242389, “LS on AIML data collection”, RAN, September 2024

[4] TR 38.843 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, Version 18.0.0, 3GPP.

[5] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1

[6] RAN1#116bis Report

[7] S2-2502788, “New SID on Core Network Enhanced Support for AI/ML Ph2