3GPP TSG-RAN2 #129bis	R2-2502604

Wuhan, China, Apr.  7th – 11th, 2025

Agenda item:	8.10.2.1

Source:	Ofinno

Title: 	Discussion on MRO for LTM

Document for:	Discussion

# Introduction

In this contribution, we discuss considerations and enhancements related MRO for LTM.

# 2. Discussion

## 2.1 Measurement information in MRO

An LTM may be performed based receiving a cell switch command MAC CE from network, in response to UE transmitting a measurement report. The measurement report may either be L3 measurement report or L1 measurement report. Current specifications, TS 38.331, supports logging (i.e., storing) of RLF information/ SHR information on detecting RLF/ SHR. This logged information contains the measurement results of the serving cell based on L3 measurements. RAN2 already agreed [2] that for LTM, UE logs available L1 measurement results for the serving cell. However, when logging the measurement results, there is no indication whether the logged measurement is based on L1 measurement result or based on L3 measurement result. Without this information, the network cannot properly adjust its measurement configuration/ cell switch configuration for LTM. To properly adjust this, it is beneficial for the network to know the kind of measurements performed (i.e., L3 measurement/ L1 measurement) for target cell and neighbor cells.

Proposal 1: Indicate in RLF report/ SHR report whether the LTM to the target cell is based on L3 measurement or L1 measurement.

Proposal 2: Indicate in RLF report/ SHR report whether the neighbor cell (i.e., LTM candidate cells) measurements is based on L3 measurement or L1 measurement.

## 2.2 RLF Report

In release 18, an LTM is supported between intra-CU cells. The LTM can support mobility with reduced interruption time. Early UL synchronization is a big component of the LTM that helps in reducing the interruption time during the LTM cell switch. For early UL synchronization, a UE receives TA value of a candidate cell via a cell switch command MAC CE. On receiving TA and considering that CG is configured for the candidate cell, the UE performs an RACH less LTM cell switch procedure to access to the candidate cell. The LTM cell switch procedure may still fail resulting in a handover failure and the UE may generate RLF report. However, sending the RLF report indicating failure of the LTM cell switch procedure does not provide the complete useful information to the network. The network may not have information whether UE received TA for the candidate cell to which the LTM cell switch procedure failed. The failure may be due to wrong TA, weak candidate cell etc. To adjust the LTM related parameters correctly, the knowledge of whether the UE detecting the LTM failure received TA or not is helpful.

Proposal 3: Indicate in RLF-report whether the UE received TA from cell switch command for the candidate cell.

In the current specification, TS 38.331, on detecting an RLF, a UE stores the RLF information in an RLF report. However, the UE does not store the RLF information for all the RLFs that are detected by the UE. The UE stores them only if the UE has the capability to store the RLF information for the RLF report. At the time of the RLF, if a measurement result for a candidate cell for CHO is available, then the UE stores CHO related information in the RLF report only if the UE supports the RLF report for CHO. Similarly, the UE stores DAPS related information in the RLF report only if the UE supports the RLF Report for DAPS. An LTM is independent to any other mobility mechanisms already supported in the specification. In order to store any LTM information in the RLF report, a new capability to indicate a UE supporting storing of LTM information in the RLF report needs to be introduced.

Proposal 4: Introduce a UE capability for indicating whether a UE supports an RLF report for LTM.

## 2.3 SHR Report

RAN2 already agreed [2] to include an explicit indicator in a SHR report whether the successful LTM execution was RACH-less or RACH-based. This is introduced with the intention to enable the network to properly adjust the configuration associated with access the target cell. This information, although useful, seems limited and in incomplete. For example, if a UE performs a RACH-less LTM cell switch procedure to access to the LTM target cell but the time elapsed for T304 is greater than the configured threshold (i.e., configured percentage of T304), the UE triggers the SHR report indicating RACH-less access. Limited to this information, what the network can adjust in its configuration, is very limited. For example, even if the network has provided the UE with a configured grant, it does not know if the UE used it. If the UE transmitted via a dynamic grant, the network does not know that the CG is not valid. The network does not know which SSB associated with the CG is validated by the UE. We think more information for RACH-less access has to be logged by the wireless device in the SHR report.

Proposal 5: Indicate whether RACH-less access is based on CG or DG based on one of:

- Explicit indication whether RACH-less access is based on CG or DG

- Explicit indication whether CG for RAHC-less access is valid or invalid

Proposal 6: Indicate index of SSB where the CG is determined to be valid/ invalid.

RAN2 already agreed [3] log some info to deduce the ltmCandidate (similar like choCandidate) in a SHR report to indicate whether a neighbour cell is an LTM candidate cell or not. We think there are multiple ways to implement this. For example, for each neighbor cell for which measurement result is available and that is an LTM candidate cell, introduce an explicit indication indicating the neighbor cell is an LTM candidate cell. In another example, L1 measurement results can only logged for LTM candidate cells. Based on the logged L1 measurement results (e.g., based on proposal 2) in the SHR report, network can identify if the neighbor cell is an LTM candidate cell or not.

Proposal 7: Indicate whether a neighbour cell is an LTM candidate cell or not. Whether the indication is using explicit or implicit indication can be decided during stage-3 implementation.

## 2.4 TA information

As discussed above, major component of an LTM that helps in reducing the interruption time during cell switch is early UL synchronization. If a TA of a candidate cell is acquired before an LTM cell switch (i.e., a cell switch command MAC CE contains valid TA value for the candidate cell), the interruption time for LTM cell switch is significantly reduce using RACH-less access to the candidate cell (i.e., target cell). If TA of a candidate cell is not acquired before the LTM cell switch (i.e., the cell switch command MAC CE does not contain valid TA value for the candidate cell), there is no reduction in the interruption time as the UE perform RACH-based access to the candidate cell (i.e., target cell).

However, a TA acquisition based on early UL synchronization and LTM cell switch are 2 independent procedures. The success of one does not guarantee success of the other. i.e., performing an early UL synchronization to a candidate cell (i.e., performing RACH in response to receiving PDCCH order to early UL synchronization) does not guarantee that a valid TA will be included in the cell switch command MAC CE. The UE may perform early UL synchronization with multiple candidate cells but can receive cell switch command MAC CE for only on candidate cell. The candidate cell associated with the cell switch command MAC CE may or may not be one of the candidate cells for which early synchronization is performed. The is useful for the network to identify the reason why cell switch is triggered to a cell where early synchronization has not been performed (i.e., cell switch with long interruption time). Similarly, it is useful for the network to identify the candidate cells for which early synchronization is performed but did not receive cell switch command MAC CE. This information helps network to reduce the signaling for the candidate cells where early synchronization is performed without receiving the cell switch command MAC CE. This information helps network to reduce interruption time by configuring early UL synchronization for the target cell (i.e., a candidate cell included in cell switch command MAC CE).

Proposal 8: A UE logs information of whether early UL synchronization is performed for a neighbor cell (LTM candidate cell).

## 2.3 RA Report

In order to optimize the access to a LTM target cell using RACH-based or RACH-less access, network requires some information from the UE. Information of whether TA for candidate cell was acquired before LTM cell switch, may be useful for the network. If the UE had a valid TA, then UE performs RACH-less access to the target cell. Since the RA report from a UE may be requested much later than when the actual access to the target cell was made, we think it is useful to indicate whether the access to the target cell is RACH-based (i.e., existing already) or RACH-less. This will help the network to optimize the behavior or early UL synchronization.

Proposal 9: A UE logs in RA report, that the access (RACH-based/ RACH-less) is for LTM cell switch.

Proposal 10: In RA report, indicate whether a TA of candidate cell was acquired for target cell for LTM cell switch, based on one of:

- Indicate whether RACH-based or RACH-less access

- Indicate TA availability

As discussed above, a UE may perform early UL synchronization with multiple candidate cells. It is possible that the UE does not receive an cell switch command MAC CE to any of these candidate cells. If a cell switch is not initiated to any of these candidate cells, then the random access performed for acquiring early UL synchronization to these cells is not logged. The network cannot optimize the configuration for early UL synchronization. Logging this information is beneficial to the network to optimize configuration/ triggering for early UL synchronization.

Proposal 10: A UE logs RA information for early uplink synchronization in raReport.

Proposal 11: Introduce new raPurpose or early uplink synchronization.

In order to acquire a TA of a candidate cell before the actual cell switch to the candidate cell, a UE performs early UL synchronization to the candidate cell. The UE may receive a PDCCH order indicating RA to the candidate cell. The PDCCH order indicates initial transmission. If a network does not receive the preamble correctly, then the network may inform the UE to perform early synchronization for the candidate cell again. The UE may receive another PDCCH order for RA to the candidate cell. This PDCCH order indicates retransmission). There is no configurable maximum number to limit the number of attempts a UE performs for early UL synchronization for a candidate cell. To optimize configuration and triggering of early UL synchronization, it is useful to indicate the number of times the UE performed ETA to the same candidate cell.

Multiple solutions are possible. In a first example, the UE may log separate entries in the raReport for each of the early synchronization attempts to a candidate cell. Each entry may include multiple parameters associated with RACH for the corresponding early UL synchronization. In a second example, the UE may log a single entry in the raReport for the early synchronization attempts to a candidate cell. The UE may include the number of times UE attempted early UL synchronization attempts (i.e., number of RACH attempts) to the candidate cell. The most useful information for the network may be the parameters associated with RACH for the last attempt of early UL synchronization (i.e., last RACH attempt).

Proposal 12: A UE logs multiple early UL synchronization attempts to a candidate cell using a single entry in raReport. UE logs the number if early UL synchronization attempts(i.e., RACH attempts) made to the candidate cell. UE logs RA information corresponding to the last attempt.

A TA acquisition based on early UL synchronization and LTM cell switch are 2 independent procedures. A UE may perform early UL synchronization with a candidate cell but can receive a cell switch command MAC CE sooner or later. Based on how long it takes for the cell switch command MAC CE to be received after performing early UL synchronization to the candidate cell (i.e. time elapsed from performing early UL synchronization to receiving cell switch command MAC CE, the TA value of the candidate cell may be valid or invalid (i.e., the UE may or may not receive the TA value). This may also have impact on the measurement configuration associated with the measurement reporting of candidate cell.

Proposal 13: A UE logs the time elapsed between receiving PDCCH order for triggering RACH (for early UL synchronization) on a candidate cell, and receiving LTM cell switch command MAC CE for cell switch to the same candidate cell.

Proposal 14: RAN2 to discuss whether the first PDCCH order or the last PDCCH order for early UL synchronization on the candidate cell is considered for logging the time elapsed.

# 3. Conclusion

The contribution discussed aspects related to MRO for LTM. The following proposals are made:

Measurement information in MRO

Proposal 1: Indicate in RLF report/ SHR report whether the LTM to the target cell is based on L3 measurement or L1 measurement.

Proposal 2: Indicate in RLF report/ SHR report whether the neighbor cell (i.e., LTM candidate cells) measurements is based on L3 measurement or L1 measurement.

RLF Report

Proposal 3: Indicate in RLF-report whether the UE received TA from cell switch command for the candidate cell.

Proposal 4: Introduce UE capability for indicating whether UE supports RLF report for LTM.

SHR Report

Proposal 5: Indicate whether RACH-less access is based on CG or DG based on one of:

- Explicit indication whether RACH-less access is based on CG or DG

- Explicit indication whether CG for RAHC-less access is valid or invalid

Proposal 6: Indicate index of SSB where the CG is determined to be valid/ invalid.

Proposal 7: Indicate whether a neighbour cell is an LTM candidate cell or not. Whether the indication is using explicit or implicit indication can be decided during stage-3 implementation.

TA information

Proposal 8: A UE logs information of whether early UL synchronization is performed for a neighbor cell (LTM candidate cell).

2.3 RA Report

Proposal 9: A UE logs in RA report, that the access (RACH-based/ RACH-less) is for LTM cell switch.

Proposal 10: In RA report, indicate whether TA of candidate cell was acquired for target cell for LTM cell switch, based on one of:

- Indicate whether RACH-based or RACH-less access

- Indicate TA availability

Proposal 10: A UE logs RA information for early uplink synchronization in raReport.

Proposal 11: Introduce new raPurpose or early uplink synchronization.

Proposal 12: A UE logs multiple early UL synchronization attempts to a candidate cell using a single entry in raReport. UE logs the number if early UL synchronization attempts(i.e., RACH attempts) made to the candidate cell. UE logs RA information corresponding to the last attempt.

Proposal 13: A UE logs the time elapsed between receiving PDCCH order for triggering RACH (for early UL synchronization) on a candidate cell, and receiving LTM cell switch command MAC CE for cell switch to the same candidate cell.

Proposal 14: RAN2 to discuss whether the first PDCCH order or the last PDCCH order for early UL synchronization on the candidate cell is considered for logging the time elapsed.

# References

- Chairman Notes: TSG RAN WG2 Meeting #129

- Chairman Notes: TSG RAN WG2 Meeting #127bis

- Chairman Notes: TSG RAN WG2 Meeting #127