3GPP TSG-RAN WG2 Meeting #129bis	R2-2502610

Wuhan, China, April. 7th – 11th, 2025

Agenda Item:	 8.13.3

Source:	 Huawei, HiSilicon

Title:	 Control Plane Procedures for Multi-hop Relay

Document for:  Discussion and Decision

# 1	Introduction

In RAN2 #129 meeting, we had good discussion on the control plan procedures for multi-hop relay, and reached the following agreements on aspects including initial access, system information, paging, and SRAP. The FFSs are highlighted in yellow [1].

RAN2#129 agreements [1]:

Agreements on Control Plane Baseline Solution: RAN2 assumes that discovery and PC5 connection establishment can be performed in each intermediate UE prior to processing the received RRCSetupRequest by the remote UE. The related FFS can be removed from the stage 2 description. For the baseline solution, the last relay sending SUI on behalf of other relay UEs is not supported. The related FFS can be removed from the stage 2 description. For the baseline solution, Rel17 SUI message and format is re-used. No further clarification in stage 2 description is needed to clarify that a relay UE can establish its RLC channel for relaying of SRB1 from its immediate child node during its own connection establishment and details will be clarified in stage 3 (i.e., follow legacy U2N procedure from Rel-17 to set up the RLC channel for SRB1). The related FFS can be removed from the stage 2 description. Agreements on system information: For system information acquisition in multi-hop, the remote UE: o When RRC_CONNECTED, uses end-to-end RRC signaling to obtain its system information directly from its connected cell. o When RRC_IDLE/RRC_INACTIVE, can request SI using PC5-RRC signaling (e.g., RemoteUEInformationSidelink message) o When in RRC_IDLE/RRC_INACTIVE, receives the required SI from PC5-RRC signaling (e.g., UuMessageTransferSidelink) For system information acquisition by the remote UE in multi-hop, the same triggers as Rel17 are supported for sending the PC5-RRC message (e.g., RemoteUEInformationSidelink) namely: o when there is a change in the required SI while in RRC_IDLE/RRC_INACTIVE, or when entering RRC_IDLE/RRC_INACTIVE o when it entering RRC_CONNECTED, a PC5-RRC message (e.g., RemoteUEInformationSidelink) is sent to cancel a previously sent required SI The last relay UE in multihop can forward SI (e.g., in a UuMessageTransferSidelink) to an intermediate Relay upon: • acquisition of the SIB(s) requested (in a hop by hop manner) by a connected child node (intermediate node and/or remote UE, but the last relay UE is not required to determine which node originated the request) • reception of updates of any SIBs requested by a remote UE or another a child relay UE (in a hop-by-hop manner), including SIB1 • deciding to perform unsolicited SIB1 forwarding An intermediate relay UE that is RRC_CONNECTED may use end-to-end RRC signaling to obtain its system information directly from its connected cell (i.e., behaving as a remote UE). The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node). FFS on the need for including a “remote UE ID” in the request. The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list. FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI. Re-use RemoteUEInformationSidelink as the PC5-RRC message transmitted by the remote UE or by the intermediate relay UE to the parent node (intermediate relay or last relay) to request the required SI. The PC5-RRC message transmitted by the last relay UE or by the intermediate relay UE that provides the SI to a child UE contains at least containers with SIB1 and other system information requested by the child UE. Re-use UuMessageTransferSidelink as the PC5-RRC message transmitted by the Last relay or by the intermediate relay UE that provides SI to the child UE. When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink). FFS if it can also receive it directly from SIB broadcast by the last relay UE’s serving cell on Uu (when the intermediate relay UE is in coverage of the same cell that serves the last relay UE). The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay), following legacy remote UE behaviour: • when there is a change in the SI required by the intermediate UE for its own use (as a remote UE) • when the intermediate UE enters RRC_IDLE/RRC_INACTIVE • when the intermediate UE enters RRC_CONNECTED (to cancel a previously sent SI request; if a child node requested the SI, the intermediate UE acquires it by dedicated signalling) The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay): • upon reception of new/changed required SI received from a remote UE/child relay UE (where the concerned SI was not previously requested) • FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay. The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node: a) Upon reception of SI received from intermediate relay or last relay containing SI requested by a child node (intermediate relay or remote UE) b) Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE) c) Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE) d) Upon reception of SIB1 received from a parent relay (i.e., this case may correspond to SIB1 update detected by the last relay, or unsolicited SIB1 forwarding by the last relay) e) Upon receiving updated SIB1 from the network (as in Rel17) f) Upon unsolicited SIB1 forwarding to a connected child node (intermediate relay UE or remote UE) FFS if b), c), and e) are limited to RRC_CONNECTED relay UE or apply in all states. FFS whether to consolidate some of the conditions in stage 3 specification text. Agreements on paging: The remote UE in multi-hop: o When RRC_IDLE/RRC_INACTIVE, can request to receive paging by sending its paging information using PC5-RRC signaling (e.g., RemoteUEInformationSidelink message) o When in RRC_IDLE/RRC_INACTIVE, can receive paging record from PC5-RRC signaling (e.g., UuMessageTransferSidelink)? For a remote UE in multi-hop, the same triggers as Rel17 are supported for sending the PC5-RRC message (e.g., RemoteUEInformationSidelink) namely: o when there is a change in the paging information while in IDLE/INACTIVE, or when entering RRC_IDLE/RRC_INACTIVE o when it entering RRC_CONNECTED, a PC5-RRC message (e.g., RemoteUEInformationSidelink) is sent to release the paging information The last relay UE in multihop can forward paging to an intermediate Relay upon receiving paging message related to a multihop remote UE (including the case of an intermediate relay UE functioning as a remote UE). When the intermediate UE receives a paging message from a parent relay on PC5, it forwards the paging message only to the remote UE being paged or the intermediate relay UE serving a remote UE being paged. The PC5-RRC message containing the paging information that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least paging UE ID and paging cycle of the remote UE and any intermediate relay UEs serving the remote UE. Re-use RemoteUEInformationSidelink as the PC5-RRC message transmitted by the remote UE or by the intermediate relay UE to the parent node (intermediate relay or last relay) to provide the paging information. The PC5-RRC message transmitted by the last relay UE or by the intermediate relay UE contains at least one or multiple paging record(s) associated with intermediate relay UE(s) and/or remote UE(s). Re-use UuMessageTransferSidelink as the PC5-RRC message transmitted by the Last relay or by the intermediate relay UE that provides paging record to the child UE(s). An intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself. FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED. When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain paging from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink). FFS if it can also receive it directly on Uu (when the intermediate relay UE is in coverage). What triggers the intermediate relay UE to request paging monitoring by the parent relay (intermediate relay or last relay) in PC5-RRC (e.g., in RemoteUEInformationSidelink)? • FFS: when there is a change in the paging information of the intermediate UE or child UE • when the intermediate UE enters RRC_IDLE/RRC_INACTIVE • when the intermediate UE enters RRC_CONNECTED (to cancel paging monitoring request) • upon reception of paging monitoring request from a remote UE/child relay UE • FFS: upon change in the ability of the intermediate UE to monitor paging on Uu (e.g., moving in/out of coverage) to initiate/cancel paging monitoring by the parent relay). The intermediate relay UE sends paging message (e.g., in UuMessageTransferSidelink) to a child node • Upon reception of paging message received from a parent node (intermediate relay or last relay) that is intended for a child node (intermediate relay or remote UE) • FFS upon acquisition (from the network) of paging message that is for a child node (intermediate relay or remote UE) Agreements on SRAP: In multi-hop L2 U2N relay, besides the agreed remote UE ID and BEARER ID, in addition, at least include D/C field in the SRAP PDU header. In multi-hop L2 U2N relay, for SRAP header, at least support the legacy single-hop U2N SRAP header, i.e., the agreed UE ID field is used for the remote UE local ID (including the case of an intermediate relay UE acting as a remote UE). FFS on whether to have one additional format to include remote UE L2 ID and what use case it is applied for. Each Layer-2 Intermediate Relay UE has a single PC5 SRAP entity. Agreements on topology: If multiple remote UEs are connected to a single intermediate relay UE, the multiple remote UEs must connect through the same upstream path (same parent relay UE(s)).

For now, the candidate solutions approach 1 and approach 2 are still on table.

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC.

The baseline procedures of approach 1 had been discussed thoroughly and can support the multi-hop relay by reusing the legacy mechanism as much as possible, while the solutions and benefits for approach 2 is still not clear. Therefore, RAN2 triggered a long discussion in last meeting, which discusses the details of approach 2 [2].

[POST129][402][Relay] Control plane approach 2 impact (Apple/Ericsson) Scope: Scope the spec impact of control plane approach 2, considering aspects already discussed (e.g., local ID allocation, QoS split, SRAP and RLC channel configurations), authorization, and service continuity impact. Assume for this discussion that the plenary will approve extension to two additional hops. Intended outcome: Deadline: Long

In this paper, we discuss the FFSs parts from last meetings and provide our opinions on whether to  support approach 2 based on the email discussion.

# 2	Discussion

## 2.1	CP procedures for Approach 1

### Connection establishment

In RAN2 #129 meeting, we agreed to remove all the FFSs in the baseline connection establishment procedure for Approach 1 [3]. However, there are couple of aspects which we could not touch upon in the previous meetings. These are related to the Latency for Connection Establishment Procedure and the Failures during connection establishment ..

#### 2.1.1.1 Latency for Connection Establishment Procedure

The first aspect  is the latency of remote UE connection establishment for IDLE/INACTIVE relay UE cases in approach 1, which is also the motivation of introducing approach 2 [2] by  the proponents.

For IDLE/INACTIVE relay UE, the he latency of remote UE connection establishment increases as the number of hops increases, mainly in the case that all the relay UEs participating in the multi-hop connection establishment are in RRC IDLE/INACTIVE state. Figure 1 shows the procedure when applying the legacy mechanism to trigger the RRC IDLE/INACTIVE relay UE, in which relay UE triggers the RRC connection establishment when receiving the SRB0 message from its adjacent remote UE. We can see that the remote UE needs to wait for the completion  of the connection establishment procedures of all the relay UEs,  which is done in a recursively way,  before it can receive the configuration for the SRB1.

Figure 1. Connection establishment procedure with IDLE/INACTIVE relay UEs by applying Rel-17 mechanism.

An important point to note is that the scenario where the Remote UE triggers connection establishment (Figure 1) while all the relay UEs are RRC IDLE/INACTIVE is unlikely to occur frequently in reality. A more common scenario is that the last and intermediate relay UEs are already in connected state and have established the connection to the network for their own traffic and then they start serving the remote UE.

Observation 1: The scenario where all the relay UEs involved in the multi hop connection establishment are in the RRC IDLE/INACTIVE state when the remote UE triggers the connection establishment is unlikely to occur frequently in reality. A more common scenario  is that the last and intermediate relay UEs (including the first relay UE) are already in connected state and have established the connection to the network for their own traffic before they start serving the remote UEs.

Furthermore, the latency problem in Approach 1 can be addressed with few simple solutions. For instance, the remote UE can prioritize to select  an RRC CONNCETED relay UE if the service requirements of the remote UE are stringent. Alternatively, we can impose a limit on the maximum hop number between the relay UEs and the network when RRC IDLE/INACTIVE relay UEs perform the multi-hop relay operations.

Another approach is to trigger the connection establishment procedures of the relay UEs in parallel, as shown in Figure 2. In step 3, the SRB0 message can be delivered to the gNB via the default PC5 relay RLC channel over the PC5 link between two UEs. This, in turn, can trigger the SRB0 message delivery of all the RRC IDLE/INACTIVE relay UEs in step 4a, 4b, and 4c in parallel. As a result, the connection establishment of the remote UE can be accelerated.

Figure 2. Parallel connection establishment mechanism for IDLE/INACTIVE relay UEs.

In the above procedure, the SRB0 messages form the remote UE can be forwarded before connection establishment of each intermediate relay UE (which corresponding to the FFS if SRB0 would be forwarded before connection establishment of each intermediate relay UE).

Proposal 1: The latency can be improved for Approach 1 using the following solutions.

- Remote UE can prioritize to select an RRC CONNCETED relay UE if the service requirements of remote UE are stringent.

- Restriction/limit on the maximum hop number can be applied between the relay UEs and the network when the RRC IDLE/INACTIVE relay UEs perform the multi-hop relay

- SRB0 messages of remote UE can be forwarded before connection establishment of each intermediate relay UE via the default PC5 relay RLC channels between UEs, which can trigger the parallel connection establishment of the RRC IDLE/INACTIVE relay UE

#### 2.1.1.3 Failures during connection establishment

Another issue is the timer for the connection establishment with multi-hop since the legacy timer is only for the single-hop. If we reuse the legacy timer value, Remote UE may fail to set up its RRC connection, due to its parent relay UE(s) taking too long time to prepare the RRC configuration for the remote UE’s initial access. We believe such  failures can be resolved with extended timers for multi-hop. T300 timer could be increased for multi-hop U2N relay operation in accordance with the hop count.

Proposal 2: Extend the T300 timer for multi-hop U2N relay operation in accordance with the hop count for connection establishment procedure.

### Protocol stack

In RAN2 #129, it was agreed each Layer-2 Intermediate Relay UE has a single PC5 SRAP entity. The Control Plane and the User Plane protocol stacks of approach 1 for multi-hop relay with one intermediate Relay UE can be shown in Figure 3. These protocol stack can be taken as the basis for the R19 multi-hop relay including two or more hops. If there are more than two hops, i.e., additional intermediate relay UE between the last relay and first relay UE, the intermediate relay UE will have hop- by-hop PC5 SRAP/RLC/MAC/PHY layers with its adjacent relay UEs

(a) Control plan

(b) User plan

Figure 3. Protocol stack for multi-hop L2 U2N relay

Proposal 3: Adopt the Control Plane and User Plane protocol stack for Multi-hop L2 U2N relay as shown in Figure 3. SRAP layer is reused for bearer mapping and remote UE identification purpose.

### 2.1.3 QoS split

Based on approach 1, the gNB determines the configurations of all the remote UEs and relay UEs,  the same as legacy in rel-17, the QoS can be guaranteed by the gNB by implementation.

Proposal 4. RAN2 confirm that the QoS split is determined by gNB and guaranteed by gNB by implementation.

### 2.1.4 Local ID allocation

For backward compatibility, in approach 1, the legacy local ID allocation mechanism in Rel-17 can be reused, to be more specific, the procedure for multi-hop relay can be:

- Step 0: the remote UE reports its L2 ID to the gNB

- Step 1: the parent relay UE reports the L2 ID of the remote UE to request the local ID allocation.

- Step 2: gNB allocates the local ID and configures the local ID for the remote UE identified by the L2 ID. The local ID will be configured to the remote UE and all the relay UE(s) along the multi-hop relaying path for the remote UE.

Same as legacy, the local ID for remote UE in multi-hop relay can be 8 bits.

Proposal 5: Reusing the single-hop relay mechanism to support the Local ID allocation for multi-hop relay:

- Parent relay UE of the remote UE reports the L2 ID to the gNB to request the local ID allocation, the uniqueness of the local ID can be guaranteed by the gNB by implementation.

- The remote UE local ID is 8 bits.

We also need to consider the uniqueness of the remote UE local ID. In rel-17, the remote UE local ID can be used to uniquely identify a remote UE within the serving area of connected Relay UE. In other words, the network can identify a remote UE in the scope of a U2N relay UE. In multi-hop relay, if the local ID is unique within the serving area of connected relay UE, i.e., the first relay UE, local ID confliction can happen in the parent relay UE or at the network side. Figure 4 provides an example of Local ID conflict happening on the last relay UE and network side for better understanding.

Figure 4. Local ID confliction on the last relay UE and network side.

Proposal 6: RAN2 need to discussion the uniqueness of the remote UE local ID which can impact the data forwarding behaviour of the relay UEs.

### SRAP PDU format and SRAP configuration

In RAN2 #129  meeting, we agreed that the remote UE ID and bearer ID can be included in the SRAP PDU header for data forwarding. For a relay UE, it will receive the SRB/DRB data from the ingress relay RLC channel of the ingress link, and forward the data to the egress relay RLC channel of the egress link.

In the UL, when the relay UE receives the data from its child UE (a relay UE or a remote UE), it will forward the data to its parent relay UE according to the SRAP PDU header and the bearer mapping configuration. The bearer mapping configuration need to contain the mapping between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel. Since SRB and DRB can share same index, ingress PC5 relay RLC channel between the relay UE and its child UE can be used to identify the SRB index and the DRB index.

In the DL, the relay UE can also forward the data based on the mapping between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel of the egress link, and ingress PC5 relay RLC channel between the relay UE and its parent UE can be used to identify the SRB index and the DRB index.

From the discussion in the above, we can have the following proposals for the SRAP PDU format and SRAP configuration design for the multi-hop relay.

Proposal 7: The SRAP configuration contains the mapping configuration between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel.

- For the first relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the egress PC5 relay RLC channel between the first relay UE and remote UE, the egress PC5 relay RLC channel between the first relay UE and its parent relay UE.

- For intermediate relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the identity (L2 ID) of its child relay UE, the egress PC5 relay RLC channel between the intermediate relay UE and its child relay UE, the egress PC5 relay RLC channel between the intermediate relay UE and its parent relay UE.

- For the last relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the identity (L2 ID) of its child relay UE, the egress Uu relay RLC channel on Uu, the egress PC5 relay RLC channel between the last relay UE and its child relay UE.

Besides, since the relay UE can connect to several child UEs if it is not the first relay UE, the path information corresponding to the egress link is needed. For handling the path information, there are two options that can be considered.

Option 1:  Derivation of implicit path information by the relay UEs - The last or the intermediate relay UE may implicitly derive the path information by mapping local ID of the remote UE (carried in the SRAP header) to the L2 ID of the child relay UE during the uplink transmission, e.g., uplink transmission of RRCSetupRequest message. This mapping can then be used to route the downlink packets for the remote UE via the child relay UE.

Option 2:  Explicit signaling of the path information to the relay UEs - The other option is to explicitly provide the path information through the identity (L2 ID) of the child relay UE in the SRAP configuration for the intermediate and last relay UE.

Proposal 8: For handling the Multi hop relay path information there are two options that can be considered

Option 1 - The path information can be implicitly delivered by last or the intermediate relay UE by mapping local ID of the remote UE (carried in the SRAP header) to the L2 ID of the child relay UE during the uplink transmission of RRCSetupRequest message

Option 2 - The path information can be explicitly provided through the identity (L2 ID) of the child relay UE in the SRAP configuration for the intermediate and last relay UE.

### 2.1.6 System information

For system information, we have the following FFSs. For each FFS, we provide some discussion which lead to some conclusions.

Table 1. Discussion for the FFSs on System information

| FFS | Discussion |
| --- | --- |
| The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node). FFS on the need for including a “remote UE ID” in the request. | We believe that the “remote UE ID” is not need in the request from an intermediate relay UE to its parent. The parent UE will forward the SI requested by the intermediate relay UE, regardless of whether SI is requested by the intermediate relay UE or by other child UE connected to this intermediate relay UE. After receiving the SI, the intermediate relay UE can forward the SI upon the request from its child UEs. In the this procedure, the remote UE ID is not needed.Conclusion: The “remote UE ID” is not needed in the request. |
| The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list. FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI. | It is straightforward that the intermediate relay UE can respond directly if the intermediate relay UE also request this SI from the parent and already received the SI form the parent. By the current method, the intermediate relay UE can always get the updated SI from the parent relay UE. Conclusion: If the intermediate relay UE can respond directly if it requested the same SI requested by its child UE. |
| When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink). FFS if it can also receive it directly from SIB broadcast by the last relay UE’s serving cell on Uu (when the intermediate relay UE is in coverage of the same cell that serves the last relay UE). | In Rel-17, the remote UE is able to receive the SI on the Uu based on UE implementation if the Uu is available. Similarly, the intermediate relay UE can act like a remote UE, i.e., receiving the SI form the parent UE as well as the Uu broadcasted SI if the Uu is available. Conclusion: The intermediate relay UE can also receive the requested SI directly from SIB broadcast by the last relay UE’s serving cell on Uu. |
| The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay): • FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay. | For the intermediate relay UE, the Uu is not always available and not stable. Therefore, same as in Rel-17, the SI forwarding based on the PC5-RRC should be the baseline, and the intermediate UE can receive SIB broadcast on Uu by UE implementation. Whether the SI can be received on Uu should not impact the SI forward on PC5. Conclusion: SI forwarding based on the PC5-RRC should always be supported as a baseline, and receiving SIB broadcast on Uu can be a supplemental method, but should not impact the SI forwarding on PC5. |
| The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node: a) Upon reception of SI received from intermediate relay or last relay containing SI requested by a child node (intermediate relay or remote UE) b) Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE) c) Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE) d) Upon reception of SIB1 received from a parent relay (i.e., this case may correspond to SIB1 update detected by the last relay, or unsolicited SIB1 forwarding by the last relay) e) Upon receiving updated SIB1 from the network (as in Rel17) f) Upon unsolicited SIB1 forwarding to a connected child node (intermediate relay UE or remote UE) FFS if b), c), and e) are limited to RRC_CONNECTED relay UE or apply in all states. FFS whether to consolidate some of the conditions in stage 3 specification text. | We did not see the motivation to limit the cases b), c), and e) to the RRC_CONNECTED relay UE. Cases b), c), and e) are the condition for the relay UE to forward the SI, no matter what the RRC state the relay UE is. Conclusion: The intermediate relay UE should send the SI to the child UE no matter what the RRC state it is in. - Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE) - Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE) - Upon receiving updated SIB1 from the network (as in Rel17) |

Proposal 9: RAN2 can confirm the following aspects for system information forwarding in multi-hop relay.

- The “remote UE ID” is not needed in the request from the child UE to its parent.

- If the intermediate relay UE can respond directly if it requested the same SI requested by its child UE.

- The intermediate relay UE can receive the requested SI directly from SIB broadcast by the last relay UE’s serving cell on Uu.

- SI forwarding based on the PC5-RRC should always be supported as a baseline, and receiving SIB broadcast on Uu can be a supplemental method, but should not impact the SI forwarding on PC5.

- The intermediate relay UE should send the SI to the child UE no matter what the RRC state it is in.

  - Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE)

  - Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE)

  - Upon receiving updated SIB1 from the network (as in Rel17)

### 2.1.7 Paging forwarding

For paging forwarding, we have the following FFSs . For each FFS, we provide some discussion which lead to some conclusions..

Table 2. Discussion for the FFSs on paging forwarding

| An intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself. FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED. | Based on the paging mechanism, if the intermediate relay UE comes into RRC_CONNECTED, it will report the UE ID of the connected child UE to the network, then the network will send the paging information for the child UEs via dedicate signaling. Conclusion: the intermediate relay UE releases the paging-related information in the parent UE for the child UEs if it moves into RRC_CONNECTED state. |
| --- | --- |
| When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain paging from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink). FFS if it can also receive it directly on Uu (when the intermediate relay UE is in coverage). | In rel-17, the remote UE is allowed to monitor paging on the Uu based on UE implementation. In multi-hop relay, we can follow this principle. Conclusion: The intermediate relay UE is allowed to receive the paging on Uu directly. |
| What triggers the intermediate relay UE to request paging monitoring by the parent relay (intermediate relay or last relay) in PC5-RRC (e.g., in RemoteUEInformationSidelink)? • FFS: when there is a change in the paging information of the intermediate UE or child UE • when the intermediate UE enters RRC_IDLE/RRC_INACTIVE • when the intermediate UE enters RRC_CONNECTED (to cancel paging monitoring request) • upon reception of paging monitoring request from a remote UE/child relay UE • FFS: upon change in the ability of the intermediate UE to monitor paging on Uu (e.g., moving in/out of coverage) to initiate/cancel paging monitoring by the parent relay). The intermediate relay UE sends paging message (e.g., in UuMessageTransferSidelink) to a child node • Upon reception of paging message received from a parent node (intermediate relay or last relay) that is intended for a child node (intermediate relay or remote UE) • FFS upon acquisition (from the network) of paging message that is for a child node (intermediate relay or remote UE) | When there is a change in the paging-related information of the intermediate relay UE or child UE, the intermediate relay UE need to update the paging-related information in the request. Conclusion 1: the intermediate relay UE need to update the paging-related information in the request to its parent if the paging information of the intermediate UE or child UE changes. It is allowed for the intermediate relay UE to monitoring the paging directly on Uu, but this mechanism should not impact the paging forwarding on PC5 because the Uu is not stable for the intermediate relay UE (which is not connected directly to the network). Conclusion 2: the intermediate relay UE can be allowed to monitoring the paging directly on Uu, but should not impact the paging forwarding on PC5 interface. On acquiring the paging message that is for a child node/UE, the intermediate relay UE can either forward it to its child UE or wait for the message to be received by the indirect link before forwarding based on its implementation. Conclusion 3: If the intermediate relay UE can acquire the paging on its own it can forward the paging message to its child UE based on its implementation. |

Proposal 10: RAN2 can confirm the following aspects for paging request and forwarding in multi-hop relay.

- The intermediate relay UE releases the paging-related information in the parent UE for the child UEs if it comes into RRC_CONNECTED state.

- The intermediate relay UE is allowed to receive the paging on Uu directly.

- The intermediate relay UE need to update the paging-related information in the request to its parent if the paging information of the intermediate UE or child UE changes.

- The intermediate relay UE is allowed to monitoring the paging directly on Uu, but should not impact the paging forwarding on PC5 interface.

- If the intermediate relay UE can acquire the paging on its own it can forward the paging message to its child UE based on its implementation.

### 2.1.8 Radio link failure handling

In approach 1, we can follow the Rel-17 principle for radio link management, i.e., the U2N Remote UE suspends the Uu radio link management, since the remote UE and the intermediate relay UE connects to network via indirect link and there is no Uu link anymore. The PC5/Uu RLF may happen for the multi-hop scenario in the following cases as depicted in Figure 5.

(a) PC5 RLF of the remote UE

(b) PC5 RLF of the first relay UE

(c) PC5 RLF of the intermediate relay UE

(d) Uu RLF of the last relay UE

Figure 5.	RLF declaration cases in multi-hop L2 U2N relay

In case (a) of Figure 5, when the remote UE detects the PC5 RLF over the PC5 link between the remote UE and the first Relay UE, it should trigger the RRC connection re-establishment or relay reselection, which is similar to the Rel-17 handling.

Proposal 11: Similar to Rel-17 SL relay, when the remote UE in multi-hop relay detects the PC5 RLF, it can trigger the RRC connection re-establishment.

In case (b) of Figure 5, when the first relay UE detects the PC5 RLF over the PC5 link between the first relay UE and the intermediate relay UE, it can send a notification message with cause value “PC5 RLF” to the remote UE (assume the link between the remote UE and the first relay UE is good) and trigger the RRC connection re-establishment or relay reselection. Upon receiving the notification message, the remote UE can determine whether to release the PC5 link with the first relay UE and triggers the RRC connection re-establishment or relay reselection. If the remote UE releases the PC5 link, the remote UE can perform the relay reselection and the cell selection, and further trigger the RRC connection re-establishment via the new relay or via the Uu link (when in RRC CONNECTED state).

Proposal 12: When the first relay UE detects the PC5 RLF, it can send a notification message with cause value “PC5 RLF” to the remote UE and trigger the RRC connection re-establishment.

Proposal 13: Upon receiving the notification message with cause value “PC5 RLF”, the remote UE can determine whether to release the PC5 link with the first relay UE and triggers the RRC connection re-establishment.

In case (c) of Figure 5, when the intermediate relay UE detects the PC5 RLF, it can send a notification message with cause value “PC5 RLF” to the first relay UE and trigger the RRC connection re-establishment or relay reselection. Upon receiving the notification message with cause value “PC5 RLF”, the first relay UE triggers RRC connection re-establishment as legacy in Rel-17. Furthermore, the first relay UE needs to notify the U2N Remote UE, but what cause value should be contained in the notification message should be discussed. The following options can be considered.

- Option 1: introduce a new cause value which indicates PC5 RLF of the intermediate relay UE, with the identity of the intermediate relay UE

- Option 2: introduce a new cause value which indicates RLF has happened on the indirect link, without the identity of the intermediate relay UE.

The remote UE behaviour when receiving the notification message from the Intermediate Relay UE can be similar as before.

In case (d) of Figure 5, the situation is similar to case (c), the only difference is the RLF happens on the Uu link of the last relay UE. In this case, the intermediate relay UE would receive the RLF indication from its parent relay UE, and need to forward the RLF indication to its child UE, it  also triggers the RRC connection re-establishment or relay reselection. In this case, the cause value in the notification message can be “Uu RLF”.

Proposal 14: Upon receiving the notification message with cause value “Uu/PC5 RLF” from the parent relay UE, the intermediate relay UE needs to send a notification message to its child UE. The following options can be considered as the cause value.

- Option 1: introduce a new cause value which indicates Uu/PC5 RLF of the relay UE, with the identity of the relay UE

- Option 2: introduce a new cause value which indicates RLF has happened on the indirect link, without the identity of the relay UE that detects the RLF.

### 2.1.9 Uu RSRP Threshold for the Intermediate Relay UE

In RAN 2 # 128 meeting we have agreed that the network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE

Agreements:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

However, in Approach 1, the Intermediate Relay UE connects to the network via the Last Relay UE before it can function as an Intermediate Relay UE. Therefore, we believe that reusing the Uu RSRP upper bound of the remote UE for the intermediate relay UE is more appropriate than introducing a new threshold specifically for the Intermediate Relay UE.

Proposal 15: Reuse the Uu RSRP upper bound of the remote UE for the intermediate relay UE, instead of defining a new threshold specifically for the intermediate relay UE

## CP procedures for Approach 2

RAN2 triggers an email discussion [POST129][402][Relay] (Control plane approach 2 impact (Apple/Ericsson)) to discuss the solutions for approach 2. In this section, we will discuss approach 2 based on this email discussion and make a brief summary. Same as in the email discussion, we only focus on the IDLE/INACTIVE relay UEs.

Table 3. Control plan aspects for the support of Approach 2

| CP aspects | Solution | Problem |
| --- | --- | --- |
| Connection establishment/ local ID allocation | - Forwarding Remote UE’s first RRC message on the SL-RLC0. - Contain the L2 ID of the remote UE in the SRAP PDU carring the first RRC message, so that the last relay UE can decode the L2 ID information of the remote UE and report it in the SUI for local ID allocation. | The local ID allocation is totally different with Rel-17 and RAN2 need to define some new SRAP PDU format and handling procedure (for the last relay UE). |
| QoS split | - The gNB can determine the QoS split based on the hop number and can forward the split QoS information hop by hop | The split QoS may not always be guaranteed since the gNB does not know the resource status of the IDLE/INACTIVE relay UEs |
| Relaying configuration on the relay UEs | - Reusing the U2U relay mechanism to determine the relaying configuration of the IDLE/INACTIVE relay UEs | Taking the U2U mechanism as baseline, the relay UE needs the flow-to-SLRB mapping information of the remote UE before determining the QoS mapping of itself. The flow-to-SLRB mapping information is delivered hop by hop. The procedure can be very complex and last long with the increase of hop number. |
| Service continuity | - Not support since case C and case D only considers the RRC_CONNECTED relay UEs | Not supported |

Observation 2: based on the email discussion, we see the following spec impact and problem to support approach 2.

- The local ID allocation is totally different with Rel-17 and RAN2 need to define some new SRAP PDU format and handling procedure (for the last relay UE).

- The split QoS may not always be guaranteed since the gNB does not know the resource status of the IDLE/INACTIVE relay UEs

- Take the U2U mechanism as baseline, the relay UE needs the flow-to-SLRB mapping information of the remote UE before determining the QoS mapping of itself. The flow-to-SLRB mapping information should be delivered hop by hop. The procedure can be very complex and last long with the increase of hop number.

- Service continuity scenarios case C and case D which only considers the RRC_CONNECTED relay UEs cannot be supported with Approach 2

## High Level Summary of Approach 1 and 2

During the [POST129][402][Relay] Control plane approach 2 impact email discussion, it was mentioned that RRC_CONNECTED intermediate relay UE(s) in approach 2 will have the same behaviour as Approach 1. In that sense we view  Approach 2 as a supplement of Approach 1, primarily designed to address the latency problem in case of RRC_IDLE/INACTVIE relay UEs in Approach 1. However, we want to mention that approach 2 is not the only way to solve the latency problem, as proposed for  proposal 1 in Sec 2.1.1 of this paper.

Observation 3: Approach 2 could be  a supplement of approach 1 to solve the latency problem in case of RRC_IDLE/INACTVIE relay UEs, but is not the only method to solve the latency problem.

Besides, we can see from Table 3 that the support of approach 2 for RRC_IDLE/INACTIVE relay UE would have  a lot of spec impacts, e.g., on the local ID allocation mechanism, SRAP PDU format, SRAP handling and so on. Besides, we doubt how many benefits approach 2 can bring since the relay UEs are all in IDLE/INATCIVE state is not a typical case.. Therefore, we do not believe it is advisable to support Approach 2, particularly considering the limited TUs remaining for the multi-hop relay topic in Rel-19.

Proposal 16. Considering the limited time left, RAN2 should focus on the support of approach 1 instead of spending time discussing  feasibility of approach 2. The first priority  is to make the multi-hop relay works in Rel-19, then RAN2 can consider the enhancements.

# 3	Conclusion

## CP procedures for Approach 1

Connection establishment

Latency for Connection Establishment Procedure

Observation 1: The scenario where all the relay UEs involved in the multi hop connection establishment are in the RRC IDLE/INACTIVE state when the remote UE triggers the connection establishment is unlikely to occur frequently in reality. A more common scenario  is that the last and intermediate relay UEs (including the first relay UE) are already in connected state and have established the connection to the network for their own traffic before they start serving the remote UEs.

Proposal 1: The latency can be improved for Approach 1 using the following solutions.

- Remote UE can prioritize to select an RRC CONNCETED relay UE if the service requirements of remote UE are stringent.

- Restriction/limit on the maximum hop number can be applied between the relay UEs and the network when the RRC IDLE/INACTIVE relay UEs perform the multi-hop relay

- SRB0 messages of remote UE can be forwarded before connection establishment of each intermediate relay UE via the default PC5 relay RLC channels between UEs, which can trigger the parallel connection establishment of the RRC IDLE/INACTIVE relay UE

Failures during connection establishment

Proposal 2: Extend the T300 timer for multi-hop U2N relay operation in accordance with the hop count for to avoid any failures during connection establishment procedure.

Protocol stack

Proposal 3: Adopt the Control Plane and User Plane protocol stack for Multi-hop L2 U2N relay as shown in Figure 3. SRAP layer is reused for bearer mapping and remote UE identification purpose.

QoS split

Proposal 4. RAN2 confirm that the QoS split is determined by gNB and guaranteed by gNB by implementation.

Local ID allocation

Proposal 5: Reusing the single-hop relay mechanism to support the Local ID allocation for multi-hop relay:

- Parent relay UE of the remote UE reports the L2 ID to the gNB to request the local ID allocation, the uniqueness of the local ID can be guaranteed by the gNB by implementation.

- The remote UE local ID is 8 bits.

Proposal 6: RAN2 need to discussion the uniqueness of the remote UE local ID which can impact the data forwarding behaviour of the relay UEs.

SRAP PDU format and SRAP configuration

Proposal 7: The SRAP configuration contains the mapping configuration between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel.

- For the first relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the egress PC5 relay RLC channel between the first relay UE and remote UE, the egress PC5 relay RLC channel between the first relay UE and its parent relay UE.

- For intermediate relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the identity (L2 ID) of its child relay UE, the egress PC5 relay RLC channel between the intermediate relay UE and its child relay UE, the egress PC5 relay RLC channel between the intermediate relay UE and its parent relay UE.

- For the last relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the identity (L2 ID) of its child relay UE, the egress Uu relay RLC channel on Uu, the egress PC5 relay RLC channel between the last relay UE and its child relay UE

Proposal 8: For handling the Multi hop relay path information there are two options that can be considered

Option 1 - The path information can be implicitly delivered by last or the intermediate relay UE by mapping local ID of the remote UE (carried in the SRAP header) to the L2 ID of the child relay UE during the uplink transmission of RRCSetupRequest message

Option 2 – The path information can be explicitly provided through the identity (L2 ID) of the child relay UE in the SRAP configuration for the intermediate and last relay UE.

System information

Proposal 9: RAN2 can confirm the following aspects for system information forwarding in multi-hop relay.

- The “remote UE ID” is not needed in the request from the child UE to its parent.

- If the intermediate relay UE can respond directly if it requested the same SI requested by its child UE.

- The intermediate relay UE can receive the requested SI directly from SIB broadcast by the last relay UE’s serving cell on Uu.

- SI forwarding based on the PC5-RRC should always be supported as a baseline, and receiving SIB broadcast on Uu can be a supplemental method, but should not impact the SI forwarding on PC5.

- The intermediate relay UE should send the SI to the child UE no matter what the RRC state it is in.

  - Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE)

  - Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE)

  - Upon receiving updated SIB1 from the network (as in Rel17)

Paging forwarding

Proposal 10: RAN2 can confirm the following aspects for paging request and forwarding in multi-hop relay.

- The intermediate relay UE releases the paging-related information in the parent UE for the child UEs if it comes into RRC_CONNECTED state.

- The intermediate relay UE is allowed to receive the paging on Uu directly.

- The intermediate relay UE need to update the paging-related information in the request to its parent if the paging information of the intermediate UE or child UE changes.

- The intermediate relay UE is allowed to monitoring the paging directly on Uu, but should not impact the paging forwarding on PC5 interface.

- If the intermediate relay UE can acquire the paging on its own it can forward the paging message to its child UE based on its implementation.

Radio Link failure handling

Proposal 11: Similar to Rel-17 SL relay, when the remote UE in multi-hop relay detects the PC5 RLF, it can trigger the RRC connection re-establishment.

Proposal 12: When the first relay UE detects the PC5 RLF, it can send a notification message with cause value “PC5 RLF” to the remote UE and trigger the RRC connection re-establishment.

Proposal 13: Upon receiving the notification message with cause value “PC5 RLF”, the remote UE can determine whether to release the PC5 link with the first relay UE and triggers the RRC connection re-establishment.

Proposal 14: Upon receiving the notification message with cause value “Uu/PC5 RLF” from the parent relay UE, the intermediate relay UE needs to send a notification message to its child UE. The following options can be considered as the cause value.

- Option 1: introduce a new cause value which indicates Uu/PC5 RLF of the relay UE, with the identity of the relay UE

- Option 2: introduce a new cause value which indicates RLF has happened on the indirect link, without the identity of the relay UE that detects the RLF.

Uu RSRP Threshold for the Intermediate Relay UE

Proposal 15: Reuse the Uu RSRP upper bound of the remote UE for the intermediate relay UE, instead of defining a new threshold specifically for the intermediate relay UE

## CP procedures for Approach 2

Observation 2: based on the email discussion, we see the following spec impact and problem to support approach 2.

- The local ID allocation is totally different with Rel-17 and RAN2 need to define some new SRAP PDU format and handling procedure (for the last relay UE).

- The split QoS may not always be guaranteed since the gNB does not know the resource status of the IDLE/INACTIVE relay UEs

- Take the U2U mechanism as baseline, the relay UE needs the flow-to-SLRB mapping information of the remote UE before determining the QoS mapping of itself. The flow-to-SLRB mapping information should be delivered hop by hop. The procedure can be very complex and last long with the increase of hop number.

- Service continuity scenarios case C and case D which only considers the RRC_CONNECTED relay UEs cannot be supported with Approach 2

## High Level Summary of Approach 1 and 2

Observation 3: Approach 2 could be a supplement of approach 1 to solve the latency problem in case of RRC_IDLE/INACTVIE relay UEs, but is not the only method to solve the latency problem.

Proposal 16. Considering the limited time left, RAN2 should focus on the support of approach 1 instead of spending time discussing feasibility of approach 2. The first priority is to make the multi-hop relay works in Rel-19, then RAN2 can consider the enhancements.

# 4	Reference

- Session Chair Notes on Relay and Positioning RAN2 #129;

- [POST129][402][Relay] Control plane approach 2 impact (Apple/Ericsson).

- R2-2500496, Report of [Post128][401][Relay] Control Plane Baseline Solution	InterDigital	discussion	Rel-19	NR_SL_relay_multihop;

- R2-2502191 Report of [POST129][402][Relay] Control plane approach 2 impact (Apple/Ericsson)