3GPP TSG-RAN WG2 Meeting #129bis	R2-2502612

Wuhan, China, 7 - 11 April, 2025

Source: 	Huawei, HiSilicon, Turkcell

Title: 	Discussion on Uplink Capacity Enhancements

Agenda Item:	8.8.3

Document for:	Discussion and decision

# 1	Introduction

The NR NTN WID includes the uplink capacity enhancement as follows [1].

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

RAN1 discussed this objective for several meetings. In RAN1#119, RAN1 agreed to support inter-slot OCC for length 2 and length 4 [2].

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. - RAN1 assumes that RAN4 does not define new UE requirements for CFO.

In this paper, we provide our views on the possible RAN2 impacts.

# 2	Discussion

In NTN scenario, a satellite usually needs to cover a much larger terrestrial area serving a great number of UEs compared with gNB in terrestrial network. In order to compensate large path-loss suffered by UE in uplink, the maximum number of repetitions for a PUSCH transmission is increased from 16 to 32 in Rel-17. Whereas, the use of repetition increases the transmission time of a UE and is accompanied with high resource utilization in uplink, which will compromise the system capacity. Considering the large number of UEs demanding for uplink transmission and the limited spectrum resources available, it is beneficial to enhance uplink system capacity/throughput.

Orthogonal cover codes (OCC) is proposed to address the issue above. In UL transmission, signals from different UEs can be multiplexed in the same time and frequency domain resource by multiplexing different OCCs on the repeated signals. The orthogonality of OCC sequence can be maintained across multiple UEs, if the power consistence and phase continuity of the repeated signals and channel coefficient are not changed dramatically during the time span of an OCC sequence.

From signaling perspective, the following need to be configured to the UE:

1) OCC length (length 2, length 4)

2) OCC index (the line index of the OCC matrix)

A specific OCC sequence can be indicated to a UE by indicating the OCC length and the OCC index: based on the OCC length, the specific OCC matrix can be generated, and the exact OCC sequence allocated to each UE is represented by the OCC index, e.g., each line in the matrix. 2 or 4 UEs perform inter-slot OCC based on the OCC sequences allocated to the UEs. It is proposed:

Proposal 1: OCC index (i.e., the line index of the OCC matrix) and OCC length need to be indicated to the UE.

The next issue is how to provide/signal the OCC configuration to the UE, considering the UE’s PUSCH transmission can be scheduled by dynamic grant or configured grant:

- For dynamic grant, where the PUSCH transmission is scheduled by DCI, the OCC signaling can be included in DCI and is to be determined by RAN1.

- For configured grant Type 1, where the PUSCH transmission is performed after reception of configuredGrantConfig indicated by RRC signaling, the OCC-related parameters can be included in the rrc-ConfiguredUplinkGrant within configuredGrantConfig.

- For configured grant Type 2, where the PUSCH transmission is activated by a DCI after the reception of RRC signaling including configuredGrantConfig without rrc-ConfiguredUplinkGrant, the OCC-related parameters can be either included in the DCI, or configured by configuredGrantConfig/pusch-config. The down-selection needs to be further discussed by RAN1.

As analyzed above, it is proposed:

Proposal 2: For configured grant Type 1, OCC configurations are provided to the UE via RRC signalling. FFS what signalling for the OCC configuration should be provided for dynamic grant or configured grant type 2.

As an optional enhancement for Rel-19 NTN UEs, UE capabilities need to be introduced for indicating the OCC support. As noted in RAN1 agreement, separate UE capabilities are introduced for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Proposal 3: RAN2 to introduce separate UE capabilities for OCC length 2 and OCC length 4, and UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

# 3	Conclusion

This contribution analyses the OCC signalling based on RAN1 progress, and proposes the following:

Proposal 1: OCC index (i.e., the line index of the OCC matrix) and OCC length need to be indicated to the UE.

Proposal 2: For configured grant Type 1, OCC configurations are provided to the UE via RRC signalling. FFS what signalling for the OCC configuration should be provided for dynamic grant or configured grant type 2.

Proposal 3: RAN2 to introduce separate UE capabilities for OCC length 2 and OCC length 4, and UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

# 4	References

- RP-241789, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- Session notes for 9.11 Non-Terrestrial Networks (NTN) for NR Phase 3, Internet of Things (IoT) Phase 3, and IoT-NTN TDD mode, Ad-Hoc Chair (Huawei), RAN1#119