3GPP TSG-RAN WG2 Meeting #129bis	R2-2502616

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.18

Source:	Nokia

Title:	Discussion on simultaneous RAN and CN paging

WID/SID:	TEI19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss a scenario in which the network addresses the same UE twice in one Paging message by including two PagingRecord IEs where one of them contains ng-5G-S-TMSI and the other fullI-RNTI where both identifiers belong to the same UE.

According to the current specification multiple PagingRecord IEs within one Paging message for one UE are not allowed. This also covers the case above case where two different types of UE identity are used (i.e. 5G-S-TMSI and fullI-RNTI).

3GPP TS 38.331 V18.5.1:

#### 5.3.2.2	Initiation

The network initiates the paging procedure by transmitting the Paging message at the UE's paging occasion as specified in TS 38.304 [20]. The network may address multiple UEs within a Paging message by including one PagingRecord for each UE. The network may also include one or multiple TMGI(s) in the Paging message to page UEs for specific MBS multicast session(s).

The above 5G requirement is a copy of the 4G requirement stated in 3GPP TS 36.331 V18.5.0 Clause 5.3.2.2 which dates to Rel-15, when the RRC INACTIVE state and fullI-RNTI RAN-initiated paging did not exist.

# 2	Discussion

## 2.1	Causes of double paging occurrence

Scenario 1 (interaction with NGAP UE Context Release procedure):

In this scenario the double paging (5G-S-TMSI & fullI-RNTI) results from the interaction between paging and NGAP UE Context Release signaling procedures (see the diagram below):

- RAN-initiated paging (fullI-RNTI ) generated before NGAP UE Context Release procedure and CN-initiated paging (5G-S-TMSI) generated after NGAP UE Context Release procedure coincide in the same Paging Occasion (PO) / Paging message,

- since in this case the transmission of RRCRelease to transition the UE from RRC_INACTIVE to RRC_IDLE is not required, the local UE state is RRC_INACTIVE, and both 5G-S-TMSI and fullI-RNTI are valid from the UE’s perspective.

Figure 1. Double paging – interaction with NGAP UE Context Release procedure

By knowing the implementation of gNB-DU and gNB-CU, one can use the appropriate configuration of both nodes and/or introduce additional delays for certain procedures to minimize the probability of dual paging (e.g. postpone NGAP UE Context Release Request/Complete until the outcome of the Paging procedure is known). However, unless double paging is a real problem, avoiding it unnecessarily delays NGAP UE Context Release procedure and/or limits the use of gNB-DU paging strategies for RAN-initiated paging (e.g. paging repetitions and/or paging postponing to the next PO in case of PO congestion).

Scenario 2 (NGAP signaling connection lost):

In this scenario the double paging (5G-S-TMSI & fullI-RNTI) results from the interaction between paging and AN Release procedure caused by AMF failure resulting in NGAP signaling connection lost (see 3GPP TS 23.502 V19.2.0 Clause 4.2.6):

- RAN-initiated paging (fullI-RNTI ) generated before UE’s state transition from CM-CONNECTED to CM-IDLE in AMF and CN initiated paging (5G-S-TMSI) generated after UE’s state transition from CM-CONNECTED to CM-IDLE in AMF coincide in the same Paging Occasion (PO) / Paging message,

- the UE’s state transition in the AMF is triggered by the detection of the NGAP signaling connection lost,

- since in this case the UE is unaware of the CM-CONNECTED to CM-IDLE transition both 5G-S-TMSI and fullI-RNTI are valid from the UE’s perspective.

Figure 2. Double paging – interaction with NGAP signaling connection lost

This scenario cannot be prevented by gNB since the CM-CONNECTED to CM-IDLE transition is a result of the AMF failure which is not controlled by the network signaling.

Observation 1: There are scenarios that result in double paging (5G-S-TMSI & fullI-RNTI) caused by interaction between paging and UE context release procedures.

Observation 2: Avoidance of scenarios causing double paging may unnecessarily delay NGAP UE Context Release procedure and/or limit the use of gNB-DU paging strategies for RAN-initiated paging.

## 2.2	UE handling of double paging

According to 3GPP TS 38.331 V18.5.1 Clause 5.3.2.3 (see the excerpt below), the way the UE responses to paging depends on type of UE’s identity included in the Paging message:

- If the paging includes fullI-RNTI (i.e. it is RAN-initiated paging) the UE initiates RRC resume procedure,

- If the paging includes UE identity allocated by upper layers (i.e. 5G-S-TMSI for CN-initiated paging) the UE forwards the UE identity to the upper layers and transitions to RRC_IDLE state, which according to TS 24.501 Clause 5.6.2.2 results in NAS Service Request procedure (i.e. new RRC and NAS connection establishment).

3GPP TS 38.331 V18.5.1:

#### 5.3.2.3	Reception of the Paging message by the UE or PagingRecord by the L2 U2N Remote UE

Upon receiving the Paging message by the UE or receiving PagingRecord from its connected L2 U2N Relay UE by a L2 U2N Remote UE, the UE shall:

[…]

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message, or

1>	if in RRC_INACTIVE, for the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE:

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

3>	if the UE is configured by upper layers with Access Identity 1:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mps-PriorityAccess;

3>	else if the UE is configured by upper layers with Access Identity 2:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mcs-PriorityAccess;

3>	else if the UE is configured by upper layers with one or more Access Identities equal to 11-15:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to highPriorityAccess;

[…]

[…]

4>	else:

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

3>	else:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

[…]

2>	else if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	if upper layers indicate the support of paging cause:

4>	forward the ue-Identity, accessType (if present) and paging cause (if determined) to the upper layers;

3>	else:

4>	forward the ue-Identity and accessType (if present) to the upper layers;

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other';

[…]

According to the current specification, in case both 5G-S-TMSI & fullI-RNTI are included in the Paging message and the UE follows the procedure steps, the UE after detection of its fullI-RNTI and initiation of RRC Resume procedure still follows the remaining procedures steps (i.e. the procedure is not ended at this step) which may result in detection of its 5G-S-TMSI, RRC_INACTIVE to RRC_IDLE transition followed by RRC establishment procedure.

The above unclear situation indicates that the standard is not prepared to handle double paging on the UE side, which considering that double paging is currently not allowed and should not occur, should not be an issue.

It is worth noting, that double paging scenarios described in the previous section were discovered by us during IODTs and all tested UEs behaved the same way. They reacted on fullI-RNTI and ignored 5G-S-TMSI most probably following the standard procedure steps and ending the processing of Paging message after initiation of the RRC Resume procedure.

Observation 3: Although the UE's support for dual paging as part of the current standard may raise some doubts, all UEs during the tests responded to RAN-initiated paging only which was not a problem for the system.

## 2.3	Single paging requirement implementation issues

Since at least Scenario 2 described in Section 2.1 cannot be avoided by the gNB, our further analysis was directed towards the feasibility study of ‘one PagingRecord for each UE’ requirement implementation in RAN based on duplicate detection and discard.

As a result, we concluded that the implementation of this requirement based on the existing standard is not possible because:

- gNB-DU which generates RRC Paging message is not aware of the 5G-S-TMSI of the UE for which it receives F1AP PAGING message for fullI-RNTI based RAN-initiated paging,

- gNB that does not store UE context but serves cell(s) belonging to UE’s RNA area (i.e., it receives XnAP RAN PAGING messages for fullI-RNTI based RAN-initiated paging from another gNB) is not aware of the 5G-S-TMSI of the UE.

- gNB-CU that stores RRC_INACTIVE UE context is not always aware of 5G-S-TMSI of the UE for which it sends F1AP PAGING message for fullI-RNTI based RAN-initiated Paging (it is aware of 5G-S-TMSI only if the UE context is created as a result of RRC connection establishment procedure).

- gNB-CU that stores UE context is not updated with the new 5G-S-TMSI when it is changed by AMF during NAS procedures.

- in the case where the gNB-CU is aware of 5G-S-TMSI of the UE in RRC_INACTIVE state and generates two F1AP PAGING messages (not necessarily simultaneously or nearly simultaneously), it cannot determine whether the two pagings will be send in the same RRC Paging message, because:

  - mapping of paging records to POs / RRC Paging messages is an exclusive function of gNB-DU,

  - gNB-CU is not always able to predict the PO in which the paging record will be sent by gNB-DU due to:

    - the lack of standard requirements for F1 interface message transfer delay, gNB-DU message processing delay, maximum allowed time between F1AP PAGING message and the next available PO,

    - unknown status of the paging record(s) transferred earlier (there is no L3 acknowledgement for F1AP PAGING message),

    - usage of gNB-DU controlled paging strategies comprising low priority paging discard and/or paging postponing in case of PO congestion and gNB-DU paging repetitions.

Observation 4: The implementation of the ’one PagingRecord for each UE’ requirement for the 5G-S-TMSI & fullI-RNTI case based on the current standard is not possible.

## 2.3	Proposed solution

In our view, the ‘one PagingRecord for each UE’ requirement for the 5G-S-TMSI & fullI-RNTI case is neither needed nor currently implementable. Thus, we propose to relax this requirement so that the simultaneous 5G-S-TMSI & fullI-RNTI paging is allowed.

Proposal 1: RAN2 to decide whether the 38.331 Clause 5.3.2.2 ‘one PagingRecord for each UE’ requirement for the simultaneous 5G-S-TMSI CN-initiated and fullI-RNTI RAN-initiated paging scenario is needed.

Proposal 2: If the existing limitation is not needed relax ‘one PagingRecord for each UE’ requirement so that simultaneous 5G-S-TMSI CN-initiated and fullI-RNTI RAN-initiated paging is allowed. Draft CR for 38.331 Rel-18 is provided for discussion and agreement [1].

Proposal 3: If the existing limitation is needed send LS to RAN3 asking for specification of signaling enabling implementation of ‘one PagingRecord for each UE’ requirement for the simultaneous 5G-S-TMSI CN-initiated and fullI-RNTI RAN-initiated paging scenario.

# 3	Conclusion

This document has made the following observations:

Observation 1: There are scenarios that result in double paging (5G-S-TMSI & fullI-RNTI) caused by interaction between paging and UE context release procedures.

Observation 2: Avoidance of scenarios causing double paging may unnecessarily delay NGAP UE Context Release procedure and/or limit the use of gNB-DU paging strategies for RAN-initiated paging.

Observation 3: Although the UE's support for dual paging as part of the current standard may raise some doubts, all UEs during the tests responded to RAN-initiated paging only which was not a problem for the system.

Observation 4: The implementation of the ’one PagingRecord for each UE’ requirement for the 5G-S-TMSI & fullI-RNTI case based on the current standard is not possible.

And proposed the following:

Proposal 1: RAN2 to decide whether the 38.331 Clause 5.3.2.2 ‘one PagingRecord for each UE’ requirement for the simultaneous 5G-S-TMSI CN-initiated and fullI-RNTI RAN-initiated paging scenario is needed.

Proposal 2: If the existing limitation is not needed relax ‘one PagingRecord for each UE’ requirement so that simultaneous 5G-S-TMSI CN-initiated and fullI-RNTI RAN-initiated paging is allowed. Draft CR for 38.331 Rel-18 is provided for discussion and agreement [1].

Proposal 3: If the existing limitation is needed send LS to RAN3 asking for specification of signaling enabling implementation of ‘one PagingRecord for each UE’ requirement for the simultaneous 5G-S-TMSI CN-initiated and fullI-RNTI RAN-initiated paging scenario.

# 4	References

[1]	R2-2502619, Simultaneous RAN and CN paging, Rel-19, Nokia, RAN2#129-bis