3GPP TSG RAN WG2 #129bis	R2-2502620

Wuhan, China, April 7 – 14, 2025

Agenda Item:	8.9.2
Source:	Toyota ITC
Title:	Discussion on Paging and Mode Switching

Document for:	Discussion and Decision

# Introduction

The RAN plenary has approved a work item for Internet of Things (IoT) Non-Terrestrial Networks (NTN) [1]. One area of focus is the store-and-forward operation, where data can be transmitted to a satellite for storage if the feeder link is unavailable. The stored data can then be offloaded to a ground station once the feeder link is restored. This document discusses the paging and mode switching aspects of this feature.

# Discussion

## Paging Monitoring

The store-and-forward feature allows a satellite to function in real-time mode for two-way connections or in store-and-forward mode for one-way traffic from IoT devices to satellite storage. Several contributions have highlighted the energy consumption associated with monitoring paging channels when satellites switch to store-and-forward operating mode. In the absence of incoming mobile-terminated traffic, it is suggested that User Equipment (UE) should not monitor these channels in idle mode.

This contribution addresses energy efficiency solutions while ensuring reliable mobile data delivery. The challenge is that mobile data may arrive before, but paging may happen after mode switching. Additionally, feeder link delays can cause mobile-terminated data to arrive post-switch. Consequently, if the UE ceases monitoring paging immediately after mode switching, data loss and resource inefficiencies may occur due to paging without a response.

Figure 1 illustrates the scenario where the next paging occasion takes place after mode switching. The UE is expected to receive the paging message at the start of the store-and-forward mode and respond accordingly.

Figure 1: Illustration of a scenario where the paging message transmission and paging response can take place after the switching to store-and-forward mode.

Solution A

In [2], it is suggested that when no mobile-terminated data exists for an area, the UE should be notified to save energy by not monitoring paging.

The advantage of this solution is its simplicity, requiring only notification to all UEs. This approach is also network-controlled, which aids in ensuring the reliability of mobile-terminated data delivery.

This solution's disadvantage is its reliance on the area. If any mobile-terminated data exists, all UEs within the same area should wait for the data to be delivered to all UEs before ceasing to monitor paging. In addition, multiple transmission attempts may be required for successful data reception to a single UE, causing delays for everyone. A satellite's large coverage area with many UEs decreases efficiency since a wider area increases the chances of paging after mode switching.

Solution B

In [3], it is proposed that the last time the feeder link was available should be provided to the UE for deciding whether to monitor paging.

This solution's advantage is its independence from other UEs because the decision is made locally.

The disadvantage is that local implementation-specific UE decisions can impact data delivery reliability. If UEs stop monitoring paging too early, the network may waste resources by paging unresponsive UEs, and UEs may miss incoming data.

Solution C

The solution requires the UE monitoring N paging occasions after entering store-and-forward mode, as configured by the network. The delay in the feeder link and the interval until the next paging occasion can be adjusted by setting the appropriate value of N. This ensures both reliable data delivery and energy efficiency through configuration.

The advantage of this solution is its configurability, making data delivery more reliable than Solution B where the decisions are local, while still providing energy efficient paging monitoring. Unlike solution A, solution C operates independently of other UEs, following standardized network-controlled behavior.

The disadvantage is that signaling the value of N needs slightly more encoded bits than indicating presence of mobile-terminated data for an area.

Standardizing solution C is recommended due to its benefits, including improved energy efficiency and reliable mobile-terminated data delivery.

Proposal 1: The UE is required to monitor N paging occasions after switching from real-time to store-and-forward mode, with N configured by the network.

## Mode Switching

Another important aspect to consider is the impact of switching operating modes. Transitioning to store-and-forward mode halts all real-time traffic, while transitioning back to real-time mode re-enables it. For the real-time UEs, this process may lead to an abrupt surge of traffic, particularly on contention-based resources, due to simultaneous access by many UEs. The previous meeting briefly discussed two scenarios for transitioning to real-time mode: switching with cell restart and without cell restart.

Use case A: With cell restart

If transitioning back to real-time mode requires cell restart, access class barring may be needed to temporarily throttle arrivals until the system can handle the load. This mechanism gradually adjusts barring parameters and eventually grants access to all users as soon as possible.

Use case B: Without cell restart

If transitioning back to real-time mode is possible without cell restart, access class barring can also be employed. However, the main issue is the potential simultaneous access of many UEs caused by the mode switching rather than the traffic load itself. Additionally, access class barring operates at a very slow pace due to its reliance on a network-driven control-loop mechanism. To manage real-time UE access upon mode switching in this use-case, a randomization method could be used. After switching to real-time mode, users would delay their access attempts by drawing a random number within a transmission window and transmitting it when the timer expires. This approach spreads out access attempts within the start of the real-time modes without a control loop, reducing mode switching impacts without using access class barring.

Proposal 2: Standardize a signaling mechanism to avoid traffic surges caused by simultaneous access when transitioning from store-and-forward to real-time operating mode without cell restart.

# Conclusion

This contribution discusses uplink capacity enhancements. Observation and proposals are summarized as follows:

Proposal 1: The UE is required to monitor N paging occasions after switching from real-time to store-and-forward mode, with N configured by the network.

Proposal 2: Standardize a signaling mechanism to avoid traffic surges caused by simultaneous access when transitioning from store-and-forward to real-time operating mode.

# Reference

- RP-240776, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, March 18-21, 2024.

- R2-2408108, “Further consideration on Store and Forward”, Huawei, HiSilicon, Turkcell, China Southern Power Grid, RAN2#127bis

- R2-2408282, “Discussion on the Store and Forward satellite operation”, HONOR