3GPP TSG-RAN WG2 #129bis	R2-2502628

Wuhan, China, 07th – 11st April 2025

Agenda Item:	8.3.3 Measurement event predictions

Source:	Ericsson

Title:	SLS Results for Event Predictions

Document for:	Discussion

# Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 [1]. In this contribution, SLS aspects of the RRM measurement prediction models are discussed and the relevant simulation results are included.

In RAN2#128 meeting, the following agreements have been taken about SLS aspects [2]:

Agreements on SLS: - As for handover model it is proposed to agree for both temporal domain case A and case B: Network start with handover preparation once a predicted measurement event is received. A handover command will be transmitted at least after preparation is completed. After that one fixed execution time is assumed. - To reuse HO failure model and corresponding metrics i.e. HO failure rate, total number of HO attempts per UE per second from 36.839 - The agreed parameters in section 2.1.3 for measurement event prediction can be reused for SLS - The agreed parameters in table 2.2.2-1 i.e. all but last 3 parameters can be reused for both FR2 temporal domain case A and FR1 temporal domain case B in SLS - Interference model in section 2.2.3 is reused for SLS - The handover preparation time and execution time are x and y ms for both FR1 and FR2 (X and Y are the same for FR1 and FR2, CB the exact value] - [CB]As for simulation based on temporal domain case B, agree following approach: If a predicted A3 event at t1 is reported at t0 (t0<=t1) then HO command is transmitted at t3, where t3=t0+max(HO prep time, t1-t0). [CB – discuss time when handover command is transmitted]

This contribution presents results for FR2 Case A simulations, including both option 2 and option 3. The focus on FR2 was chosen because previous studies indicated strong performance in FR1, making FR2 more compelling for the SLS evaluation.

# Discussion

For SLS simulation two options has been agreed upon when it comes to how actions should be taken from the predictions made by the AI model. In Figure 1 an illustration of the two options can be seen.

Figure 1: Illustration of option 2 and option 3 for action related to HO procedures when predictions are involved.

In option 2 predictions are done when one cell fulfils the entering condition and if an A3 event is deemed to be fulfilled from the predictions then handover preparation is started by the network. But the network then waits with sending the handover command until the UE reports that the A3 event is also fulfilled based of measurements. The effect of this is that the handover execution done by the UE is delayed compared to option 3 but still faster than legacy. Option 3 on the other hand carries out both HO preparation and sends HO command to the UE purely based on predictions this means that HO execution is started earlier but can be more effected by errors in the predictions.

The used values for A3 parameters and non SLS related parameters are presented in Table 1.

Table 1: Parameters used in the SLS and none SLS evaluation.

| Parameters | Baseline value |
| --- | --- |
| A3 event offset (db) | 2 |
| TTT (ms) | 320 |
| UE speed (km/h) | 60,90,120 |
| OW length (ms) | 400 |
| PW length (ms) | 320 |
| Max ETD (ms) | 80 |

## Results

Results from the SLS study is presented in this part and the results are compared with the intermediate KPIs, for example F1-score. First is a study on the intermediate results done and a hypothesis on what effect they can have on the SLS results. After that the SLS results are presented and following discussion around if the hypothesis align with them.

### Intermediate KPI’s

In Table 2 intermediate KPI’s for FR2 case A can be found. Results for AI-model and sample and hold is presented, which is the agreed baseline for the study.

Table 2: Intermediate KPI's

| Speed | 60 Kph |  | 90 Kph |  | 120 Kph |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | AI-model | Sample and Hold | AI-model | Sample and Hold | AI-model | Sample and Hold |
| True A3 Events | 5764 | 5790 | 6827 | 6857 | 8570 | 8590 |
| False A3 Events | 169 | 358 | 217 | 386 | 195 | 337 |
| Missed A3 Events | 26 | 0 | 30 | 0 | 20 | 0 |
| Precision | 0,971 | 0,941 | 0,969 | 0,946 | 0,977 | 0,962 |
| Recall | 0,995 | 1 | 0,995 | 1 | 0,997 | 1 |
| F1 score | 0,983 | 0,970 | 0,982 | 0,972 | 0,987 | 0,980 |
| Avg RSRP error (db) | 0,832 | 1,136 | 0,773 | 1,263 | 0,822 | 1,355 |

From the table, we can observe that the AI model generates fewer false events than the Sample and Hold model. However, with this the AI model occasionally misses events that the Sample and Hold model successfully captures. This can be seen as a consequence of fewer false predictions by the AI model. When we analyze the impact of missed and false event predictions on system performance, we find that missed event predictions results in having minimal effect on the system. This is because the AI model is implemented to raise events based on both predictions and measurements. Consequently, a missed event by the AI will be detected by legacy logic, resulting in "only" legacy behavior.

On the other hand, falsely predicted events can have a more significant impact on the system. They can lead to the following types of handovers:

- Handover to a suboptimal cell, which is still good enough to prevent Radio Link Failure (RLF) or Handover Failure (HOF) and good enough to not trigger a handover back to the source cell or to a new cell.

- Handover to a poor cell, resulting in RLF or HOF due to inadequate coverage by the new serving cell.

- Handover to a suboptimal cell, causing a Ping-Pong handover or a brief duration of stay in the new cell.

In considering the potential impact of the three scenarios one expects the latter two to have the biggest effect on end-user performance. The first scenario has some associated effects, but they are not as severe since you are still in a cell that is less than an A3 event offset off from the best cell. A3 event is what triggers handovers in the tested scenario. Therefore, it would be reasonable to anticipate that the Sample and Hold model might be penalized for its higher rate of false predictions compared to the AI model.

On the intermediate KPI’s the Sample and hold have more false event predictions compared to AI-model.

False event prediction carries a higher cost than missed event predictions in the SLS implementation used.

### SLS results

The models tested in the SLS runs include AI-model options 2 and 3, the Sample and Hold model, and legacy implementations with Time-To-Trigger (TTT) settings of 320 ms and 160 ms. So, in addition to the agreed-upon setups, we also tested a legacy implementation with a shorter TTT than the agreed one. This shorter TTT is half of the duration used in both the AI models and the agreed legacy implementation.

All simulations were conducted with a static number of User Equipments (UEs) and identical simulation durations, allowing for direct comparison of absolute numbers across different models. Simulations were performed with users moving at speeds of 60, 90, and 120 kph.

#### Radio Link Failures

The number of radio link failures which happened during the simulation runs can be seen in Figure 2.

Figure 2: Graph showing the number of radio link failures for the different models.

The results indicate that the AI model utilizing option 3 actions, along with the Sample and Hold model, demonstrate the best performance in terms of minimizing radio link failures. Additionally, the AI model option 2 shows result closer to the legacy simulation. This is expected, as option 2 takes actions solely based on measurements, with only handover preparation occurring based on predictions. In scenarios involving extended handover preparation, this setup can save approximately 40 milliseconds, potentially giving it an edge over the legacy model. However, the simulations clearly show that further improvements can be achieved by also issuing the handover command based on predictions. Furthermore, it's evident that the legacy implementation with a TTT of 160 ms outperforms the legacy model with a TTT of 320 ms. This may suggest that the agreed-upon TTT is longer than optimal.

AI model with option 3 and Sample and Hold is the best performing model in terms of radio link failures for the agreed TTT.

AI model with option 2 is better than the legacy approach with the agreed TTT but is beaten by Option 3.

### Number of handovers

Figure 3 illustrates the number of handovers per UE per second that occurred during the simulation runs.

Figure 3: Graph showing the number of handovers per UE per second for the different models and the different speeds.

Figure 3 shows that the AI model using option 2 closely aligns with the legacy implementation, which is expected given that both models make decisions based on measurements. Conversely, the AI model with option 3 results in a higher number of handovers compared to both legacy setups and AI option 2. Notably, the Sample and Hold model has the highest number of handovers among all the tested configurations. However, it is important to note that the number of handovers alone may not be a critical metric in assessing overall performance more as a check to see that the AI model option 3 do not trigger an unreasonable number of handovers.

AI model with option 3 has more handovers compared to both AI model with option 2 and legacy implementation.

AI model with option 3 do not seem to trigger an unreasonable number of handovers.

Sample and Hold has the greatest number of handovers.

Legacy implementation with 160 ms TTT has more handovers than legacy implementation with 320 ms TTT.

#### Other Handovers KPIs

In addition to KPIs such as the number of radio link failures and handovers, it is valuable to examine other parameters. For this meeting, the effect of different setups on the number of short times of stay (ToS), the occurrence of Ping-Pong handovers, and the duration spent in the best cell was investigated. Table 3 presents the results related to these KPIs for the simulation run with UEs moving at 120 kph.

Table 3: Table showing number of Ping-Pong-, Short stay-handovers and percentage of time spent in the best cell.

| Speed | 120 Kph |  |  |  |
| --- | --- | --- | --- | --- |
| Setup | Ping Pong | Short ToS | Ratio in Best Cell | Total Number of Handovers |
| Legacy TTT 320ms | 1 | 31455 | 0,75 | 188706 |
| Legacy TTT 160ms | 110 | 36034 | 0,81 | 195419 |
| AI Option3 | 314 | 40590 | 0,88 | 201409 |
| AI Option2 | 0 | 31604 | 0,75 | 188844 |
| Sample and Hold | 406 | 42971 | 0,88 | 204150 |

The table reveals that the number of Ping-Pong handovers increases with AI option 3 and the Sample and Hold model. Additionally, in the legacy setup with a shorter TTT, the number of Ping-Pong handovers is higher compared to the run with a TTT of 320 ms. However, the overall number of Ping-Pong handovers remains low, as expected with such fast-moving UEs in a deployment characterized by relatively short inter-site distances.

Number of Ping-Pong handovers are higher for AI option 3 and Sample and hold compared to AI option 2 and Legacy.

The overall number of Ping-Pong handovers are low.

There is a significant number of short ToS instances, where a UE is handed over shortly after a previous handover. This is not unexpected for fast-moving UEs in a deployment with short inter-site distances. The AI model option 2 exhibits fewer short ToS compared to the Sample and Hold model and AI option 3, but it aligns with the legacy setup with a TTT of 320 ms. In this deployment, the inter-site distance is 200 meters, and at a speed of 120 kph, or 33.3 meters per second, a handover would ideally occur approximately every 3 seconds, assuming the UE consistently crosses the "widest" part of the cell, which is not always the case. Consequently, it is reasonable to expect that about one-fifth of all handovers result in short ToS if these stays are categorized as such when they last less than one second.

AI option 2 results in fewer short time of stay incidents compared to AI option 3.

The ratio of time spent in the best cell shows that the Sample and Hold model has the highest performance at 88.2%, followed closely by AI option 3 at 88.0%. Both models outperform the legacy setups, with the legacy TTT 160 ms configuration achieving 80.6%, compared to 74.8% for the legacy TTT 320 ms setup. AI option 2 matches the legacy TTT 320 ms setup with a ratio of 74.8%. These results suggest that AI option 3 and Sample and Hold are more effective at keeping UEs in the optimal cell, highlighting the advantages of predictive actions and shorter TTT durations, especially in a simple scenario with such a high velocity user as we have here.

The Sample and Hold model and AI option 3 achieve the highest ratios of time spent in the best cell, for the agreed scenario.

Both models outperform legacy setups, indicating that predictive actions and shorter TTT durations can improve UE retention in optimal cells, for the agreed scenario and TTT.

### Correlation of Intermediate KPIs and SLS KPIs

To summarize the contribution, the SLS results appear consistent with the intermediate KPIs. Both the Sample and Hold model and the AI model exhibit similar F1-scores, with SLS results closely aligned for Sample and Hold and AI option 3. AI option 2 matches the legacy implementation with the same TTT, yet demonstrates improved performance compared to the legacy model. One theoretical advantage of option 2 over option 3 is its increased robustness against incorrect AI predictions. However, our scenario may be too simplistic, and the AI models perform sufficiently well for this advantage to become apparent. For example, the number of radio link failures, Ping-Pongs and RSRP error are low for all setups. Also Sample and Hold perform sufficiently good which also indicate that the tested scenario is simplistic.

Intermediate KPIs seems to align with SLS results.

The scenario tested may not be sufficiently challenging for the AI models, resulting in AI option 3 yielding better outcomes while option 2's design strengths remain underexplored. Sample and hold’s good performance also points in this direction.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	On the intermediate KPI’s the Sample and hold have more false event predictions compared to AI-model.

Observation 2	False event prediction carries a higher cost than missed event predictions in the SLS implementation used.

Observation 3	AI model with option 3 and Sample and Hold is the best performing model in terms of radio link failures for the agreed TTT.

Observation 4	AI model with option 2 is better than the legacy approach with the agreed TTT but is beaten by Option 3.

Observation 5	AI model with option 3 has more handovers compared to both AI model with option 2 and legacy implementation.

Observation 6	AI model with option 3 do not seem to trigger an unreasonable number of handovers.

Observation 7	Sample and Hold has the greatest number of handovers.

Observation 8	Legacy implementation with 160 ms TTT has more handovers than legacy implementation with 320 ms TTT.

Observation 9	Number of Ping-Pong handovers are higher for AI option 3 and Sample and hold compared to AI option 2 and Legacy.

Observation 10	The overall number of Ping-Pong handovers are low.

Observation 11	AI option 2 results in fewer short time of stay incidents compared to AI option 3.

Observation 12	The Sample and Hold model and AI option 3 achieve the highest ratios of time spent in the best cell, for the agreed scenario.

Observation 13	Both models outperform legacy setups, indicating that predictive actions and shorter TTT durations can improve UE retention in optimal cells, for the agreed scenario and TTT.

Observation 14	Intermediate KPIs seems to align with SLS results.

Observation 15	The scenario tested may not be sufficiently challenging for the AI models, resulting in AI option 3 yielding better outcomes while option 2's design strengths remain underexplored. Sample and hold’s good performance also points in this direction.

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

RAN2 Chairman’s Notes, RAN2#128