3GPP RAN WG2 Meeting #129bis	R2-2502637

Wuhan, China, April 7th – 11th 2025

Agenda Item:	8.1.2.2

Source:	InterDigital

Title:	LCM for UE-sided model for Beam Management use case

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback. In RAN2#127 [2] the following procedure for reporting applicable functionalities was agreed:

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.

This contribution discusses the above procedure for applicable functionality determination and reporting, specifically remaining details on the applicable functionality reporting procedure.

# Discussion

## Option B) Inference related parameters for applicability report

UE determines applicable functionalities based on configurations/parameters provided in Step 3. Possible configurations include: A) one or more of CSI-ReportConfig for inference configuration; and/or B) One set or multiple sets of inference related parameters for applicability report only (not for inference). It is noted that these configuration(s) are considered in addition to the NW-side additional conditions (if provided), UE-side additional conditions and model availability in device.

Signalling for the full inference configuration (i.e., Option A) is via CSI-ReportConfig. Signalling for inference related parameters for applicability report (i.e., Option B) remains FFS according to the following agreements:

- The full inference configuration is sent in CSI-ReportConfig.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

Using CSI-ReportConfig to signal Option B allows a unified signaling solution for both Options, but has several drawbacks compared to OtherConfig. For example, CSI-ReportConfig is heavily related to the beam management use case, whereas OtherConfig is a more general signalling framework which may be extended to other AIML use cases, providing a more future proof signaling framework for applicability reporting.

Observation 1:	Using OtherConfig for signalling inference related parameters (i.e., Option B) allows for easy extention to other AIML use cases, improving future compatability.

CSI-reportConfig contains many parameters which may not be needed for applicability determination, some of which are mandatory. This can increase signaling overhead compared to OtherConfig where only parameters strictly needed for applicability determination and reporting can be included. It is noted however that final conclusion on parameters used for applicability determination is still being discussed in RAN1, and the scale of this issue is reliant on this discussion outcome.

Observation 2:	Using OtherConfig for signalling inference related parameters (i.e., Option B) reduces signaling overhead by only including parameters needed for applicability determination.

Therefore, to support a future proof and signalling efficient solution, it is proposed that inference related parameters for applicability report (i.e., Option B) be sent via OtherConfig.

Proposal 1:	Signal inference related parameters for applicability report (Option B) via OtherConfig.

UE performs applicable functionality reporting in Step 4 via RRCReconfigurationComplete for both Option A and B. For Option A, since the full inference configuration is already available as of Step 3, for periodic CSI reporting the UE autonomously activates the applicable functionalit(ies) upon reporting applicable functionalities via RRCReconfigurationComplete in Step 4. Activation of inference configuration for Option B is currently FFS, according to the following agreements [5]:

- If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- FFS option B

For Option B, the network may provide the corresponding full inference configuration for applicable functionalities in Step 5 via a subsequent RRCReconfiguration message. However, since the NW has already received the applicability report in the previous RRCReconfigurationComplete message (from Step 4), the UE can immediately activate inference upon receiving RRCReconfiguration in Step 5.

Observation 3:	For Option B, UE can immediately activate applicable functionalities upon receiving full configurations in RRCReconfiguration at Step 5 since applicable reporting already occurred.

Therefore, if Option B is configured in Step 3, for periodic CSI reporting, the UE autonomously activates applicable functionalities upon receiving RRCReconfiguration containing the corresponding full inference configuration in Step 5. (i.e., without waiting to transmit the subsequent RRCReconfigurationComplete message).

Proposal 2:	If Option B is configured in Step 3, for periodic CSI reporting, the UE autonomously activates applicable functionalities upon receiving RRCReconfiguration containing the full inference configuration in Step 5.

## Applicability reporting

A high-level framework for applicability reporting was agreed in RAN2#127 [2] and further refined in RAN2#127bis [3], RAN2#128 [4], and RAN2#129 [5] based on the following agreements:

RAN2#127bis: - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signalled. FFS on the applicability reporting content. - UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality) RAN2#128: - When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network. - FFS detailed design - FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability. RAN2#129: - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause - Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

It is currently FFS whether the UE reports the explicit cause of non-applicability. A functionality may be non-applicable for many reasons outside of network control, such as model unavailability, model not applicable/trained for current UE side additional conditions (e.g., model not trained for the current UE speed), etc. However, if a functionality is not applicable due to missing information from the network (e.g., inference configuration), this may be easily corrected (e.g., via a UE request/response procedure, or via proactive reporting etc.). In addition to reporting a function as “applicable” or “non-applicable”, the UE should indicate the reason for non-applicability (e.g., model not available, not trained for current UE conditions, missing information, UE/NW side conditions etc.).

Proposal 3:	UE reports the explicit cause an AI/ML functionality is non-applicable.

# Conclusion

In this contribution the following observations and proposals are made regarding LCM for beam management:

Observation 1:	Using OtherConfig for signalling inference related parameters (i.e., Option B) allows for easy extention to other AIML use cases, improving future compatability.

Observation 2:	Using OtherConfig for signalling inference related parameters (i.e., Option B) reduces signaling overhead by only including parameters needed for applicability determination.

Observation 3:	For Option B, UE can immediately activate applicable functionalities upon receiving full configurations in RRCReconfiguration at Step 5 since applicable reporting already occurred.

Proposal 1:	Signal inference related parameters for applicability report (Option B) via OtherConfig.

Proposal 2:	If Option B is configured in Step 3, for periodic CSI reporting, the UE autonomously activates applicable functionalities upon receiving RRCReconfiguration containing the full inference configuration in Step 5.

Proposal 3:	UE reports the explicit cause an AI/ML functionality is non-applicable.

# References

RP-243244 – WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface – Qualcomm

R2-2409210 - RAN2#127 chair notes

R2-2409502 - RAN2#127bis chair notes

R2-2500002 - RAN2#128 chair notes

Draft-RAN2_129_Meeting_Report