3GPP RAN WG2 Meeting #129bis	R2-2502638

Wuhan, China, April 7th – 11th 2025

Agenda Item:	8.1.2.3

Source:	InterDigital

Title:	LCM for Positioning use case

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback. In RAN2#127bis [2] the following procedure for reporting applicable functionalities was agreed for Case 1 AIML positioning:

1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

With further agreements in RAN2#128 [3]:

1: For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

And RAN2#129 [4]:

1: Introduce AI/ML positioning Case 1 as a new positioning method. 2: Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR. 3: FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed. FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation, 4: The content of error cause is discussed while drafting stage3 CRs. 5: As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list. 6: UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed.

This contribution discusses aspects of the above procedure for applicable functionality determination and reporting, specifically remaining details on the applicable functionality reporting procedure .

# Discussion

## Applicability reporting

UE reports a change of applicable functionality to the LMF via an unsolicited LPP provide capabilities message. Unlike UAI (used in beam management) which has built in control mechanisms to restrict excessive reporting, there are no LMF controls on UE transmission of an unsolicited LPP provide capabilities message. It is currently FFS whether additional LMF control is needed.

A key reason to report a change in applicability is to prevent continued inference with a non-applicable functionality. Although the network could implicitely determine a functionality has become non-applicable by receiving an error message when requesting the inferred location information, this can introduce unnecessary delay in both deactivating the non-applicable functionality and in receiving the UE position (e.g., if no alternative positioning method is configured). Therefore, there should at least be no LMF control on reporting a change from applicable to non-applicable.

Observation 1:	Relying on an error message to determine a functionality has become on-applicable increases positioning delay, especially if no other (e.g., non-AIML) positioning method is configured.

The main concern with no LMF control on unsolicited capability reporting comes if there is also no LMF control on reporting a change from non-applicable to applicable, which can lead to a scenario where applicability constantly changes from applicable to non-applicable and vice versa causing excessive signalling overhead. However, this seems to most likely be a corner case sceanario considering a positioning functionality should be valid over an area. Also, if the network notices a configuration constantly changing, it can simply deactivate the functionality altogether to prevent further applicability reporting.

Observation 2:	If a functionality is constantly oscillating between applicable and non-applicable, the network can simply de-configure the functionality to prevent excessive applicability reporting.

It is therefore proposed that no additional LMF control is introduced to restrict applicability reporting via unsolicited LPP ProvideCapabilities message, which is consistent with legacy behaviour.

Proposal 1: 	No additional LMF control is introduced to restrict applicability reporting via unsolicited LPP ProvideCapabilities message.

## Functionality (de)activation

RAN2 has the following understanding on terminologies for AIML LCM:

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refer to functionalities that the UE is ready to apply for inference

- Activated functionalities refer to functionalities already enabled for performing inference

Based on existing agreements from RAN2#127bis, UE uses the LPP ProvideCapabilities message to indicate supported functionalities to the LMF in Step 2 and applicable functionalities in Step 4. When the UE considers a functionality as “activated” is not currently defined in AIML Positioning use case.

Upon indication of applicable functionalities, the LMF can request the inferred location information using the LPP RequestLocationInformation message in Step 5 and UE reports the inferred location using LPP ProvideLocationInformation message in Step 6. A functionality should therefore be considered as “activated” upon receiving a request for AIML positioning from LMF in Step 5.

Proposal 2: 	An AIML positioning functionality is considered “actived” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information.

Notifying the LMF that a previously reported applicable functionality has become non-applicable is important to ensure functionality deactivation and appropriate reconfiguration (e.g., to legacy non-AIMl positioning method) to support the continuation of accurate positioning. In RAN2#129 it was agreed that UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality, including from applicable to non-applicable.

Upon reception of the non-applicability message, in the BM use case the network is expected to deactivate the non-applicable functionality to avoid continued inference (which will most likely yield poor results). The same behaviour should be expected from the LMF upon reception of a non-applicability indication.

Proposal 3: 	LMF is expected to deactivate actived functionality when it receives non-applicability indication from UE (i.e., UE doesn’t autonomously deactivate).

## Fallback

In RAN2#128 the following it was agreed regarding UE behaviour when it cannont fulfill AIML-based positioning due to functionality non-applicability [3]:

- If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP ProvideLocationInformation message with error cause.  FFS if other fallback options are considered.

If for any reason UE cannot perform the AIML based positioning upon LMF location estimation request, it is currently under discussion whether there should be a fallback option apart from the error reporting. Based on discussion in [4], views were divergent with those supportive of introducing fallback mechanisms primarily noting the latency benefits in e.g., UE autonomous switching to a non-AIML positioning method. The following is currently FFS [4]:

- FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

However, since UE can notify the network when a functionality becomes non-applicable, the LMF should be notified quickly of any change in applicability via a proactive unsolicited UE capability report. The error scenario then becomes that the UE detects a change in applicability and notifies the LMF, but receives a location estimation request before corrective action can be taken. As the time between these messages should be relatively short, this may be considered a corner case and should not violate any LCS client requirements which require a fallback method.

Furthermore, the LMF can configure multiple positioning methods simultaneously with existing specification. If additional redundancy is needed, the LMF can configure both an AIML-based and non-AIML based positioning method. Upon reception of an LPP ProvideLocationInformation message with error cause for the AIML based positioning request, the LMF can then request a location estimate from the non-AIML positioning method. Therefore, it is suggested that for this release, additional fallback options (e.g., UE autonomous switching) are not considered. This behaviour should be the case regardless of whether multiple positioning methods are configured or not, which is left to the network.

Proposal 4: 	UE-autonomous switching between AI/ML and non-AI/ML positioning methods is not supported within this release.

# Conclusion

In this contribution the following observations and proposals are made regarding LCM for Positioning use case:

Observation 1:	Relying on an error message to determine a functionality has become on-applicable increases positioning delay, especially if no other (e.g., non-AIML) positioning method is configured.

Observation 2:	If a functionality is constantly oscillating between applicable and non-applicable, the network can simply de-configure the functionality to prevent future applicability reporting.

Proposal 1: 	No additional LMF control is introduced to restrict applicability reporting via unsolicited LPP ProvideCapabilities message.

Proposal 2: 	An AIML positioning functionality is considered “actived” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information.

Proposal 3: 	LMF is expected to deactivate actived functionality when it receives non-applicability indication from UE (i.e., UE doesn’t autonomously deactivate).

Proposal 4: 	UE-autonomous switching between AI/ML and non-AI/ML positioning methods is not supported within this release.

# References

RP-243244 – WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface – Qualcomm

R2-2409502 – RAN2#127bis chair notes

R2-2500002 - RAN2#128 chair notes

Draft-RAN2_129_Meeting_Report