3GPP TSG-RAN WG2 Meeting #129bis	R2-2502641

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.10.4

Source:	Nokia

Title:	MRO for NTN RACH-less Access

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core – Rel-19

Document for:	Discussion and Decision

# Introduction

As part of Rel-18, RAN2 specified different features to reduce the signalling overhead in NR NTN and reduce mobility latency. One of these features was the RACH-less Handover (HO) that aims at skipping Msg1 and Msg2 of the RACH procedure during the access of the target cell.

This paper discusses details regarding the optimization of RACH-less access in NTN deployments and possible SHR & RLF Report enhancements. We also discuss an unresolved issue on area scope for logged MDT for NTN and propose a way forward solution.

# 2	RACH-less Access in NTN

The main functions of the Random Access (RA) procedure during HO are: i) to ensure TA alignment, ii) obtain power control parameters, and iii) obtain UL transmission grant for initial transmission over PUSCH. In terrestrial networks, with 2-step RA procedure, the UE obtains most of this information via Msg2 (i.e., Random Access Response or RAR message).

NTN deployments, despite being challenging due to the high-speed mobility of the network nodes (i.e., satellites), position of satellites are deterministic because the satellites follow fixed trajectories – this allows for some of the network parameters to be very much predictable. In NTN, parameters such as TA and doppler shift are pre-compensated at the UE side based on satellite ephemeris that the UE obtains from system information (i.e., SIB19).

RACH-less HO in NTN is supported as part of normal HO, Conditional HO (CHO) and also when using time-based CHO.

The schematic of RACH-less normal HO in NTN is given in Figure 1.

Figure 1: Handover Procedure with RACH-less Access of T-Cell

TA Alignment

The HO command (RRCReconfiguration message) will include an indication (NTA) whether the TA offset is zero or it is assumed to be the same as in the serving cell. The UE then uses this to compute the overall TA (including additional information broadcast in SIB19, using the UE location and satellite ephemeris). This TA is then used to do UL synchronization of the target cell (without using the RA procedure)

UL Grant for Target Cell

Two approaches were defined Configured Grant (CG) and Dynamic Grant (DG). Both of these are shown in Figure 1.

The UE receives a Type-1 CG (Configured Grant) as part of the RRCReconfiguration message – it contains parameters to indicate SSB index, time/frequency resource allocations, antenna port, MCS, power, and RSRP threshold among others. Once the UE has detached from the source cell, it attempts to transmit in the time and frequency resources contained in the CG only if the RSRP of the indicated SSB index is above a certain threshold.

Alternatively, the UE can be also configured to access via a DG. In this case, the UE receives in the HO command only the SSB/beam information of the target cell. Once the UE has detached from the source cell, detect the target cell, and acquired DL synchronization it starts monitoring the PDCCH channel waiting for a PUSCH transmission assignment (e.g., DCI format 0_0 or 0_1). In this case, the UE is not required to check for an RSRP threshold before transmitting.

Observation 1: RACH-less access involves optimization in multiple areas. It requires a lot of information to be transferred to the UE prior to or along with the HO Command which are processed by the UE to realize RACH-less access of the target Cell.

We look at some of the potential errors that could occur in the RACH-less access mechanism and the resultant failures and almost failures.

Figure 1 – Dynamic Grant case, Step (3): It has two sub-steps

Sub-step #1: After detecting the target cell and acquiring DL synchronization, UE uses quasi co-location (QCL) (SSB given in source as part of the RRC Reconfiguration vs DMRS of target) to monitor PDCCH of target.

Sub-step #2: The UE monitors PDCCH waiting for a DCI assignment for PUSCH transmission. The UL grant also contains additional information to perform the transmission such as modulation and coding formats, time and frequency resource allocations, HARQ configuration or power parameters.

The key thing to note here is that the UE does these actions on its own and the network is not aware of the time elapsed between successful reception of the HO command and the UE initiating the monitoring of the PDCCH.

For this reason, the NW must ensure the time of transmitting the DCI with PUSCH assignment in Sub-step #2 under step 3 (in other words, it has to consider if the UE had enough time to complete the Sub-step #1 and is “listening”).

Even though the RACH-less HO procedure may succeed, there could be issues like the UE having to  monitor the PDCCH for a longer time for it to acquire PUSCH related configuration (i.e, NW estimates more delay in when the UE will start monitoring PDCCH) or the NW may have to transmit the UL grant for repeated times over a given search space until the UE acquires it (i.e., due to NW underestimating the time when the UE will start monitoring). This issue may result in longer time for successful HO with the following consequences

- UE’s energy consumption increases if it must monitor the PDCCH longer

- Sub-optimal utilization of allocated resources: resources in the NW are allocated for longer duration in case of the NW has to send the UL grant repeatedly - these resources could be used for other UEs purposes)

Observation 2: The RACH-less access defined for NTN mobility could lead to sub-optimal scenarios which would require constant measurement, monitoring and performance optimization.

Since this is a successful (but sub-optimal) HO, the SHR would need to be enhanced to cover RACH-less access for NTN.

Proposal 1: RAN2 to consider enhancing SHR for RACH-less access for NTN.

We have covered the near-failures scenario above. The following are the possible NTN-specific failures scenarios that could happen in RACH-less access.

- Outdated/drifted SIB19 information for estimation of the target cell parameters received in the HO command. UE’s TA/doppler estimation may drift over time, if the estimation is off the optimal value, it may force the NW to allocate more resources to each PUSCH assignment to protect those resources.

- Power control parameters were not adjusted for latest channel impairments (a UE may receive sub-optimal PC parameters which may cause an unsuccessful PUSCH transmission).

- The ssb_index in HO Command could be changed/optimized. If a UE is located on the cell edge between two cells, it may result that the UE experiences better signal conditions on the adjacent cell (e.g., signals from both cells are fully uncorrelated since those belong to different satellites).

- Step 3 Sub-step 1 could fail because when the UE monitors for PDCCH, it is unable to decode the same due to bad radio conditions

- Step 3 Sub-step 2 could fail because the UE is unable to acquire the UL grant in time and timer T304 expires.

Observation 3: The RACH-less access defined for NTN mobility can lead to many failures scenarios that would require some MRO enhancements.

Since these failure result in RLF, the RLF Report would have to be enhanced for this purpose.

Proposal 2: RAN2 to consider enhancing RLF Report for RACH-less access for NTN.

# 3	 Enhancements to SHR & RLF Report

Successful HO report (SHR) was introduced in Rel-17 to capture near failure events that happen during a successful HO. SHR is triggered based on running timers (T310/T312/T304) exceeding a preconfigured threshold. In some cases, additional SHR triggering conditions have been defined.

In order to optimize the near-failures in RACH-less access in NTN, the enhancements could be two-fold – add measurements related to the scenario in SHR and define a new SHR trigger to measure these new metrics.

Enhance the Successful HO Report definition to add the following NTN RACH-less HO specific information

-	PDCCH Monitoring Time Until Success

-	No of PUSCH Attempts Until Success

-	Full TA, Doppler estimation and PC control parameters used during PUSCH transmissions

-	Target NTN-config in SIB19

-	Target NTN-config in RRC Reconfiguration message

Proposal 3: Enhance the SHR Report message definition with NTN RACH-less specific reporting.

Define a new SHR trigger to capture the newly defined measurements upon successful completion of NTN RACH-less HO

Proposal 4: Define a new SHR trigger for the UE to log the enhancements measurements specific to NTN RACH-less access.

All the above defined measurements are applicable for RLF cases too.

Additionally, a new RLF Cause value for NTN RACH-Less HO Failure should be added.

Proposal 5: Enhance the RLF Report message definition with NTN RACH-less specific measurements and a new RLF Cause specific to NTN RACH-less access failure.

# 4	 Area scope for logged MDT for NTN

In RAN2#129 there was some discussion on MDT for NTN based on an incoming LS from RAN3 [R2-2500026/R3-247908]. In the LS, RAN3 asked for confirmation from RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN. RAN2 discussed this question from RAN3 by discussing a proposal from R2-2501257 which said, “the signalling structure agreed for encoding geographical area in case of MBS NTN should be introduced for geographical area based logged MDT for NTN”. In essence, this proposal was saying it is feasible to reuse the geographical area concept defined for MBS NTN for Area Scope for logged MDT for NTN. Unfortunately, RAN2 could not reach consensus on the discussed proposal and stopped the discussion without any conclusions or agreeing on what to reply to RAN3. It is our understanding that RAN3 is waiting for a reply from RAN2. The action to RAN2 in the RAN3 LS R2-2500026/R3-247908 specifically states “RAN3 also kindly requests RAN2 to provide feedback to the question on MDT for NTN”. So, RAN2 needs to continue the discussion and decide on what to reply to RAN3.

In the discussions on this issue so far, 4 companies agreed to use the signalling structure agreed for encoding geographical area in case of MBS NTN should be introduced for geographical area based logged MDT for NTN, but 2 companies were against it. The main concern against this proposal is the requirement for UE in RRC_IDLE/RRC_INACTIVE to check UE position and the impact on UE power consumption if a geographical area scope is used.

LPP positioning protocol already supports positioning in RRC_IDLE and in Rel-17 and Rel-18 LPP was further enhanced to support positioning in RRC_INACTIVE along with other power savings enhancements for UE positioning. If the concern is about mandatory UE support of positioning protocols to be able to support MDT for NTN, then there are standalone positioning techniques (techniques UE can use without relying on LPP protocol support) including standalone GNSS positioning which is already used for MDT in LTE. RAN2 agreements to use area scope for MBS for NTN also demostrates that positioning requirement for UE and UE power consumption impacts are not an issue. On the other hand, NTN coverage area being broader than terrestrial network (TN) coverage area, it is more efficient to use a geographic area encoding to map NTN area to TN area than map the NTN area to a large list of TN cells. So, we think it is technically feasible to use the same area scope solution for NTN for both MBS and MDT. RAN2 should at least reply to RAN3 indicating it is technically feasible but leave the final decision to RAN3. RAN2 need not decide on signalling over Uu until RAN3 decides on the solution for area scope for MDT NTN.

Observation 4: LPP protocol already supports UE positioning in RRC_IDLE/RRC_INACTIVE and other UE power savings enhancements for positioning in Rel-17 and Rel-18.

Observation 5: There is no mandatory requirement for UE to support LPP and UE can use standalone positioning e.g., standalone GNSS which does not require LPP support or network support for any assistance.

Observation 6: NTN coverage area being broader than terrestrial network (TN) coverage area, it is more efficient to use a geographic area encoding to map NTN area to TN area than map the NTN area to a large list of TN cells

Proposal 6: RAN2 to send a reply LS for the incoming LS from RAN3, R2-2500026/R3-247908, and indicate it is technically feasible to use a similar Uu signalling structure as adopted for encoding geographical area in the case of MBS NTN, for geographical area based logged MDT for NTN.

# 4	Conclusion

This document has made the following observation and proposals for NTN RACH-less access optimization and MDT for NTN:

NTN RACH-less access optimization:

Observation 1: RACH-less access involves optimization in multiple areas. It requires a lot of information to be transferred to the UE prior to or along with the HO Command which are processed by the UE to realize RACH-less access of the target Cell.

Observation 2: The RACH-less access defined for NTN mobility could lead to sub-optimal scenarios which would require constant measurement, monitoring and performance optimization.

Proposal 1: RAN2 to consider enhancing SHR for RACH-less access for NTN.

Observation 3: The RACH-less access defined for NTN mobility can lead to many failures scenarios that would require some MRO enhancements.

Proposal 2: RAN2 to consider enhancing RLF Report for RACH-less access for NTN.

Proposal 3: Enhance the SHR Report message definition with NTN RACH-less specific reporting.

Proposal 4: Define a new SHR trigger for the UE to log the enhancements measurements specific to NTN RACH-less access.

Proposal 5: Enhance the RLF Report message definition with NTN RACH-less specific measurements and a new RLF Cause specific to NTN RACH-less access failure.

MDT for NTN:

Observation 4: LPP protocol already supports UE positioning in RRC_IDLE/RRC_INACTIVE and other UE power savings enhancements for positioning in Rel-17 and Rel-18.

Observation 5: There is no mandatory requirement for UE to support LPP and UE can use standalone positioning e.g., standalone GNSS which does not require LPP support or network support for any assistance.

Observation 6: NTN coverage area being broader than terrestrial network (TN) coverage area, it is more efficient to use a geographic area encoding to map NTN area to TN area than map the NTN area to a large list of TN cells

Proposal 6: RAN2 to send a reply LS for the incoming LS from RAN3, R2-2500026/R3-247908, and indicate it is technically feasible to use a similar Uu signalling structure as adopted for encoding geographical area in the case of MBS NTN, for geographical area based logged MDT for NTN.