3GPP TSG-RAN WG2 #129bis	R2-2502647

Athens, Greece, 7th – 11th April 2025

Agenda Item:	8.10.3

Source:	Ericsson

Title:	Discussion on slice base cell re-selection information in Logged MDT

Document for:	Discussion

# 1	Introduction

After RAN3#127 meeting, RAN3 has sent the following LS (R3-250613):

Overall description RAN3 has discussed how to optimize slice based cell reselection via e.g. logged MDT and identified the following scenario: A UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, thereby UE have to camp on cells belonging to the lower ranked NSAG or eventually use legacy cell reselection procedure to find a suitable cell. RAN3 would like to ask RAN2 to take the above scenario into account, and to discuss whether and how it can be addressed and whether any standard changes are needed.

In this contribution, we discuss the LS and provide arguments on the proposed solution

# 2	Discussion

## 2.1	Logging slice-based cell reselection information

The LS comprises of a scenario where a UE is unable to find a suitable cell in the frequencies corresponding to the highest ranked NSAG and thereby has to camp on a cell belonging to the lower ranked NSAG or use legacy cell reselection policies to find a cell.

For example, assume the network supports slice A in frequency F1. Hence, the network may configure the NSAG priorities at the UE stipulating the UE to reselect to frequency F1. The UE, however, is unable to find a suitable cell in frequency F1 and is forced to re-select to a cell served by frequency F2. It should be noted that the UE selected a cell in frequency F2 using legacy cell reselection priorities.

From network perspective, this may happen only if there is a coverage hole at F1 in the location of the UE.

The scenario described in the LS may occur only if there is a coverage hole in the frequencies corresponding to the highest ranked NSAG.

In current logged MDT procedure, the UE logs measurements for up to 3 inter-frequency neighbour cell per frequency. Thus, not finding a suitable cell in a given frequency F1 may result in below different behaviours from the UE.

- UE may log measurements of some neighbour cells in the F1. However, the measurement results would show that the quantities are poor.

- UE may not log any information regarding F1, since it was unable to measure any cell in the frequency.

In both cases, however, network can deduce coverage problem in F1. Since the UE may also provide location information in the logged MDT report, network would be able to identify any random coverage hole also.

Moreover, in legacy, network can configure the UE with an interfrequencyTargetInfo IE to perform measurements in a specific frequency. Any poor coverage or coverage hole can be identified by the network using the measurement results. Hence, network may configure frequency F1 as part of interfrequencytargetInfo and UE would log measurements for cells found in frequency F1. Hence, any coverage gap in frequency F1 would be identified by the network.

Legacy logged MDT measurements are sufficient for identifying coverage gaps in a given frequency

Since the network is aware of the planned slices on a given frequency, it is capable to identify that there are some places with unavailable slices in that frequency and UE needs to use legacy cell reselection criterion to find a suitable cell.

On the other hand, NSAG specific frequency priorities are cell specific parameters and are transmitted using SIB16. UE accesses SIB16 to read such information and utilize that to determine priorities during cell reselection. Since in legacy logged MDT, UE reports the current serving cell information, it is trivial for the network to identify if any slice specific cell reselection is configured and if the UE is following such configuration.

Legacy logged MDT measurements contains serving cell information of a given UE and network can identify if any slice specific cell reselection is configured in the cell. Hence, network can estimate which cell UE should re-select to continue to have access to a specific slice and whether the target cell provides sufficient coverage or not.

The above mechanism is schematically shown in the following figures.

Figure 1: UE camps on the serving frequency, according to SIB16 of the serving cell, finds F1 as highest priority frequency and attempt to find a suitable cell in F2 to camp on. The MDT report indicates that in F1 there is a full coverage over the current serving cell of the UE, so any slice specific cell reselection should be successful.

Figure 2: UE camps on the serving frequency, according to SIB16 of the serving cell, finds F1 as highest priority frequency and attempt to find a suitable cell in F2 to camp on. However, F1 does not provide the full coverage over the serving cell of the UE. Hence any slice specific cell reselection should fail in red area. This is already deducible from the current logged MDT report

Hence, in our view, using the legacy logged MDT measurements and information from SIB16, it is trivial for the network to create a slice coverage map and identify any gaps; additional information from the UE is not required.

Send LS reply to RAN3 mentioning the scenario identified in RAN3 can be supported using legacy logged MDT procedure, no further enhancements are needed.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Legacy logged MDT measurements are sufficient for identifying coverage gaps in a given frequency

Observation 2	Legacy logged MDT measurements contains serving cell information of a given UE and network can identify if any slice specific cell reselection is configured in the cell. Hence, network can estimate which cell UE should re-select to continue to have access to a specific slice and whether the target cell provides sufficient coverage or not.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The scenario described in the LS may occur only if there is a coverage hole in the frequencies corresponding to the highest ranked NSAG.

Proposal 2	Send LS reply to RAN3 mentioning the scenario identified in RAN3 can be supported using legacy logged MDT procedure, no further enhancements are needed.