3GPP TSG-RAN WG2 Meeting #129bis                                    	R2-2502655

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.9.2

Source:	Qualcomm Incorporated

Title:	Switching of S&F mode

Document for:	Discussion and Decision

- Introduction

In RAN2#129 meeting, following agreements were made.

Agreements:

- RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

- Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

In this document, we provide further details for solution of switching S&F mode and normal mode.

- Discussion

Figure 1 An example of switching of normal mode and S&F mode

Whenever the satellite establishes feeder link connection (feeder link becomes available), it is also possible that a cell may want to exit from the S&F mode operation (for example, switch the S&F mode to transparent architecture mode or only eNB onboard mode). In this case, the UEs supporting S&F mode operation may also support normal mode operation, in which case, these UEs may stay in the same satellite and may not need to take any action for searching another satellite (e.g., for cell reselection) due to the switch.

However, if cellBarred-NTN-r17 is changed or the S&F mode indication is changed or feeder link information needs to be updated, the existing procedure should follow to update the SIB1, which requires UE to take action on updating the system information. This requires network sending paging message adding signaling overhead.

There should be a time information provided when such switching occurs so that UE and application can make better decision on the use of traffic types, e.g., time sensing traffic can wait until the switch time. This may be indication that S&F indication in SIB31 or cellBarred-NTN-r17 in SIB1 may change or feeder link information may change. Following is agreed in RAN2#129.

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

Currently the t-Service (cell stop time) is provided in SIB3. The switching time of S&F mode to normal mode can be exactly same as t-Service time, for example, MME over satellite changes or feeder link info (e.g., common TA, Kmac) resulting in cell change. A Signalling optimization can be done by just providing an indication (i.e., 1 bit) instead of broadcasting TimeOffsetUTC switch time in SIB31 (i.e., 20 bits).

If t-Service is equal to switching time of S&F mode to normal mode, broadcast 1 bit indication instead of 20bits of TimeOffsetUTC.

Whenever the satellite loses feeder link connection, cell supporting S&F mode should make sure that S&F indication is included in SIB. In this case, the cell can release all UEs that do not support S&F mode operation in RRC_CONNECTED to IDLE mode. In this case, the cell may also be releasing the legacy UEs.

The SIB may need to be updated to provide updated information of feeder link or S&F mode indication and next switch time. We think it is very important to avoid paging notification to eMTC and NB-IoT UEs. The existing cell stop time mechanism can be used to inform the legacy UEs that they need to find other cells. This can be useful for both cases, i.e., when feeder link will be lost or when feeder link will be available.

For new UEs, a new indication (probably in SIB31) can be introduced to indicate that the cell stop time is associated with the update of S&F mode operation, i.e., at cell stop time, the cell loses feeder link and operates in S&F mode.

For switching time from normal mode to S&F mode, the existing cell stop time (t-Service) is reused.

For legacy UEs, the existing behavior of t-Service applies when normal mode is switched to S&F mode at t-Service.

For new UEs, additional indication is provided to associate the existing cell stop time with the switching time from normal mode to S&F mode.

Upon mode switch, discuss how to update the system information without paging if barring bit or S&F mode indication or presence of common TA is changed for new UEs.

As mentioned earlier, application may need to be aware of the switch time such that it can optimize the traffic when to start time sensitive and when to start delay tolerant services. NAS may also need to be aware for NAS message reattempt strategy. With the switch time information, we think there is no need for additional information such as new release cause.

UE’s AS layers forwards the switching time information to upper layers.

Cell selection/reselection

SA2 has sent response LS in [1] and has specified following in TS 23.401.

NOTE 6:	The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode.

NOTE 7:	How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

In our understanding, even if UE has a list of satellites provided, the UE can still choose to go to other satellites. This confirms the RAN2 understanding:

The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and

communicate with a satellite which is not included in the MME-configured satellite list.

For NB-IoT, cell reselection is based on the ranking of neighbor cells (i.e., frequency priorities are not considered). When the UE is in IDLE mode, it may find both S&F mode cells and normal mode cells. The UE may want to connect to the normal mode cell even if the ranking of S&F mode cell is better. When the UE is waiting for the satellite included in list, it may have UL data to transmit and there can be other satellites or TN cells available.

Therefore, we think if the UE has information on the normal mode cells, UE should be allowed to prioritize normal mode cell over S&F mode cell.

When connected to S&F mode cell, it can be left to UE in RRC_IDLE to periodically search for better and normal mode cell (i.e., cells with feeder link connection available).

When in RRC_IDLE and waiting to connect to a S&F mode satellite, it can be left to UE to prioritize the search for better and normal mode cell (i.e., cells with feeder link connection available).

- Conclusion

Following observation and proposals are made:

Proposal 1	If t-Service is equal to switching time of S&F mode to normal mode, broadcast 1 bit indication instead of 20bits of TimeOffsetUTC.

Proposal 2	For switching time from normal mode to S&F mode, the existing cell stop time (t-Service) is reused.

Proposal 3	For legacy UEs, the existing behavior of t-Service applies when normal mode is switched to S&F mode at t-Service.

Proposal 4	For new UEs, additional indication is provided to associate the existing cell stop time with the switching time from normal mode to S&F mode.

Proposal 5	Upon mode switch, discuss how to update the system information without paging if barring bit or S&F mode indication or presence of common TA is changed for new UEs.

Proposal 6	UE’s AS layers forwards the switching time information to upper layers.

Proposal 7	When connected to S&F mode cell, it can be left to UE in RRC_IDLE to periodically search for better and normal mode cell (i.e., cells with feeder link connection available).

Proposal 8	When in RRC_IDLE and waiting to connect to a S&F mode satellite, it can be left to UE to prioritize the search for better and normal mode cell (i.e., cells with feeder link connection available).

- Reference

[1] R2-2501753, “Reply LS on Satellite IDs for store-and-forward operation”, From SA2, To RAN2