3GPP TSG-RAN WG2 #129bis	R2-2502662

# Athens, Greece, 17th – 21th February 2024

Agenda Item:	 8.1.2.3

Source:	Ericsson

Title:	LCM For Positioning

Document for:	Discussion

# 1	Introduction

The following agreements on the topic of LCM for the positioning Case 1 were made in RAN2#129.

Agreements

1:	Introduce AI/ML positioning Case 1 as a new positioning method.

2:	Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR.

3:	FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

4: 	The content of error cause is discussed while drafting stage3 CRs.

5: 	As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

6: 	UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

In this paper we discuss the below FFS proposal from RAN2#129

3: FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation

# 2 Fallback/Switching

The term Fallback was defined as fallback from AI/ML positioning method to a non-AI/ML based positioning method whereas switching was defined to switch from non-AI/ML based positioning method to AI/ML positioning method. In essence fallback is trying the second preferred option when the first option does not work out and switching is going back to first preferred option.

Irrespective of the term that is used (i.e. fallback or switching), we provide the below analysis that for both cases there should be no specification impact especially for a baseline functionality for Case 1 for Rel-19, when multiple positioning methods are considered.

Typically, multiple methods are used for the case of hybrid positioning which are complementing each other; example two methods such as 1) GNSS and 2) sensor-based positioning methods can be used as hybrid positioning method. This is generally useful for the case when the UE moves into a tunnel where the UE may lose the GPS signal and simply use the sensory information to compute the relative location from the last known GPS signal. Similarly, the NW may use multiple UE assisted positioning methods such as Time difference of arrival and angle-based arrival method to fuse these measurements in order to improve positioning accuracy.

For case 1 (i.e. where the UE reports the location using AI/ML positioning method); the LMF knows if the AI/ML based, or a non-AI/ML-based positioning method should be selected for example considering the area the UE is in and based on the UE capabilities. That is, LMF would be aware which method works well in a certain area and select the method accordingly. We do not see that AI/ML and non-AI/ML methods are complementing each other nor should they be considered as a hybrid positioning method.

LMF should be aware whether AI/ML based method works in a cell where UE is in and thus there would be no need for multiple methods that should be provided. For case 1, AI/ML based method and non-AI/ML based method are not hybrid or complementary methods. Moreover, it is an overhead to provide Assistance data for two methods.

RAN2 does not to consider configuration of multiple methods for case 1 AI/ML and leave it to NW implementation.

Currently, there is no specified UE behaviour described for the case when LMF configures multiple positioning methods to the UE. Instead, the UE by implementation may handle both in parallel or chooses to perform them in sequence. That is, how the UE handles those methods are left to UE implementation.

Hence, even if multiple methods are provided to the UE, there should be no specification impact.

LMF may choose to provide multiple methods, which is a NW implementation. Besides, it is also up to UE implementation on how to handle the multiple methods; i.e if UE wants to perform measurements in parallel or in sequence etc.

In that regard, we consider that RAN2 does not need to pursue the below Agreement:

3: FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation

RAN2 does not specify UE autonomous fallback or switching procedures between AIML-/non-AIML-positioning methods but it is left to UE implementation how to fallback/switch when multiple methods are configured by LMF.

## Training Data Collection for Case 1

### 3.1 Using Legacy Procedure for AD request for training

As per RAN1 agreements (Only keeping option for Case 1):

Agreement For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

The training data collection for Case 1, which only requires PRS configuration, is similar to existing procedure where UE requests assistance data of PRS configuration for legacy positioning measurements. Thus legacy procedure/ways for UE to request PRS configuration including UE-initiated assistance data request and UE-initiated on-demand PRS procedure can be reused. The training data collection for Case 1 is part of procedure for AI/ML positioning, and the PRS configuration request for this should not be either prioritised or deprioritized compared with legacy positioning, thus UE does not include the purpose of request when sending request for PRS configuration.

For data collection for AI/ML models on the UE side, the UE may request assistance data to LMF using UE-initiated assistance data request procedure as specified in clause 8.12.3.1.2.2 of TS 38.305 or UE-initiated on-demand PRS procedure as specified in clause 7.6.2 of TS 38.305 without the need to include the purpose of request.

Legacy procedure can be reused for data collection for case1.

### 3.2 Generating Label for data collection and associated parameters

RAN1 Agreements Working Assumption For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope.

AI/ML is used in area where there can be NLOS. Further, the PRS symbols may not be transmitted with perfect sync and there may be Real Time Difference. The LMF should be able to guide what is the expected positioning error during training phase to the UE based upon related Integrity parameters (e.g., standard deviation of RTD) so that the UE can understand if the obtained positioning can be used as a label (ground truth) or should be discarded. Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for AI/ML training may provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for the case of AI/ML training can provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

LMF provides the guidance for trustworthy label based upon Positioning Integrity (maximum allowed Positioning error or uncertainty for a trustworthy label)

# Conclusion

In the previous sections we made the following observations:

Observation 1	LMF should be aware whether AI/ML based method works in a cell where UE is in and thus there would be no need for multiple methods that should be provided. For case 1, AI/ML based method and non-AI/ML based method are not hybrid or complementary methods. Moreover, it is an overhead to provide Assistance data for two methods.

Observation 2	LMF may choose to provide multiple methods, which is a NW implementation. Besides, it is also up to UE implementation on how to handle the multiple methods; i.e if UE wants to perform measurements in parallel or in sequence etc.

Observation 3	For data collection for AI/ML models on the UE side, the UE may request assistance data to LMF using UE-initiated assistance data request procedure as specified in clause 8.12.3.1.2.2 of TS 38.305 or UE-initiated on-demand PRS procedure as specified in clause 7.6.2 of TS 38.305 without the need to include the purpose of request.

Observation 4	Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for the case of AI/ML training can provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 does not to consider configuration of multiple methods for case 1 AI/ML and leave it to NW implementation.

Proposal 2	RAN2 does not specify UE autonomous fallback or switching procedures between AIML-/non-AIML-positioning methods but it is left to UE implementation how to fallback/switch when multiple methods are configured by LMF.

Proposal 3	Legacy procedure can be reused for data collection for case1.

Proposal 4	LMF provides the guidance for trustworthy label based upon Positioning Integrity (maximum allowed Positioning error or uncertainty for a trustworthy label)

# References

TS 38.305, Stage2 for Positioning