3GPP TSG-RAN WG2 Meeting # 129bis                	R2-2502672

Wuhan, China, April 7th – 11th, 2025

Source:	Philips International B.V., NEC

Title:	Discussion on RAN impact of UE usage of RAT restrictions

Agenda Item:	8.18

Document for:	Discussion and Decision

# Introduction

At RAN2 #129, RAN2 discussed the LS sent by CT1 to RAN2 in R2-2500004 [1] asking RAN2 to investigate the impact of the CT1 work item ECRATU introducing an optional feature for the network to indicate RAT utilization restriction information to the UE with EMM and 5GMM procedures.

CT1 is introducing an optional feature for the network to indicate RAT utilization restriction information to the UE with EMM and 5GMM procedures.

[…]

The following access technologies can be restricted: GERAN, UTRAN, E-UTRAN, satellite E-UTRAN, NG-RAN, satellite NG-RAN. E-UTRAN access technology encompasses all terrestrial E-UTRAN cells. NG-RAN access technology encompasses all terrestrial NG-RAN cells.

RAN2 identified a couple of AS layer spec impacts for the IDLE/INACTIVE mode and endorsed the CRs in R2-2501531, R2-1501532, R2-2500438 and R2-2500967.

In this contribution, we would like to further discuss the CONNECTED mode RAN impact for the E-UTRAN of UE usage of RAT restrictions.

# Discussion

In [2], it is discussed the CONNECTED mode impact of RAT restrictions. TS 38.413 is referenced to show NGAP protocol already supports signalling of transferring RAT restrictions information from AMF to NG-RAN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Primary RAT Restriction | M |  | BIT STRING { e-UTRA (0), nR (1), nR-unlicensed (2), nR-LEO (3), nR-MEO (4), nR-GEO (5), nR-OTHERSAT (6), e-UTRA-LEO (7), e-UTRA-MEO (8), e-UTRA-GEO (9), e-UTRA-OTHERSAT (10)} (SIZE(8, …, 16)) | Each position in the bitmap represents a Primary RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE. Bits 11-15 reserved for future use. The Primary RAT is the RAT used in the access cell, or target cell. |

Observation 1: The RAT restrictions information for CONNECTED mode is already handled by the NGAP protocol from AMF to NG-RAN.

However, we understand the above observation only applies to NG-RAN, and it is necessary to also check whether the same observation applies to E-UTRAN.

According to TS 36.413 V18.3.0, the following RAT Restriction Information has been defined for S1AP protocol for signalling RAT restriction information from MME to E-UTRAN.

#### 9.2.1.22 Handover Restriction List This IE defines roaming or access restrictions for subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE, e.g., handover and CCO, or for SCG selection during dual connectivity operation. If the eNB receives the Handover Restriction List IE, it shall overwrite previously received restriction information. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. <skipped text>. RAT Restrictions; 0..<maxnoofEPLMNsPlusOne>; This IE contains RAT restriction related information as specified in TS 23.401 [11].; YES; ignore. >PLMN Identity; M; 9.2.3.8; -. >RAT Restriction Information; M; BIT STRING { LEO (0), MEO (1), GEO (2), OTHERSAT (3), NR-LEO (4), NR-MEO (5), NR-GEO (6), NR-OTHERSAT (7)} (SIZE(8, …)); Each position in the bitmap represents a RAT. If a bit is set to "1", the respective RAT is restricted for the UE. If a bit is set to "0", the respective RAT is not restricted for the UE.; -.

From the definition of RAT Restriction Information IE, we understand only RAT restriction related to satellite NG-RAN is signalled, and RAT restriction related to NG-RAN is not signalled.

Observation 2: The current S1AP protocol only signals the RAT restriction information for satellite NG-RAN, and RAT restriction information for NG-RAN is not signalled.

According to TS 36.331 V18.5.0, inter-RAT HO is supported from E-UTRA to NR in CONNECTED mode.

### 5.4.3 Mobility from E-UTRA #### 5.4.3.1 General The purpose of this procedure is to move a UE in RRC_CONNECTED to a cell using another Radio Access Technology (RAT), e.g. GERAN, UTRA, CDMA2000 systems, NR, or handover a UE to an E-UTRA cell connected to another type of CN.

Observation 3: Inter-RAT HO is supported from E-UTRA to NR in CONNECTED mode.

From Observation 2, we understand E-UTRAN does not have the RAT restriction information for NG-RAN as it is not signalled from MME by S1AP protocol. As a result, E-UTRAN may still make the decision to handover to NR even if NG-RAN is restricted according to the NAS layer “PLMNs with associated RAT restrictions”, then the handover will fail on the UE side when UE checks that NG-RAN is restricted by NAS layer “PLMNs with associated RAT restrictions”. Therefore, there is AS layer spec impact because of this new failure condition.

Observation 4: AS layer spec impact exists for E-UTRAN in CONNECTED mode HO due to NG-RAN restriction information not being signalled by S1AP protocol.

In order to fully support the CONNECTED mode mobility from E-UTRA to NR, RAN2 is kindly requested to consider the following proposal.

Proposal 1: RAN2 is kindly requested to further discuss the CONNECTED mode mobility AS layer impact of the new CT1 work item ECRATU and decide how to proceed.

# Conclusion

This contribution elaborates on the RAN impact of UE usage of RAT restrictions.

A summary of the observations and proposal can be found below:

Observation 1: The RAT restrictions information for CONNECTED mode is already handled by the NGAP protocol from AMF to NG-RAN.

Observation 2: The current S1AP protocol only signals the RAT restriction information for satellite NG-RAN, and RAT restriction information for NG-RAN is not signalled.

Observation 3: Inter-RAT HO is supported from E-UTRA to NR in CONNECTED mode.

Observation 4: AS layer spec impact exists for E-UTRAN in CONNECTED mode HO due to NG-RAN restriction information not being signalled by S1AP protocol.

Proposal 1: RAN2 is kindly requested to further discuss the CONNECTED mode mobility AS layer impact of the new CT1 work item and decide how to proceed.

# References

- R2-2500004, UE usage of the RAT restrictions (C1-247193; contact: Apple), CT1, 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, February 17-21, 2025

- R2-2500112, Discussion on RAT Restriction, OPPO, 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, February 17-21, 2025