3GPP TSG-RAN WG2 #129bis	R2-2502677

# Wuhan, China, 7th – 11th April 2025

Agenda Item:	8.8.4

Source:	Ericsson

Title:	Support for broadcast services in NR NTN

Document for:	Discussion, Decision

# 1	Introduction

The third WID on NTN for NR was agreed in [1], which includes an objective on specifying signalling for broadcast services, as follows:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signalling between CN and NG-RAN. [RAN3]

So far, RAN2 has reached several agreements regarding the shape/format of the intended service area as well as some of the related UE behaviours when located inside and outside the intended service area (ISA). However, signalling means as well as service continuity are still under discussion. This contribution presents our views on the open issues and on supporting PWS in NTN cells.

# 2	MBS service continuity in NTN

#### 2.1	Background

MBS Service continuity for UEs in RRC_IDLE or RRC_INACTIVE was enhanced with the following mechanisms to reduce service interruption and user experience degradation (section 16.10.6.5 in TS 38.300):

- A list of neighbour cells providing the one or more MBS broadcast services as the serving cell. It is informed with a list provided in MCCH (MBSBroadcastConfiguration IE) through the parameter mbs-NeighbourCellList.

- Information about which frequency is providing which MBS broadcast services. It is informed through User Service Description (USD) and SIB21. Specifically, SIB21 contains MBS Frequency Selection Area ID (FSAI) which identifies a preconfigured area within, and in proximity to, which the cell(s) announces the FSAI and the associating frequency.

The first mechanism allows a UE to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using point-to-multipoint (PTM) transmission. The second mechanism helps the UE avoid the need to read MBS broadcast related system information and potentially MCCH on neighbour frequencies. The UE can figure out the TMGI-Frequency mapping from USD/Service Announcement or from the combination of USD/Service Announcement and SIB21. In addition, the UE is allowed to make the desired frequency highest priority in cell reselection. From the network perspective, the mapping between frequencies and FSAI is fixed and established through OAM.

#### 2.2	NTN specific changes

In previous RAN2 meetings, there has been an increased interest in enhancing service continuity considering the concept of Intended Service Area (ISA), which is specific to NTN. In our view, the introduction of ISA does not entail any necessary changes to legacy mechanisms for service continuity in MBS broadcast when it comes to UE originated mobility. That means, the list of neighbour cells providing the same MBS broadcast services (now with the associated ISA) in MCCH, the FSAI in SIB21, and the TMGI-frequency mapping or TMG-SAI mapping in the USD (which is delivered to the UE via Service announcement) can be used in NTN deployments.

- Legacy (TN) mechanisms for service continuity due to UE mobility in MBS Broadcast can be reused in NTN.

RAN2 has agreed that the ISA(s) may expand beyond the coverage of a cell and overlap neighbour cells, i.e., they are not tied to cell boundaries but represent geographical locations. Therefore, an ISA may not only cover the serving cell but also neighbouring cells. In addition, RAN2 concluded that there are no requirements for the UE to start or release MBS Radio Bearers (MRB) upon entering or leaving the ISA. Therefore, when the UE moves between cells providing the same MBS service, we think it can simply perform frequency prioritization and cell reselection following legacy mechanisms. Once the UE has finished cell reselection, it may acquire the new SIB and MCCH to re-evaluate whether it is still located within the ISA(s) of the MBS service it is interested in or it is currently receiving.

- A UE does not need to consider the ISA to reselect to a different cell and continue receiving an MBS service.

A few companies have also proposed to associate FSAI in SIB21 with the ISAs defined in the new SIB. We understand this could be useful when a UE camping in a cell wants to prioritize between different frequencies to receive a certain MBS service. However, from network perspective, FSAI is a static concept which is configured via OAM and pre-defined for certain MBS service and a specific deployment (e.g., an event in a large stadium). In contrast, the ISA may be originated from the MBS application function (AF). Therefore, it is not straightforward how ISAs can be mapped into FSAIs and, clearly, such mapping has impact in other WGs since it would lie outside RAN2’s scope.

- The mapping between FSAI and ISAs introduces complexity in the network and has potential impact on core network specifications.

In summary, for UE initiated mobility events, there is an increased complexity and signalling overhead without a clear benefit when the UE needs to consider the ISA. Thus, we consider that a UE can continue obtaining an MBS service upon cell change using existing service continuity mechanisms.

MBS service continuity enhancements for mobility due to UE movement between NTN cells are down prioritized in Rel-19.

We see the need to address service continuity in NTN originated due to satellite movement. Specifically, one aspect where NTN differs from terrestrial networks (TN) is that the time duration that an NGSO satellite serves a quasi-earth-fixed cell is just a few minutes. Each new serving satellite typically implies a new cell, therefore, an MBS UE in RRC_IDLE or RRC_INACTIVE needs to perform cell reselection and potentially acquire MBS related configuration (e.g., MCCH and new SIB) quite often. This can be a challenge in terms of signalling overhead and service interruption since it is difficult from network perspective to align satellite switches with the MCCH modification period, and there is a risk the UE does not obtain the MCCH update in time, leading to service interruption. Therefore, we suggest RAN2 to discuss this issue and find potential solutions.

- The continuous switch of satellites/cells in an NTN deployment may lead to both increased signalling overhead and service interruption in MBS.

RAN2 focuses on service continuity enhancements required due to satellite movement.

A potential solution to the foregoing problem is to avoid the need to reacquire MCCH upon cell reselection unless the contents of MCCH are updated. For instance, the gNB could apply the same MTCH configuration in a geographic area no matter which satellite/cell serves that area, in this way the MTCH configuration does not change due to satellite movement. Similar to the list of neighbour cells (mbs-NeighbourCellList IE), the network can also inform the UE of which upcoming satellites/cells provide the same MBS services as the serving cell. In this way the neighbour cell list could be updated less frequently, i.e., not necessarily upon every satellite/cell switch. Such improvements mitigate the need of reacquiring MCCH upon every satellite/cell switch which decreases service interruption and power consumption.

In a quasi-Earth fixed cell, the network provides information about the MBS broadcast configuration and MBS services provided by the upcoming satellite(s) that will serve the area.

RAN2 has agreed that the legacy SIB modification procedure is applied to update the intended service area (ISA) definition in the new SIB. In Earth-moving-cells, the ISAs being served by the cell will frequently change with the cell movement, potentially leading to frequent system information updates. Even if a UE does not move, it may need to unnecessarily re-acquire the new SIB when a new ISA is added as the cell moves. In our view, there are two potential approaches to solve the issue:

- The new SIB may include ISA definitions that exceed the current cell borders, i.e., the network includes ISA that the satellite will cover in the future. A similar mechanism was adopted in Rel-18 to the TN area definitions to optimize UE measurements. This option would not have specification impact.

- The ValueTag of the new SIB does not change when modifying the list of ISA definitions for Earth-moving cells.

Each option has its drawbacks. The former approach may increase the signalling overhead, while the latter introduces different behaviours between (quasi)-Earth-fixed cells and Earth-moving cells. Thus, we kindly ask RAN2 to discuss the issue and consider the above two alternatives, potentially striving for a single solution that suits both types of deployments.

RAN2 discusses the following two alternatives to mitigate the Earth-moving cell issue with the frequency of SIB updates:

- The new SIB includes ISAs of MBS services that the cell will cover in the future.

- The ValueTag of the new SIB does not change.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	Legacy (TN) mechanisms for service continuity due to UE mobility in MBS Broadcast can be reused in NTN.

Observation 2	A UE does not need to consider the ISA to reselect to a different cell and continue receiving an MBS service.

Observation 3	The mapping between FSAI and ISAs introduces complexity in the network and has potential impact on core network specifications.

Observation 4	The continuous switch of satellites/cells in an NTN deployment may lead to both increased signalling overhead and service interruption in MBS.

Based on the discussion in the previous sections we propose the following:

Proposal 1	MBS service continuity enhancements for mobility due to UE movement between NTN cells are down prioritized in Rel-19.

Proposal 2	RAN2 focuses on service continuity enhancements required due to satellite movement.

Proposal 3	In a quasi-Earth fixed cell, the network provides information about the MBS broadcast configuration and MBS services provided by the upcoming satellite(s) that will serve the area.

Proposal 4	RAN2 discusses the following two alternatives to mitigate the Earth-moving cell issue with the frequency of SIB updates:

	The new SIB includes ISAs of MBS services that the cell will cover in the future.

	The ValueTag of the new SIB does not change.

# 5	References

RP-240775, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3” 3GPP TSG RAN#103, Mar. 2024.