3GPP TSG RAN WG2 Meeting #129bis	R2-2502703

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.17

Source:	CMCC

Title:	Support of IoT-NTN TDD mode

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of IoT-NTN TDD mode and some agreements were achieved as follows [1]:

Agreements:

- RAN2 will continue studying paging aspects based, on RAN1 progress

- RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed.

- RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network.

- RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated.

- RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system.

- RAN2 can continue the discussion also on RAR window

- Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

According to RAN1 discussion progress, the latest agreements are reached as following[2]:

Agreement The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be [48…50] ms at the ULSRP - NOTE: This corresponds to the largest separation between DL and UL Iridium slot pairs. Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA Agreement The downlink subframes within D=8 are fixed to: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) Agreement NPSS/NSSS/NPBCH transmissions are dropped in non-D NB-IoT subframes - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - FFS: handling for SIB1-NB Agreement The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications - NPUSCH is mapped as per current specifications. - NOTE: “non-U NB-IoT subframes” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the TDD structure. Agreement The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications - NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications. - FFS: additional NB-IoT TDD-specific rules for handling non “NB-IoT DL subframes” - FFS: Introduction of new periodicities for NPDCCH monitoring (e.g. by the introduction of scaling factor) - FFS: whether all the configuration values for NPDCCH are valid for NB-IoT NTN TDD. - FFS: handling of DL transmission gap - NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. Conclusion For determining the offset between the 90ms TDD structure and the 10240ms H-SFN: the UE may derive the 90ms TDD structure based on observations of downlink signals. No specification impact.

In this contribution, we would like to provide some further considerations on support of IoT-NTN TDD mode.

Discussion

User plane impact

  - 2.1.1 RA

In last RAN2 meeting, we assumed that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system. And the existing NPDCCH repetition range is {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1}. The current RAR window size for NB-IoT is provide as following:

ra-ResponseWindowSize-r13			ENUMERATED {

pp2, pp3, pp4, pp5, pp6, pp7, pp8, pp10},

ra-ResponseWindowSize Duration of the RA response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods and so on. For FDD: The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s). For TDD: The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 20.48s).

Based on network implementation, the existing RAR window value range could cover the new TDD pattern, and so does mac-ContentionResolutionTimer similarly.

Proposal 1: The existing ra-ResponseWindowSize and mac-ContentionResolutionTimer value range could cover the new TDD pattern based on network configuration.

  - 2.1.2 HARQ

And for HARQ of NB-IoT NTN FDD, in R18, we supported DL HARQ feedback disabled to support long propagation delay based on network configuration. For NB-IoT NTN TDD, considering the characteristic of the new TDD structure, for one HARQ process, the interval between a new transmission and a retransmission or two retransmission would be too long. That is a feedback used to determine whether a retransmission is needed may be not important. And we could consider an solution (e.g. add related description in TS 36.300) that if the NB-IoT NTN system operates in TDD mode, the HARQ feedback is disabled as default.

Proposal 2: It is proposed that if the NB-IoT NTN system operates in TDD mode, the HARQ feedback is disabled by default, and to be simple, we could just add related description in TS 36.300.

Control plane impact

  - 2.2.1 Cell bar

We agreed to reuse the legacy barring bit and FFS cellBarred or cellBarred-NTN. According to the WID scope, the new TDD pattern operates in 1616-1626.5 MHz MSS allocated band, and TN deployment is not expected in this band. Then legacy NTN UE operating band in FR1 is shown in the following:

Table 5.2-1: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 MHz – 1626.5 MHz | 2483.5 MHz – 2500 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

It can be seen that, there is only some overlap between the FDD UL operating band n254 and the 1616-1626.5 MHz MSS allocated band, which means for DL, legacy NTN UE would not search on this band. That is to say both cellBarred or cellBarred-NTN could be used for access control for the R19 NB-IoT NTN UE supporting the new TDD pattern, there is no confusion.

Proposal 3: Both cellBarred or cellBarred-NTN could be used for access control for the R19 NB-IoT NTN UE supporting the new TDD pattern.

  - 2.2.2 System information scheduling

Based on the scope, there are only 8 consecutive DL subframes in the TDD pattern, then SIB1-NB only could be transmitted at most once in one TDD pattern period, which could not even support the repetition mechanism against the poor coverage that some IoT devices deployed. Then for other SIB-NB information scheduled by SIB1-NB, the corresponding SI window may be also affected. As in the current TS 36.331, for NB-IoT UE, the related SI window determination is as following:

- 5.2.3a Acquisition of an SI message by BL UE or UE in CE or a NB-IoT UE When acquiring an SI message, the BL UE or UE in CE or NB-IoT UE shall: - determine the start of the SI-window for the concerned SI message as follows: 2> if the concerned SI message is configured in the schedulingInfoList, schedulingInfoListExt (if present) or if the concerned SI message is configured in the posSchedulingInfoList and si-posOffset is not configured; 3> for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list of SI messages configured by schedulingInfoList, schedulingInfoListExt (if present) in SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and posSchedulingInfoList in SystemInformationBlockType1-BR; 3> determine the integer value x = (n – 1)*w, where w is the si-WindowLength-BR (or si-WindowLength in NB-IoT); 3> if the UE is a NB-IoT UE: 4> the SI-window starts at the subframe #0 in the radio frame for which (H-SFN * 1024 + SFN) mod T = FLOOR(x/10) + Offset, where T is the si-Periodicity of the concerned SI message and, Offset is the offset of the start of the SI-Window (si-RadioFrameOffset); - <<omitted>>

However, for NB-IoT TDD mode, the start calculation of SI-window needs to take the new TDD pattern into account. For example, the input parameter x affected by the si-WindowLength whose value range may not enough for the new TDD pattern. And based on the agreement of RAN1 #119, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods (Iridium TDMA structure is shown in figure 1), which would lead to that the actual calculated subframe #0 may not be used. Therefore we need to discuss the impacts for SI acquisition based on the new TDD pattern.

Figure 1 Legacy system with TDD pattern deployed in the 1616-1626.5 MHz band

Proposal 4: SI acquisition needs to consider the new TDD pattern.

  - 2.2.3 Paging

Per the existing TS 36.304, one Paging Frame (PF) is one Radio Frame, which may contain one or multiple Paging Occasion(s). PO, PF are determined by following formula:

PF is given by following equation: SFN mod T= (T div N)*(UE_ID mod N) Index i_s pointing to PO from subframe pattern defined in 7.2 will be derived from following calculation: i_s = floor(UE_ID/N) mod Ns Parameters used for the calculation of the PF, i_s: - T: DRX cycle of the UE. - nB: 4T, 2T, T, T/2, T/4, T/8, T/16, T/32, T/64, T/128, and T/256, and for NB-IoT also T/512, and T/1024. - N: min(T,nB) - Ns: max(1,nB/T)

The available paging occasion location is provided in 7.2, subframe 9, {4,9} , {0,4,5,9} for Ns=1,2,4 respectively. Based on the last RAN1 meeting agreement, the downlink subframes within D=8 are fixed to [3 4 5 6 7 8 9 0] (across two consecutive radio frames). Then considering the actual available DL subframes, some calculated PO may fall into inactive DL subframes, which could be postponed directly to next available PO.

Observation 1: The DL subframes of the new TDD pattern could cover the existing paging occasion located subframes.

Proposal 4: Enhancements for PO may be not needed, and it could be postponed directly to next actual available PO if the calculated PO falls into inactive DL subframes.

  - 2.2.4 Measurement Gap

Based on the current TS 36.133, in E-UTRA-NR dual connectivity mode, NR - E-UTRA dual connectivity mode and E-UTRA standalone mode, all gap patterns #0~11 in Table 8.1.2.1-1 can be configured  for measurements of NR carriers only, and gap pattern#0, 1, 2, 3, 4, 6, 7, 8, 10 can be configured for measurements of E-UTRA carriers with the applicability as specified in Table 8.1.2.1-1.

Table 8.1.2.1-1: Gap Pattern Configurations supported by the UE

| Gap Pattern Id | MeasurementGap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) | Minimum available time for inter-frequency and inter-RAT measurements during 480ms period (Tinter1, ms) | Measurement Purpose |
| --- | --- | --- | --- | --- |
| 0 | 6 | 40 | 60 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 1 | 6 | 80 | 30 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 2 | 3 | 40 | 24NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference as specified below. inter-RAT NR |
| 3 | 3 | 80 | 12NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference according as specified below. inter-RAT NR |
| 4 | 6 | 20 | 120 Note 1 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 5 | 6 | 160 | Note 3 | inter-RAT NR |
| 6 | 4 | 20 | 72 Note 1, 5, 7 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 7 | 4 | 40 | 36 Note 1, 5, 8 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 8 | 4 | 80 | 18Note 1, 5, 9 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 9 | 4 | 160 | Note 3 | inter-RAT NR |
| 10 | 3 | 20 | 48 Note 1, 5 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD Note 6 |
| 11 | 3 | 160 | Note 3 | inter-RAT NR |
| 24 | 10 | 80 | 54Note 1 | Inter-Frequency E-UTRAN FDD and TDD Note 11 |
| 25 | 20 | 160 | Note 3 | Note 11 |
| NOTE 1: When determing UE requirements using Tinter1 for GP2, 3, 4, 6, 7, 8, 10, Tinter1 = 60 for GP2, GP4, GP6, GP7, GP10 and Tinter1 = 30 for GP3, GP8 and GP24 shall be used. NOTE 2: Void. NOTE 3: This gap pattern can be used only for measurement of NR carriers, and Tinter is not applicable. NOTE 4: Void NOTE 5: Void. NOTE 6: This gap pattern is supported by UEs which are configured to perform both E-UTRA inter-frequency measurement and inter-RAT NR measurement or supported by UEs configured to perform inter-RAT NR measurement only. NOTE 7: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 48ms corresponding to the first 3ms of the 4ms gap NOTE 8: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 24ms corresponding to the first 3ms of the 4ms gap NOTE 9: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 12ms corresponding to the first 3ms of the 4ms gap NOTE 10: For UE only supporting measGapPatterns-NRonly-r16 for the corresponding gap patterns among GP2-11, the corresponding gap patterns are only applicable to measurement of inter-RAT NR. NOTE 11: Measurement gap patterns #24 and #25 can be requested by NR PCell [38] only when the UE is configured at least with any of RSTD, UE Rx-Tx, or PRS-RSRP measurements requiring such gaps and can only be used during the corresponding positioning measurement period |  |  |  |  |

From our perspective, the parameters MeasurementGap Length and measurement gap repetition period would be affected by the available DL subframes. And based on the current measurement gap configuration, the value range of MeasurementGap Length may be suitable to the new TDD pattern based on network configuration. While for the measurement gap repetition period, some new values could be considered based on the available DL subframes in one new TDD pattern period, for example, 90ms (i.e. corresponding to the period of the new TDD pattern) and multiples of 90ms. Therefore, measurement gap configuration enhancements could be discussed to match the new TDD pattern.

Proposal 5: It is proposed that measurement gap configuration enhancements (e.g introduction of new measurement gap repetition period values) could be discussed to match the new TDD pattern.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on IoT-NTN TDD mode and have the following proposals:

Proposal 1: The existing ra-ResponseWindowSize and mac-ContentionResolutionTimer value range could cover the new TDD pattern based on network configuration.

Proposal 2: It is proposed that if the NB-IoT NTN system operates in TDD mode, the HARQ feedback is disabled by default, and to be simple, we could just add related description in TS 36.300.

Proposal 3: Both cellBarred or cellBarred-NTN could be used for access control for the R19 NB-IoT NTN UE supporting the new TDD pattern.

Observation 1: The DL subframes of the new TDD pattern could cover the existing paging occasion located subframes.

Proposal 4: Enhancements for PO may be not needed, and it could be postponed directly to next actual available PO if the calculated PO falls into inactive DL subframes.

Proposal 5: It is proposed that measurement gap configuration enhancements (e.g introduction of new measurement gap repetition period values) could be discussed to match the new TDD pattern.

Reference

- R2-129 NR-NTN-IoT-NTN (Sergio)_ EOM

- Chair notes RAN1#120 eom0

- 3GPP TS 36.300 V18.4.0 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 18)

- 3GPP TS 36.331 V18.4.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 18)

- 3GPP TS 36.304 V18.3.0 Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode (Release 18)

- 3GPP TS 36.133 V18.8.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 18)