3GPP TSG RAN WG2 Meeting #129bis	R2-2502705

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.2.2

Source:	CMCC

Title:	Discussion on A-IoT paging

Document for:	Discussion

Introduction

In RAN2#129 meeting [1], the following agreements have been made:

Agreements 1.Parallel service requests by the same reader is not supported. 2.The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). 3.The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID 4.1 bit solution is excluded. FFS the size. Aim to have a reasonable size. Agreements on paging ID 1.The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. 2.The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer Agreements 1.The A-IoT paging message can include a number of msg1 resources 2.From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

Then in this contribution, we would like to provide our views on A-IoT paging with Topology 1.

Discussion

## 2.1 General analysis

### 2.1.1 Multi-reader scenario

In RAN2#129 meeting, the following agreement are made for multi-reader scenario [1]:

RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it.

However, within the post-email discussion, some companies have the concern that it may not be sufficient to mitigate the impact of multi-reader scenario and thinks that reader ID is also needed in A-IoT paging message. From our point of view, the most important impact brought by multi-reader scenario is interference. To be specific, when multi-reader scenario occurs, the devices located in overlapping areas covered by different readers may simultaneously receive R2D messages from different readers, thus can not decode any R2D message due to the low SINR. For this situation, introducing reader ID will not bring any benefits. Therefore, we prefer to avoid the occurrence of multi-reader scenarios or mitigating the impact as much as possible through following implementation methods:

- In R19, only D1T1-B where CW node is out of the Topology 1 is supported. So it is very likely that when one reader transmits R2D messages and receives D2R messages, adjacent readers transmit CW waves in actual deployment scenarios. Therefore, multi-reader scenario may also be naturally avoided as much as possible in practical scenarios.

- For Topology 1, each TRP acts as one reader as discussed in RAN3. To this end, it can be achieved through gNB implementation to control the transmission power of each TRP, thereby avoiding overlapping coverage between adjacent readers. With this, the device(s) is less likely to receive the another (different) service request from different reader.

Observation 1: The most important impact brought by multi-reader scenario is interference which can not be avoided/mitigated by introducing reader ID.

Observation 2: There are some implementation methods to avoid the occurrence of multi-reader scenarios or mitigating its impact as much as possible.

Proposal 1: In R19, reader ID is not needed for multi-reader scenario.

### 2.1.2 Paging identifier

#### 2.1.2.1 Relationship between paging identifier and temporary ID

In SA3, the temporary ID is agreed to protect A-IoT device ID and it has also been discussed in SA2 for the control of temporary ID. Therefore, from our point of view, the identifier received from CN and the paging identifier contained in A-IoT paging message can be a temporary ID. Whether the paging identifier can only be the temporary ID needs further input from SA2/SA3.

Proposal 2: The paging identifier contained in A-IoT paging message can be a temporary ID. Whether the paging identifier can only be the temporary ID needs further input from SA2/SA3.

#### 2.1.2.2 Visibility of paging identifier to A-IoT MAC layer

In RAN2#129 meeting, it has been agreed that [1]:

2.The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer

In previous meeting, some companies think that the paging identifier may need to be visible to A-IoT MAC layer in gNB-reader so that sub-grouping scheme can be performed. From our perspective, we think that it is an optimization scheme for the A-IoT procedure and totally transparent to A-IoT devices. Besides, we do not find much benefits of this scheme.

Apart from this, since AS ID has also been discussed to be used at least for the purpose of D2R scheduling and R2D reception, then it seems that the device identifier transmitted from CN, which is denoted as gNB-CN device ID here, should be visible to at least the A-IoT MAC layer in gNB-reader, so that the gNB-reader can map it to the corresponding AS ID or vice versa. We think that whether gNB-CN device ID is needed and the relationship between gNB-CN device ID and paging identifier have not determined by RAN3/SA2, so RAN2 has not seen the clear necessity for the paging identifier to be visible to the A-IoT MAC layer, up to now.

Observation 3: Sub-grouping performed by the reader is an optimization scheme for the A-IoT procedure and totally transparent to A-IoT device.

Observation 4: Whether gNB-CN device ID is needed and the relationship between gNB-CN device ID and paging identifier have not determined by RAN3/SA2.

Proposal 3: Up to now, RAN2 has not seen the clear necessity for the paging identifier to be visible to the A-IoT MAC layer.

#### 2.1.2.3 Design of paging identifier

In RAN2#129 meeting, it has been further clarified for the meaning of “one identifier” in scope of WI, that is:

1.The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

Therefore, there are three cases for the paging identifier in A-IoT paging message, that is one single device identifier, one group identifier/filtering criteria and no identifier. Note that the design of one single device identifier or one group identifier/filtering criteria should be determined in SA2, thus there is no RAN2 specific impact on the design of paging identifier, while this do not excluded the design of paging identifier field which may contains some other indications in A-IoT MAC layer as analyzed in section 2.2.

Observation 5: It is possible that there is no paging identifier field in one A-IoT paging message.

Proposal 4: There is no RAN2 specific impact on the design of paging identifier.

### 2.1.3 Transaction ID

In RAN2#129 meeting, the following agreements have been made [1]:

3.The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID 4.1 bit solution is excluded. FFS the size. Aim to have a reasonable size.

- As for the generation of transaction ID, it is not in RAN2 scope.

- In terms of the transaction ID, it is only used in A-IoT radio interface by RAN2 and is totally transparent to CN and physical layer. Thus, we think that we can determine the size of transaction ID by ourselves. In Re-19, since only gNB-reader is in scope and multi-reader scenario can be avoided by implementation as introduced in Section 2.1.1, the size of transaction ID does not need to be too long. While with the consideration that forward compatibility for UE-reader, longer transaction ID seems to be better. Based on above analysis, we think 4-bit transaction ID is enough.

Proposal 5: The method for gNB-reader generating transaction ID is not in RAN2 scope.

Proposal 6: RAN2 is asked to agree to a 4-bit transaction ID.

### 2.1.4 Indication of random access type

In RAN2#129 meeting, it has been agreed that [1]:

- 1.For Rel-19, only 3-step CBRA is supported for A-IoT - 2.We will specify both CBRA and CFRA. - 3.CFRA is not supported for group ID

With the consideration that CFRA can only be used in the case that only one specific device ID is contained in an A-IoT paging, thus whether CBRA or CFRA is triggered may be implicitly indicated by the type of paging identifier. To be specific, from A-IoT device’s perspective, if only its device ID is indicated in A-IoT paging message, CFRA is triggered, otherwise, if group identifier which the device belongs to or no identifier is indicated in A-IoT paging message, CBRA is triggered.

Observation 6: CFRA can only be used in the case that only one specific device ID is contained in an A-IoT paging in R19.

Proposal 7: For an A-IoT device, if only its device ID is indicated in A-IoT paging message, CFRA is triggered, otherwise, if the group identifier which the device belongs to is indicated or no identifier is indicated, CBRA is triggered.

### 2.1.5 Indication of D2R transmission resources

In RAN2#129 meeting, it has been agreed that [1]:

1.The A-IoT paging message can include a number of msg1 resources

Based on the previous study in RAN1, the D2R transmission resources may include at least time-domain (such as access occasion) and/or frequency-domain (such as the frequency-shift). In RAN1#120 meeting, it has been further agreed that [2]:

Agreement Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access. - The maximum value of Y >1 is determined in AI 9.4.2, based on feasible values of small frequency shift to be decided in AI 9.4.2. - Signalling details should be discussed and determined in AI 9.4.4.

Therefore it is clear that both the TDMed resources (X) and FDMed resources (Y) may need to be indicated separately in some cases.

More importantly, since there are also some parameters such as D2R chip duration, payload size for PDRCH transmission, etc., are discussed in RAN1 to be indicated by R2D control information, but haven’t been determined through physical-layer control signaling or high-layer signaling yet, one important remaining issue to be discussed is that whether the MSG1 resources and/or other parameters for D2R transmission resources need to be indicated in A-IoT paging message or in physical layer control signaling.

From our point of view, the detailed parameter of D2R transmission resources and whether all or part of them need to be indicated in high-layer signaling should be discussed and determined by RAN1. What RAN2 needs to do is wait for RAN1’s agreement and design the indication of D2R transmission resources when certain parameters need to be indicated in high-layer.

Observation 7: There are also some parameters such as D2R chip duration, payload size for PDRCH transmission, etc., are discussed in RAN1 to be indicated by R2D control information, but haven’t been determined through physical-layer control signaling or high-layer signaling yet.

Proposal 8: RAN2 is asked to wait for RAN1’s progress of D2R transmission resources, and see whether any parameter needs to be indicated in A-IoT MAC layer.

## 2.2 General design for A-IoT paging message

Based on above analysis and agreement in RAN2#129 meeting, there are two other fields which have been agreed to be included in an A-IoT paging message except for message type, that is, paging identifier and transaction ID. Note that whether indication of D2R transmission resources is also needed should be determined in RAN1. For transaction ID, we think that it may be absent for A-IoT paging message which triggers CFRA, since one A-IoT device should always respond to the A-IoT paging which only targets to itself, regardless of the transaction ID. Therefore, we think that the transaction ID is an optional field in A-IoT paging message with fixed length. As for paging identifier, it is an optional field because it has been agreed that no identifier can be contained in A-IoT paging message to page all A-IoT devices. Besides, we think that it is also very likely that different types of paging identifier, e.g. specific device identifier or group identifier, will be in variable length. Finally, if indication of D2R transmission resources is also required to be carried in A-IoT paging message, we think it may also be an optional field, because no explicit indication of D2R transmission resource seems to be possible for CFRA case.

Based on above analysis and the following agreement in RAN2#129 meeting that “At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s)”, we think that the following principle should be followed:

- For each optional field in A-IoT paging message, one indication is needed to indicated whether it exists in current A-IoT paging message.

- For each field (whether optional or mandatory) with variable length, a length indication is needed to indicated its length in current A-IoT paging message.

- For the mandatory field(s) with fixed length, it(they) can be tightly arranged without any additional indication.

Observation 8: There are two fields which have been agreed to be included in an A-IoT paging message, that is, paging identifier and transaction ID. Whether indication of D2R transmission resources is also needed should be determined in RAN1.

Observation 9: Transaction ID is an optional field with fixed length.

Observation 10: Paging identifier is an optional field and is very likely to be in variable length.

- Proposal 9: At least for the design of A-IoT paging message, RAN2 is asked to agree to the following designing principles:

- For each optional field in A-IoT paging message, one indication is needed to indicated whether it exists in current A-IoT paging message.

- For each field (whether optional or mandatory) with variable length, a length indication is needed to indicated its length in current A-IoT paging message.

- For the mandatory field(s) with fixed length, it(they) can be tightly arranged without any additional indication.

To this end, for the transaction ID field, no any additional indication is needed. In terms of paging identifier, one indication is needed to indicate whether it exists in current A-IoT paging message, and if it does, a length indication is also needed. To this end, we think the following fields may be included in one A-IoT paging message:

- Message type (mandatory)

- Indication which indicates whether transaction ID is included or not (mandatory)

- Transaction ID (optional)

- Indication which indicates whether paging identifier is included or not (mandatory)

- Paging identifier field & length indication (optional)

- D2R transmission resources field (to be determined in RAN1 whether it is needed)

A schematic diagram of the format of A-IoT paging message are shown as following:

Figure 1. Schematic diagram of the format of A-IoT paging message which pages all devices

Figure 2. Schematic diagram of the format of A-IoT paging message which pages group of devices

Figure 3. Schematic diagram of the format of A-IoT paging message which pages one specific device

Finally, in terms of the forward compatibility that multiple identifiers can be contained in one A-IoT paging message, some companies think that design a reserved field is enough. However, we think that reserving some bits may not work well, because it is difficult for RAN2 to determine how many bits should be reserved and it is also uncertain whether the reserved bits can solve the scenarios of using A-IoT paging that may be expanded in the future. Therefore, we prefer to design A-IoT paging messages for R19 and R20 separately, that is, they are different in message type.

- Proposal 10: The following fields may be included in the A-IoT paging message:

- Message type (mandatory)

- Indication which indicates whether transaction ID is included or not (mandatory)

- Transaction ID (optional)

- Indication which indicates whether paging identifier is included or not (mandatory)

- Paging identifier field & length indication (optional)

- D2R transmission resources field (to be determined in RAN1 whether it is needed)

- Proposal 11: RAN2 is asked to take the format of A-IoT paging message shown in Figure 1, 2 and 3 as a start point.

- Proposal 12: RAN2 is asked to design A-IoT paging messages for R19 and R20 separately.

## 2.3 Subsequent A-IoT paging

In RAN2, it has been discussed and agreed that one or more subsequent A-IoT paging messages can be sent by gNB-reader to trigger the A-IoT device(s) which fails to complete the procedure initiated by initial A-IoT paging message. In fact, with the consideration of the unavailability of A-IoT devices caused by their insufficient energy or fading in PRDCH, from the reader's perspective, the subsequent A-IoT paging message it sends may be treated as initial A-IoT paging messages by some A-IoT devices. To this end, we think that there should be no difference between the subsequent A-IoT paging message and initial A-IoT paging message for at least the fields other than D2R transmission resources field (if needed in A-IoT paging). If  RAN1 determines that the D2R transmission resources field is also need in A-IoT paging, RAN2 can further discuss whether this field can be different between subsequent paging message and initial paging message.

Observation 11: With the consideration of the unavailability of A-IoT devices caused by their insufficient energy or fading in PRDCH, from the reader's perspective, the subsequent A-IoT paging message it sends may be treated as initial A-IoT paging messages by some A-IoT devices.

- Proposal 13: There should be no difference between the subsequent A-IoT paging message and initial A-IoT paging message for at least the fields other than D2R transmission resources field (if needed in A-IoT paging).

Proposal 14: RAN2 is asked to first wait for RAN1 determination on whether the D2R transmission resources field should be included in A-IoT paging before discussing whether this field can be different between subsequent paging message and initial paging message.

## 2.4 Ways for A-IoT device determining MSG1 resources

In RAN2#129 meeting, the following two ways are discussed and agreed:

- 2. From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

From our point view, there is no essential difference between Way-1 and Way-2. To this end, we suggest that RAN2 can discuss why the message (new R2D message or something like paging message) is needed and which information is needed for A-IoT device determining MSG1 resources at first before discussing whether a new R2D message is used or A-IoT paging message is reused.

The reason why this message (new R2D message or something like paging message) is needed is the poor timing accuracy of device 1 in R19. To be specific, the initial clock accuracy of device 1 is about 10^4 ~ 10^5 ppm which is the evaluation assumption in RAN1 as captured in TR 38.769, thus it should be really difficult for device 1 determining the transmission/access occasion efficiently only with its internal timing. To this end, this message (new R2D message or something like paging message) is just to indicate to the A-IoT device that a slot/time-domain occasion is started and no any additional information (such as: paging identifier, transaction, etc.) is needed, which also explains why we also think that it may be L1 signaling. Based on this, we think that a new R2D message can be introduced, because all the fields in A-IoT paging message is not needed for the message for A-IoT device determining MSG1 resources.

Observation 12: The reason why this message (new R2D message or something like paging message) is needed is that the timing accuracy of device 1 in R19 is poor.

Observation 13: This message (new R2D message or something like paging message) is just to indicate to the A-IoT device that a slot/time-domain occasion is started and no any additional information (such as: paging identifier, transaction, etc.) is needed.

Proposal 15: A new R2D message other than the paging message (Way1-1) is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on A-IoT paging as following:

Observation 1: The most important impact brought by multi-reader scenario is interference which can not be avoided/mitigated by introducing reader ID.

Observation 2: There are some implementation methods to avoid the occurrence of multi-reader scenarios or mitigating its impact as much as possible.

Observation 3: Sub-grouping performed by the reader is an optimization scheme for the A-IoT procedure and totally transparent to A-IoT device.

Observation 4: Whether gNB-CN device ID is needed and the relationship between gNB-CN device ID and paging identifier have not determined by RAN3/SA2.

Observation 5: It is possible that there is no paging identifier field in one A-IoT paging message.

Observation 6: CFRA can only be used in the case that only one specific device ID is contained in an A-IoT paging in R19.

Observation 7: There are also some parameters such as D2R chip duration, payload size for PDRCH transmission, etc., are discussed in RAN1 to be indicated by R2D control information, but haven’t been determined through physical-layer control signaling or high-layer signaling yet.

Observation 8: There are two fields which have been agreed to be included in an A-IoT paging message, that is, paging identifier and transaction ID. Whether indication of D2R transmission resources is also needed should be determined in RAN1.

Observation 9: Transaction ID is an optional field with fixed length.

Observation 10: Paging identifier is an optional field and is very likely to be in variable length.

Observation 11: With the consideration of the unavailability of A-IoT devices caused by their insufficient energy or fading in PRDCH, from the reader's perspective, the subsequent A-IoT paging message it sends may be treated as initial A-IoT paging messages by some A-IoT devices.

Observation 12: The reason why this message (new R2D message or something like paging message) is needed is that the timing accuracy of device 1 in R19 is poor.

Observation 13: This message (new R2D message or something like paging message) is just to indicate to the A-IoT device that a slot/time-domain occasion is started and no any additional information (such as: paging identifier, transaction, etc.) is needed.

Proposal 1: In R19, reader ID is not needed for multi-reader scenario.

Proposal 2: The paging identifier contained in A-IoT paging message can be a temporary ID. Whether the paging identifier can only be the temporary ID needs further input from SA2/SA3.

Proposal 3: Up to now, RAN2 has not seen the clear necessity for the paging identifier to be visible to the A-IoT MAC layer.

Proposal 4: There is no RAN2 specific impact on the design of paging identifier.

Proposal 5: The method for gNB-reader generating transaction ID is not in RAN2 scope.

Proposal 6: RAN2 is asked to agree to a 4-bit transaction ID.

Proposal 7: For an A-IoT device, if only its device ID is indicated in A-IoT paging message, CFRA is triggered, otherwise, if the group identifier which the device belongs to is indicated or no identifier is indicated, CBRA is triggered.

Proposal 8: RAN2 is asked to wait for RAN1’s progress of D2R transmission resources, and see whether any parameter needs to be indicated in A-IoT MAC layer.

- Proposal 9: At least for the design of A-IoT paging message, RAN2 is asked to agree to the following designing principles:

- For each optional field in A-IoT paging message, one indication is needed to indicated whether it exists in current A-IoT paging message.

- For each field (whether optional or mandatory) with variable length, a length indication is needed to indicated its length in current A-IoT paging message.

- For the mandatory field(s) with fixed length, it(they) can be tightly arranged without any additional indication.

- Proposal 10: The following fields may be included in the A-IoT paging message:

- Message type (mandatory)

- Indication which indicates whether transaction ID is included or not (mandatory)

- Transaction ID (optional)

- Indication which indicates whether paging identifier is included or not (mandatory)

- Paging identifier field & length indication (optional)

- D2R transmission resources field (to be determined in RAN1 whether it is needed)

- Proposal 11: RAN2 is asked to take the format of A-IoT paging message shown in Figure 1, 2 and 3 as a start point.

- Proposal 12: RAN2 is asked to design A-IoT paging messages for R19 and R20 separately.

- Proposal 13: There should be no difference between the subsequent A-IoT paging message and initial A-IoT paging message for at least the fields other than D2R transmission resources field (if needed in A-IoT paging).

Proposal 14: RAN2 is asked to first wait for RAN1 determination on whether the D2R transmission resources field should be included in A-IoT paging before discussing whether this field can be different between subsequent paging message and initial paging message.

Proposal 15: A new R2D message other than the paging message (Way1-1) is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.

Reference

- “RAN2 chair’s notes”, RAN2#129, Greece, Athens, RAN2 Chair (InterDigital).

- “RAN1 chair’s notes”, RAN2#120, Greece, Athens, RAN1 Chair.