3GPP TSG-RAN WG2 Meeting #129bis	R2-2502711

Wuhan, China, Apr 7th – 11th, 2025

Agenda Item:	8.3.5

Source:	CMCC

Title:	Discussion on LCM for RRM measurement prediction with UE sided model

WID/SID:				FS_NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

In last meeting RAN2#129 [1], RAN2 discussed the work plan of AI mobility SI and achieved the following agreements.

- Agreements on priority

- RLF prediction will not be studied in Rel-19

- Further RRM prediction simulations are not expected.  For next specification general impact study phase we will consider spatial, temporal, and frequency domain cell level prediction.   FFS if L3 beam level needs to be considered.  For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

- Continue with generalization across cell configurations for RRM prediction

- Continue with measurement events

In this contribution, we provide our initial consideration on the LCM framework and required signaling and protocol for RRM measurement prediction with UE sided model.

# 2 Discussion

2.1 Life Cycle Management

Figure 4.4-1 in TR 38.843[2] illustrates the functional framework for AI/ML for NR air interface. The aim of this framework is to cover a general functional architecture addressing both model-ID-based LCM and functionality-based LCM. We think the functional framework could be reused in AI/ML for Mobility.

Figure 4.4-1: Functional framework for AI/ML for NR Air Interface [2]

Proposal 1: The functional framework introduced for AI/ML for NR air interface could be reused in AI/ML for Mobility.

For LCM in AI/ML for NR air interface WI, the following aspects are studied in RAN2:

-	UE capability

-	Functionality/model monitoring

-	Functionality/model selection, activation, deactivation, switching, and fallback operation.

-	Model training

-	Model delivery/transfer

-	Model inference operation

-	Data collection

Since only one side model (i.e. UE sided model or NW sided model) is studied in Rel-19 AI/ML for Mobility, there is no need to consider Model delivery/transfer for LCM in AI/ML for Mobility. Moreover, model training is more up to implementation. And it is no need to be covered in LCM study of AI/ML for Mobility. Therefore, we think the following aspects could be studied for LCM in AI/ML for Mobility:

-	UE capability

-	Functionality/model monitoring

-	Functionality/model management: selection, activation, deactivation, switching, and fallback operation.

-	Data collection

Proposal 2: The following aspects could be studied for LCM in AI/ML for Mobility:

-	UE capability

-	Functionality/model monitoring

-	Functionality/model management: selection, activation, deactivation, switching, and fallback operation.

-	Data collection

LCM study in AI/ML for NR air interface WI includes Functionality-based LCM and Model ID-based LCM. Considering model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions as determined/identified between UE-side and NW-side [2] and more details need to be studied, now RAN2 discussion focus on Functionality-based LCM.

Observation 1: For LCM in AI/ML for NR air interface WI, RAN2 discussion focus on Functionality-based LCM.

Proposal 3: RAN2 takes Functionality-based LCM framework as the starting point for the study on LCM and specification impact for AI/ML for Mobility.

2.2 Definition of AI/ML-enabled Feature and functionality

According to the following definition of AI/ML functionality identification in AI/ML for NR air interface specified in TR 38.843[2], we could observe that AI/ML-enabled Feature refers to a Feature where AI/ML may be used and there may be either one or more than one Functionalities defined within an AI/ML-enabled feature. That means functionality could have finer granularity than AI/ML-enabled feature.

For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

Observation 2: According to the description in TR 38.843[2], AI/ML-enabled Feature refers to a Feature where AI/ML may be used and there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

For AI/ML for Mobility, AI/ML-enabled features could be defined based on the use cases including RRM measurement prediction, Measurement events prediction and HO failure/RLF prediction (not be studied in Rel-19).

Proposal 4: AI/ML-enabled features in AI/ML for Mobility could be defined based on the use cases, including:

- AI/ML-enabled RRM measurement prediction

- AI/ML-enabled Measurement events prediction

Based on the different granularity of functionality, there will be several ways to define the Functionalities within an AI/ML-enabled feature. For example, the AI/ML functionalities for RRM measurement prediction could be defined as temporal domain prediction, frequency domain prediction and spatial domain prediction. With the finer granularity, the AI/ML functionalities for RRM measurement prediction could be defined based on the sub use cases for cell level measurement prediction or beam level measurement prediction.

Observation 3: Based on the different granularity of functionality, there will be several ways to define the Functionalities within an AI/ML-enabled feature.

Agreements 1 For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results

Agreements - Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results.

Based on the above analysis, we show some examples for the definition of functionalities for RRM measurement prediction:

- Option 1: Based on the prediction type, the AI/ML functionalities for RRM measurement prediction could be:

- Temporal domain prediction

- Frequency domain prediction

- Spatial domain prediction

- Option 2: Based on the finer granularity (e.g., combine prediction types and sub-use cases), in Temporal domain prediction, Frequency domain prediction and Spatial domain prediction the AI/ML functionalities could be further divided into:

- To predict beam level results, then generate cell level results based on the predicted beam results

- To directly predict cell level results based on cell level results

- To directly predict cell level results based on beam level results

Proposal 5: RAN2 considers the granularity of functionality, for example,

- Option 1: Based on the prediction type, the AI/ML functionalities for RRM measurement prediction could be:

- Temporal domain prediction

- Frequency domain prediction

- Spatial domain prediction

- Option 2: Based on the finer granularity (e.g., combine prediction types and sub-use cases), in Temporal domain prediction, Frequency domain prediction and Spatial domain prediction the AI/ML functionalities could be further divided into:

- To predict beam level results, then generate cell level results based on the predicted beam results

- To directly predict cell level results based on cell level results

- To directly predict cell level results based on beam level results

2.3 UE capability and Applicable functionality reporting

For AI/ML for NR air interface, the legacy UE capability framework serves as the baseline to report UE’s supported AI/ML-enabled Feature/FG. We think the legacy UE capability framework could also be reused as the baseline for AI/ML for Mobility. Therefore, for RRM measurement prediction use case, this information is indicated in UE AS capability in RRC (e.g., UECapabilityEnquiry/UECapabilityInformation).

In AI/ML for NR air interface, RRCReconfiguration message is used to carry the full inference configuration and/or inference related info which are used for UE’s decision on applicability/inapplicability of functionality. And the initial applicability/inapplicability reporting is carried in RRCReconfigurationComplete message. When there are changes for applicability/inapplicability of functionality, the subsequent applicability/inapplicability reporting could be carried in UAI. The basic procedure for UE capability and applicability functionality reporting in AI/ML for NR air interface WI is shown in Fig 1.

Fig 1. Basic procedure for applicability functionality reporting

For AI/ML for Mobility, we think the above signalling procedure for UE capability and applicable functionality reporting could be reused. Specifically, RRCReconfiguration message is used to carry the inference configuration/inference related info of RRM measurement prediction. The initial applicability/inapplicability reporting of the functionality in RRM measurement prediction is carried in RRCReconfigurationComplete message. And UAI is reused to carry the subsequent applicability/inapplicability reporting.

Proposal 6: The signalling procedure for UE capability and applicable functionality reporting in Fig1 could be reused for AI/ML for Mobility:

- UE capability framework (i.e. UECapabilityEnquiry/UECapabilityInformation) could be used to report UE’s supported AI/ML-enabled Feature (e.g., AI/ML-enabled RRM measurement prediction and AI/ML-enabled Measurement events prediction) and/or supported functionalities for AI/ML for Mobility.

- RRCReconfiguration message could be used to carry the inference configuration and/or inference related info of RRM measurement prediction

- RRCReconfigurationComplete message could be used to carry the initial applicability/inapplicability reporting of the functionality

- UAI could be used to carry the subsequent applicability/inapplicability reporting.

In AI/ML for NR air interface WI, inference related info is provided from NW to UE as assistance information to help UE identify applicability/inapplicability the functionality, the information includes

- the associated ID

- Set A related information

- Set B related information

- Report content related information

Observation 4: In AI/ML for NR air interface WI, inference related info is provided from NW to UE as assistance information to help UE identify applicability/inapplicability the functionality.

Considering there are two study goals for RRM measurement prediction in AI/ML for Mobility, including measurement overhead reduction and HO performance improvement, the content of inference related info of these two study goals maybe different. For example, for measurement overhead reduction, at least the measurement reduction rate and model input and output related info should be considered in inference related info; for HO performance improvement, Observation Window, Prediction Window, model input and output related info should be considered in inference related info.

Proposal 7: For 1st goal (measurement overhead reduction), the following content of inference related info could be considered as the starting point for RRM measurement prediction:

- Temporal domain prediction: MRRT, Observation Window, Prediction Window, model input and output related info (e.g., L3 cell level RSRP), sample period, FR1/FR2, etc.

- Frequency domain prediction: input frequency (e.g., 2GHz), output frequency (e.g., 4GHz), model input and output related info (e.g., L3 cell level RSRP), etc.

- Spatial domain prediction: MRRS, beam pattern, model input and output related info (e.g., beam level RSRP), FR1/FR2, etc.

Proposal 8: For 2nd goal (HO performance improvement), the following content of inference related info could be considered as the starting point for RRM measurement prediction:

- Temporal domain prediction: Observation Window, Prediction Window, model input and output related info (e.g., L3 cell level RSRP), sample period, FR1/FR2, etc.

2.4 Monitoring and management

In AI/ML for NR air interface, Functionality management refers to activation/deactivation/fallback/switching of AI/ML functionality, which could be indicated via 3GPP signalling (e.g., RRC, MAC-CE, DCI) for functionality-based LCM. Based on the description in TR 38.843[2], we could observe the management decision could be taken by the network or by the UE.

Specifically, there are two options for functionality/model management in AI/ML for NR air interface.

- Option 1: Decision by the network

1a) Network-initiated

1b) UE-initiated and requested to the network

- Option 2：Decision by the UE

2a) Event-triggered as configured by the network, UE’s decision is reported to the network

2b) UE-autonomous, UE’s decision is reported to the network

2c) UE-autonomous, UE’s decision is not reported to the network

According to the progress AI/ML for NR air interface WI, we could observe that Option 1a is taken as baseline, Option 1b and 2b would be further discussed, and Option 2c is not considered in Rel-19.

Observation 5: There are two options for functionality/model management in AI/ML for NR air interface. According to the progress AI/ML for NR air interface WI, Option 1a is taken as baseline, Option 1b and 2b would be further discussed, and Option 2c is not considered in Rel-19.

Figure 7.2.1.1-1: Network decision, network-initiated AI/ML management [2]           Figure 7.2.1.1-2: Network decision, UE-initiated AI/ML management [2]

Figure 7.2.1.1-3: UE decision, event-triggered as configured by the network [2]              Figure 7.2.1.1-4: UE autonomous, decision reported to the network [2]

Proposal 9: Option 1a (Network decision, network-initiated AI/ML management) could be considered as the starting point for the study of functionality/model management in AI/ML for Mobility.

Regarding to the configuration in above figures, we think the functionality monitoring related info (KPIs of RRM measurement prediction, e.g., RSRP difference), the measurement and reporting configuration should be considered in AI/ML for Mobility.

Proposal 10: The content of configuration for functionality management in AI/ML for Mobility, at least the following information and configuration should be considered:

- Functionality monitoring related info (KPIs of RRM measurement prediction, e.g., RSRP difference)

- UE measurement and reporting configuration

2.5 UE side data collection

Data collection for training:

According to the discussion in AI/ML for NR air interface WI, we could observe the training of UE-sided model is only in OTT-server of UE vendors. The following Options of data collection for UE-sided model training are considered in AI/ML for NR air interface:

- Option 1: UE collects and directly transfers training data to the Over-The-Top (OTT) server

1a) OTT (3GPP transparent)

1b) OTT (non-3GPP transparent)

- Option 2: UE collects training data and transfers it to Core Network. Core Network transfers the training data to the OTT server

- Option 3: UE collects training data and transfers it to OAM. OAM transfers the needed data to the OTT server

According to the progress of AI/ML for NR air interface WI, Option 1b was excluded and Option 2 and Option 3 are still under discussion. We could wait for the progress on the solution of training data collection and transfer for UE sided model in AI/ML for NR air interface WI.

Proposal 11: Wait for the progress in AI/ML for NR air interface WI on the solution of training data collection and transfer for UE sided model.

Cosidering there are different implementation of AI/ML model, at least L3 cell level measurement results (e.g., RSRP) and/or L1 beam level measurement results  and/or L3 beam level measurement results  should be collected from UE for the data content for UE sided model training for RRM measurement prediction. Other assistance information could be further discussed. Specifically,

- Temporal domain prediction: beam/cell level RSRPs of one or more than one intra-frequency cell(s)

- Frequency domain prediction: beam/cell level RSRPs of one or more than one inter-frequency cell(s)

- Spatial domain prediction: beam level RSRPs of one or more than one intra-frequency cell(s)

Proposal 12: For the data content for UE sided model training, at least L3 cell level measurement results and/or L1 beam level measurement results and/or L3 beam level measurement results should be collected from UE for RRM measurement prediction, e.g., beam/cell level RSRPs of one or more than one intra-frequency cell(s) or inter-frequency cell(s).

Data collection for inference:

Based on the agreed sub use cases for cell level measurement prediction and beam level measurement prediction in the stage of evaluation for RRM measurement prediction, we understand L3 cell level measurement results (e.g., RSRP) and/or L1 beam level measurement results and/or L3 beam level measurement results should be considered for AI/ML model inference in RRM measurement prediction.

Proposal 13: At least L3 cell level measurement results and/or L1 beam level measurement results and/or L3 beam level measurement results should be considered as model input for AI/ML model inference in RRM measurement prediction.

# 3 Conclusion

In this paper, we provide our initial consideration on LCM framework and required signaling and protocol for RRM measurement prediction with UE sided model. We have the following observation and proposals:

Life Cycle Management

Proposal 1: The functional framework introduced for AI/ML for NR air interface could be reused in AI/ML for Mobility.

Proposal 2: The following aspects could be studied for LCM in AI/ML for Mobility:

-	UE capability

-	Functionality/model monitoring

-	Functionality/model management: selection, activation, deactivation, switching, and fallback operation.

-	Data collection

Observation 1: For LCM in AI/ML for NR air interface WI, RAN2 discussion focus on Functionality-based LCM.

Proposal 3: RAN2 takes Functionality-based LCM framework as the starting point for the study on LCM and specification impact for AI/ML for Mobility.

Definition of AI/ML-enabled Feature and functionality

Observation 2: According to the description in TR 38.843[2], AI/ML-enabled Feature refers to a Feature where AI/ML may be used and there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

Proposal 4: AI/ML-enabled features in AI/ML for Mobility could be defined based on the use cases, including:

- AI/ML-enabled RRM measurement prediction

- AI/ML-enabled Measurement events prediction

Observation 3: Based on the different granularity of functionality, there will be several ways to define the Functionalities within an AI/ML-enabled feature.

Proposal 5: RAN2 considers the granularity of functionality, for example,

- Option 1: Based on the prediction type, the AI/ML functionalities for RRM measurement prediction could be:

- Temporal domain prediction

- Frequency domain prediction

- Spatial domain prediction

- Option 2: Based on the finer granularity (e.g., combine prediction types and sub-use cases), in Temporal domain prediction, Frequency domain prediction and Spatial domain prediction the AI/ML functionalities could be further divided into:

- To predict beam level results, then generate cell level results based on the predicted beam results

- To directly predict cell level results based on cell level results

- To directly predict cell level results based on beam level results

UE capability and Applicable functionality reporting

Proposal 6: The signalling procedure for UE capability and applicable functionality reporting in Fig1 could be reused for AI/ML for Mobility:

- UE capability framework (i.e. UECapabilityEnquiry/UECapabilityInformation) could be used to report UE’s supported AI/ML-enabled Feature (e.g., AI/ML-enabled RRM measurement prediction and AI/ML-enabled Measurement events prediction) and/or supported functionalities for AI/ML for Mobility.

- RRCReconfiguration message could be used to carry the inference configuration and/or inference related info of RRM measurement prediction

- RRCReconfigurationComplete message could be used to carry the initial applicability/inapplicability reporting of the functionality

- UAI could be used to carry the subsequent applicability/inapplicability reporting.

Observation 4: In AI/ML for NR air interface WI, inference related info is provided from NW to UE as assistance information to help UE identify applicability/inapplicability the functionality.

Proposal 7: For 1st goal (measurement overhead reduction), the following content of inference related info could be considered as the starting point for RRM measurement prediction:

- Temporal domain prediction: MRRT, Observation Window, Prediction Window, model input and output related info (e.g., L3 cell level RSRP), sample period, FR1/FR2, etc.

- Frequency domain prediction: input frequency (e.g., 2GHz), output frequency (e.g., 4GHz), model input and output related info (e.g., L3 cell level RSRP), etc.

- Spatial domain prediction: MRRS, beam pattern, model input and output related info (e.g., beam level RSRP), FR1/FR2, etc.

Proposal 8: For 2nd goal (HO performance improvement), the following content of inference related info could be considered as the starting point for RRM measurement prediction:

- Temporal domain prediction: Observation Window, Prediction Window, model input and output related info (e.g., L3 cell level RSRP), sample period, FR1/FR2, etc.

Monitoring and management

Observation 5: There are two options for functionality/model management in AI/ML for NR air interface. According to the progress AI/ML for NR air interface WI, Option 1a is taken as baseline, Option 1b and 2b would be further discussed, and Option 2c is not considered in Rel-19.

Proposal 9: Option 1a (Network decision, network-initiated AI/ML management) could be considered as the starting point for the study of functionality/model management in AI/ML for Mobility.

Proposal 10: The content of configuration for functionality management in AI/ML for Mobility, at least the following information and configuration should be considered:

- Functionality monitoring related info (KPIs of RRM measurement prediction, e.g., RSRP difference)

- UE measurement and reporting configuration

Data collection for training

Proposal 11: Wait for the progress in AI/ML for NR air interface WI on the solution of training data collection and transfer for UE sided model.

Proposal 12: For the data content for UE sided model training, at least L3 cell level measurement results and/or L1 beam level measurement results and/or L3 beam level measurement results should be collected from UE for RRM measurement prediction, e.g., beam/cell level RSRPs of one or more than one intra-frequency cell(s) or inter-frequency cell(s).

Data collection for inference

Proposal 13: At least L3 cell level measurement results and/or L1 beam level measurement results and/or L3 beam level measurement results should be considered as model input for AI/ML model inference in RRM measurement prediction.

# Reference

- R2_129_ChairNotes_25-02-21_final, Feb 2025.

- 3GPP TR 38.843: " Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".