3GPP TSG-RAN WG2 Meeting #129bis	R2-2502712

Wuhan, China, Apr 7th – 11th, 2025

Agenda Item:	8.3.5

Source:	CMCC

Title:	Discussion on LCM for RRM measurement prediction with NW sided model

WID/SID:				FS_NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

In last meeting RAN2#129 [1], RAN2 discussed the work plan of AI mobility SI and achieved the following agreements.

- Agreements on priority

- RLF prediction will not be studied in Rel-19

- Further RRM prediction simulations are not expected.  For next specification general impact study phase we will consider spatial, temporal, and frequency domain cell level prediction.   FFS if L3 beam level needs to be considered. For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

- Continue with generalization across cell configurations for RRM prediction

- Continue with measurement events

In this contribution, we provide our initial consideration on the LCM framework and required signaling and protocol for RRM measurement prediction with NW sided model.

# 2 Discussion

LCM functional framework and studied aspects are discussed in our contribution on RRM measurement prediction with UE sided model [2]. This contribution will analysis the specific aspects related to RRM measurement prediction with NW sided model.

2.1 Signalling procedure for NW sided model

In the previous RAN2 meetings, the following agreements were achieved for NW-sided model in AI/ML for NR air interface WI.

RAN2#125bis Agreements

1	RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

2	RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

3	RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

For AI/ML for Mobility, we have common understanding that there are two study goals for this study item, including

- 1st goal: UE Measurement overhead reduction

- 2nd goal: HO performance improvement

For the first study goal, RRM measurement prediction is helpful to reduce UE measurements efforts and power consumption. According to the agreements shown in section 2.1, we could observe that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) in AI/ML for NR air interface. But for RRM measurement prediction with NW sided model, we think if the NW sided model/functionality activation and UE measurements efforts could be reduced, at least UE should be informed about the Measurement reduction related information.

Observation 1: For AI/ML for NR air interface, UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.).

Proposal 1: For RRM measurement prediction with NW sided model, if the NW sided model/functionality activation and UE measurements efforts could be reduced, at least UE should be informed about the Measurement reduction related information.

Based on the description in TR 38.843[3], there are two options for functionality/model management in AI/ML for NR air interface. Based on the agreements in RAN2#125bis, UE will not be involved in any gNB/LMF-sided model/functionality management decision making. Therefore, Option 1a (Network decision, network-initiated AI/ML management) is the baseline for NW sided model.

Observation 2: For AI/ML for NR air interface, UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.).

Figure 7.2.1.1-1: Network decision, network-initiated AI/ML management [3]

Specifically,

- For the Configuration in above figure, the Measurement configuration and Reporting configuration should be included, which could be used to collect data and assistance information from UE for NW sided mode management, training, inference and performance monitoring.

- The Performance or Assistance Information in above figure, it is used to report the requested measurement results or assistance information from UE to NW for NW sided mode management, training, inference and performance monitoring.

- Management Instruction in above figure may include Measurement reduction related information, if the NW sided model/functionality activation and UE measurements efforts could be reduced.

Proposal 2: Option 1a (Network decision, network-initiated AI/ML management) could be used not only for functionality/model management but also for data collection for NW sided model.

2.2 Data collection for NW sided model

In the discussion for AI/ML for NR air interface, there are two options for data collection for NW sided model as shown below. And the immediate MDT procedure in OAM-centric is considered as the baseline.

- gNB-centic: gNB can configure the UE to initiate/terminate the data collection procedure. (L3 signaling)

- OAM-centric：OAM provides the configuration (via the gNB) needed for the UE to initiate/terminate the data collection procedure.

For Data collection for NW sided model in RRM measurement prediction, the immediate MDT procedure in OAM-centric could also be taken as the baseline.

Proposal 3: The immediate MDT procedure in OAM-centric could be taken as the baseline for Data collection for NW sided model in RRM measurement prediction.

Data collection for training:

Cosidering there are different implementation of AI/ML model, at least L3 cell level measurement results (e.g., RSRP) and/or L1 beam level measurement results and/or L3 beam level measurement results should be collected from UE for the data content for NW sided model training for RRM measurement prediction. Other assistance information could be further discussed. For example,

- Temporal domain prediction: beam/cell level RSRPs of one or more than one intra-frequency cell(s)

- Frequency domain prediction: beam/cell level RSRPs of one or more than one inter-frequency cell(s)

- Spatial domain prediction: beam level RSRPs of one or more than one intra-frequency cell(s)

Proposal 4: For the data content for UE-side model training, at least L3 cell level measurement results (e.g., RSRP) and/or L1 beam level measurement results and/or L3 beam level measurement results should be collected from UE for NW sided model training, e.g., beam/cell level RSRPs of one or more than one intra-frequency cell(s) or inter-frequency cell(s).

Data collection for inference:

Based on the agreed sub use cases for cell level measurement prediction and beam level measurement prediction in the stage of evaluation for RRM measurement prediction, we understand L3 cell level measurement results (e.g., RSRP) and/or L1 beam level measurement results and/or L3 beam level measurement results should be considered for AI/ML model inference in RRM measurement prediction.

Moreover, based on the simulation results of RRM measurement prediction provided by some companies, we observe that some assistance information (e.g., UE location information) may be helpful to improve the prediction accuracy. assistance information for model inference could be further discussed.

Proposal 5: At least L3 cell level measurement results and/or L1 beam level measurement results and/or L3 beam level measurement results should be considered for AI/ML model inference in RRM measurement prediction.

Data collection for monitoring:

During NW sided model activation, NW should keep the model performance monitoring. There is a straightforward solution that NW collects UE measurement results based on periodic or event triggered reporting as the baseline, and calculates KPIs of RRM measurement prediction (e.g., RSRP difference) to evaluate prediction accuracy. This solution needs the data collection of RRM measurement results from UE.

Proposal 6: L3 cell level measurement results and/or L1 beam level measurement results and/or L3 beam level measurement results should be considered for AI/ML model monitoring. UE measurement and reporting configuration for monitoring could be different from that for UE measurement for mobility.

# 3 Conclusion

In this paper, we provide our initial consideration on LCM framework and required signaling and protocol for RRM measurement prediction with NW sided model. We have the following observation and proposals:

Signaling procedure for NW sided model

Observation 1: For AI/ML for NR air interface, UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.).

Proposal 1: For RRM measurement prediction with NW sided model, if the NW sided model/functionality activation and UE measurements efforts could be reduced, at least UE should be informed about the Measurement reduction related information.

Observation 2: For AI/ML for NR air interface, UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.).

Proposal 2: Option 1a (Network decision, network-initiated AI/ML management) could be used not only for functionality/model management but also for data collection for NW sided model.

Data collection for NW sided model

Proposal 3: The immediate MDT procedure in OAM-centric could be taken as the baseline for Data collection for NW sided model in RRM measurement prediction.

Data collection for training

Proposal 4: For the data content for UE-side model training, at least L3 cell level measurement results (e.g., RSRP) and/or L1 beam level measurement results and/or L3 beam level measurement results should be collected from UE for NW sided model training, e.g., beam/cell level RSRPs of one or more than one intra-frequency cell(s) or inter-frequency cell(s).

Data collection for inference

Proposal 5: At least L3 cell level measurement results and/or L1 beam level measurement results and/or L3 beam level measurement results should be considered for AI/ML model inference in RRM measurement prediction.

Data collection for monitoring

Proposal 6: L3 cell level measurement results and/or L1 beam level measurement results and/or L3 beam level measurement results should be considered for AI/ML model monitoring. UE measurement and reporting configuration for monitoring could be different from that for UE measurement for mobility.

# Reference

- R2_129_ChairNotes_25-02-21_final, Feb 2025.

- R2- R2-2502711, Discussion on LCM for RRM measurement prediction with UE sided model, CMCC, April 2025.

- 3GPP TR 38.843: " Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".