3GPP TSG-WG2 Meeting #129bis			R2-2502720

Wuhan, China, Apr. 7th-11th, 2025

Agenda item:	8.7.6

Title:	Further consideration on XR rate control

Source:	CMCC

Document for:	Discussion

Introduction

The last meeting [1] discusses some critical issues on congestion and rate control for XR and leaves the following agreements and FFS:

Agreements on XR rate control - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). Companies to check with their SA4 colleagues whether there are any issues with this

This contribution discusses further details on rate control for XR, especially on rate control indication.

Discussion

  - per QoS flow indication

Guidance – Core of the contribution with detailed explanation of the problem, assumptions and proposed solutions. Several clauses/sub-clauses are of course allowed.

From our perspective, the introduction of uplink rate control in XR is primarily intended to adapt to variations in wireless channel conditions and uplink resource allocation. These variations can either recover to the expected state within a short period or persist for a longer duration. For example, the deterioration of the wireless channel may last for a short time, but it may also continue for a long time. The volume of high-priority data also affects the time when rate control should be applied, and the gNB has a more comprehensive grasp of this information.

Therefore, an effective rate control mechanism should be capable of addressing both short-term and long-term congestion scenarios.

For short-term rate control, MAC layer mechanisms can provide rapid rate adaptation, as the MAC layer is responsible for LCP and radio resource allocation. However, MAC-based rate control has limitations. MAC entity can hardly directly trigger rate adjustments for higher-layer entities e.g., video/audio codec at App layer, and MAC entity is constrained by higher layer rate control configurations, e.g., PBR and BSD. Moreover, MAC-layer handles LCP at the LCH/DRB level. In scenarios where multiple QoS flows are mapped to the same LCH, MAC layer rate control cannot explicitly indicate which specific QoS flow requires rate adjustment, leading to potential inefficiencies. Also, according to the layered architecture agreed for a long time in 3GPP, QoS flow related information shouldn’t be exposed to MAC entity.

For long-term rate control, e.g., a large volume of high-priority data can block the transmission of low-priority data, adjusting static parameters such as GFBR, MFBR, or AMBR can be more efficient. In such cases, due to MAC entity has no approach to coordinates with high layer entities, if a MAC CE were to indicate QoS flow-level rate control information, additional internal coordination based on implementation within the gNB between MAC and higher L2 sublayers would be required, which contradicts the principles of the 3GPP layered architecture. A more structured approach is to use RRC signaling to indicate QoS flow-level rate control adjustments.

Observation 1a: Short-term rate control can be efficiently managed at the MAC layer but lacks the ability to explicitly control QoS flow-level rate adjustments.

Observation 1b: Long-term rate control is better handled through adjustments to QoS flow-level parameters (e.g., GFBR, MFBR, AMBR) via RRC signalling, rather than MAC CE-based mechanisms.

Proposal 1a: Rate control at MAC layer should remain focused on short-term congestion adaptation, ensuring efficient LCH/DRB-level rate adjustments.

Proposal 1b: QoS flow-level rate control for long-term congestion handling should be managed via RRC signaling rather than MAC CE.

  - Rate control MAC CE

In the design of XR rate control indication, specifying the rate indication mechanism requires careful consideration of efficiency, granularity, and implementation complexity at UE side.

The choice between exponential and linear distribution for rate indication significantly impacts the flexibility and precision of rate control. Exponential scaling is often preferred in scenarios where a wide dynamic range is required while maintaining a compact bit representation with a constant maximum quantification error.

This is particularly beneficial for XR applications, which may experience rapid changes in bandwidth demand. An exponential scale allows finer control at lower rates while efficiently covering higher rate ranges without excessive bit consumption. Conversely, a linear distribution provides uniform granularity across the range but may result in inefficient representation when large variations in rate requirements exist.

Proposal 2: The rate table should follow exponential distribution, as it has a constant maximum quantization error across within its range.

The number of bits or codepoints allocated for rate indication must balance signaling overhead and precision.

A higher bit width enables finer granularity and a broader range but increases signaling overhead. Considering the typical rate adaptation needs of XR applications, a reasonable bit width should be chosen to ensure sufficient adaptability without unnecessary signaling overhead. Based on existing 3GPP specifications for rate indication mechanisms, a bit width in the range of 4 to 6 bits can provide an adequate trade-off.

Proposal 3: As a start, RAN2 agrees that the bit width of rate table ranges from 4 to 6 bits.

Additionally, the use of multipliers can further enhance efficiency by allowing rate indication with fewer bits while maintaining sufficient flexibility. By employing predefined scaling factors, the rate representation can cover a wider range without requiring an excessive number of bits. However, introducing multipliers increases decoding complexity and may require additional interpretation logic at both the transmitter and receiver. The decision to include a multiplier should consider the trade-off between signaling efficiency and implementation complexity.

Conclusion

This contribution offers the following observations and proposals:

Rate control indication:

Observation 1a: Short-term rate control can be efficiently managed at the MAC layer but lacks the ability to explicitly control QoS flow-level rate adjustments.

Observation 1b: Long-term rate control is better handled through adjustments to QoS flow-level parameters (e.g., GFBR, MFBR, AMBR) via RRC signalling, rather than MAC CE-based mechanisms.

Proposal 1a: Rate control at MAC layer should remain focused on short-term congestion adaptation, ensuring efficient LCH/DRB-level rate adjustments.

Proposal 1b: QoS flow-level rate control for long-term congestion handling should be managed via RRC signaling rather than MAC CE.

MAC CE design:

Proposal 2: The rate table should follow exponential distribution, as it has a constant maximum quantization error across within its range.

Proposal 3: As a start, RAN2 agrees that the bit width of rate table ranges from 4 to 6 bits.

References

[1]	R2_129 R18 MBS, R18 QoE and R19 XR session notes (Dawid)_EOM