# 3GPP TSG-RAN WG2 Meeting #129bis	                                                        R2-2502728

# Wuhan, China, Apr. 7th – 11th, 2025

# Agenda item:	8.6.2

# Source:	CMCC

# Title:	Discussion on Conditional LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#129 meeting, RAN2 reached the following agreements related to conditional LTM [1]:

Agreements on MAC issues:

- CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell.

- If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid.

- If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM.

- The CLTM TAT is per candidate configuration.

- FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

- Absolute TA value is included in the MAC CE for CLTM early TA acquisition.

- The candidate configuration ID is included in the MAC CE for CLTM early TA acquisition. And one configuration ID for one TA in one MAC CE.

Agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

In this paper, we further discuss the open issues of conditional LTM.

- Discussion

  - Scenario and coexistence

For  Rel-18 intra-CU LTM, the following scenarios are supported in [2]:

LTM supports both intra-gNB-DU and intra-gNB-CU inter-gNB-DU mobility. LTM supports both intra-frequency and inter-frequency mobility, including mobility to inter-frequency cell that is not a current serving cell. LTM is supported only for licensed spectrum. The following scenarios are supported: - PCell change in non-CA scenario and non-DC scenario; - PCell and SCell(s) change in CA scenario; - Dual connectivity scenario, PCell and MCG SCell(s) change and intra-SN PSCell and SCG SCell(s) change without MN involvement. LTM for simultaneous PCell and PSCell change is not supported.

Observation 1: intra-CU LTM can be used in DC and non-DC scenario.

However, based on the revised WID [3], intra-CU conditional LTM is limited in non-DC case:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

Besides, the current discussion on conditional LTM mainly focuses on the SA scenario, in which conditional LTM is used to modify UE’s PCell.  And it’s not clear whether it can be supported in SCell or PScell change in CA cases. In our opinion, since RAN2 has made progress in supporting early DL/UL synchronization, while the main difference between legacy intra-CU LTM and conditional LTM is the entity for LTM decision. It seems that there’re no technique issues to support the above scenarios in CLTM.

Therefore, intra-CU conditional LTM can be supported PCell change in non-CA scenario and for PCell and SCell(s)  change in CA scenario..

Proposal 1: Conditional LTM can be supported for PCell change in non-CA scenario and for PCell and SCell(s)  change in CA scenario.

For Rel-18 intra-CU LTM, cross-feature issues for intra-CU LTM was discussed, and it could coexist with coverage enhancement, MIMO 2TA and (e)RedCap. As mentioned above, the main difference of LTM and CLTM is due to the execution decision entity changed from NW to UE, and we don’t see techniques to support the coexistence of CLTM and coverage enhancement, MIMO 2TA and (e)RedCap.

Proposal 2: the coexistence of CLTM and coverage enhancement, MIMO 2TA and (e)RedCap is supported.

  - CLTM evaluation and execution

In last meeting, RAN2 agreed that network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. Therefore, for a set of candidate cells, some of them may be configured with L1 execution condition, while the others are configured L3 execution condition. Besides, RAN2 agreed that When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.. But it is not clear how the UE handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution, especially when the execution condition of the LTM candidate cells are based on different layer( L1/L3)’s measurement. In our view, in this scenario, it can be UE implementation to select a cell fulfilling the CLTM execution condition, to avoid too much UE complexity.

Proposal 3: When multiple candidate cells satisfy the CLM execution condition, it is up to UE implementation to select a cell as target cell and perform CLTM.

In RAN2#128 meeting, a new timer was introduced for TA maintenance in CLTM [4]:

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

And RAN2 achieved further agreements based on the concept of TA timer in last meeting:

Agreements on MAC issues:

- CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell.

- If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid.

- If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM.

- The CLTM TAT is per candidate configuration.

- FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

- Absolute TA value is included in the MAC CE for CLTM early TA acquisition.

- The candidate configuration ID is included in the MAC CE for CLTM early TA acquisition. And one configuration ID for one TA in one MAC CE

One open issue about TA timer is whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution. Since the TA timer is for the UE to identify whether the TA of a specific CLTM candidate cell is valid when its execution condition is satisfied, and this is independent of UE’s serving cell or target cell, though the TA may change due to UE’s mobility, but considering that LTM is mainly used in FR2, the coverage of the cell is small, therefore, we think the TA is still valid at UE side upon CLMT execution, and the related timer can be kept running at UE side, even when the MAC is reset based on R18 L2 handling ID.

Proposal 4: The received CLTM TAs of other candidate cells is still valid upon CLTM execution and the related TA timers can be kept running at UE side, even when the MAC is reset based on R18 L2 handling ID.

If Proposal 3 is approved, the next issue is how to inform UE’s target cell the running TA timers to avoid duplicate TA acquiring triggered by UE’s target cell.  To achieve this, two potential solutions can be considered, one is that UE can report the valid TA related information to the target cel after access to the target cell; the other way is to exchange the TA related information via Xn interface.

Proposal 5: The valid TA related information can be indicated to the target cell via the following ways:

- UE reports the valid TA related information after access to the target cell;

- The source cell indicate TA related information via Xn interface.

What’s more, current discussion is mainly about the timer on UE side, it’s not clear whether there’s a corresponding timer on serving cell side, and this may have effect on TAT updating.

Proposal 6: Whether there’s TA timer per candidate cell at network side should be clarified.

  - Collision with other mobility mechanism(s)

Based on [5], if the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message for mobility because an LTM command has been triggered to the UE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value. In other words, if there’s HO and LTM collision, DU may refuse CU’s HO command, in other words, LTM has higher priority than L3 HO.

Observation 2: If L3 HO and LTM almost triggered simultaneously, DU may refuse CU’s UE CONTEXT MODIFICATION REQUEST message for mobility, which means the priority of LTM is higher than L3 HO.

Considering the introduction of conditional LTM, two collision cases should be discussed:

- If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied. In this case, UE follows L3 HO command or LTM cell switch command.

- If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time. In this case, we think LTM may have higher priority like priority of legacy LTM if higher than L3 HO, UE performs LTM.

Proposal 7: If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied, UE follows L3 HO command or LTM cell switch command.

Proposal 8: If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time, LTM may have higher priority than CHO, and UE performs LTM.

- Conclusion

In this contribution, we discuss inter-CU LTM related issue. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: intra-CU LTM can be used in DC and non-DC scenario.

Observation 2: If L3 HO and LTM almost triggered simultaneously, DU may refuse CU’s UE CONTEXT MODIFICATION REQUEST message for mobility, which means the priority of LTM is higher than L3 HO.

Proposals:

Proposal 1: Conditional LTM can be supported for PCell change in non-CA scenario and for PCell and SCell(s)  change in CA scenario.

Proposal 2: the coexistence of CLTM and coverage enhancement, MIMO 2TA and (e)RedCap is supported.

Proposal 3: When multiple candidate cells satisfy the CLM execution condition, it is up to UE implementation to select a cell as target cell and perform CLTM.

Proposal 4: The received CLTM TAs of other candidate cells is still valid upon CLTM execution and the related TA timers can be kept running at UE side, even when the MAC is reset based on R18 L2 handling ID.

Proposal 5: The valid TA related information can be indicated to the target cell via the following ways:

- UE reports the valid TA related information after access to the target cell;

- The source cell indicate TA related information via Xn interface.

Proposal 6: Whether there’s TA timer per candidate cell at network side should be clarified.

Proposal 7: If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied, UE follows L3 HO command or LTM cell switch command.

Proposal 8: If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time, LTM may have higher priority than CHO, and UE performs LTM.

- References

- Report of 3GPP TSG RAN WG2 meeting #129;

- TS 38.300 NR and NG-RAN Overall Description; Stage 2 (Release 18);

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom;

- Report of 3GPP TSG RAN WG2 meeting #128;

- TS 38.473 F1 application protocol (F1AP) (Release 18).