3GPP TSG-RAN WG2 Meeting #129bis       	                      R2-2502729

Wuhan, China, 7th-11th April 2025

Agenda item:	8.1.2.2

Source:	CMCC

Title:	Discussion on LCM for UE-sided model for BM

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed LCM for UE-sided model for BM based on RAN1 reply LS [2] and achieved the following agreements.

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

- Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.   FFS if we report explicit cause

- If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- The provided periodic CSI configuration should be consistent with reported UE capabilities

- FFS option B

- Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

- Semi-persistent reporting, activated by MAC CE/DCI

- Aperiodic CSI reporting, activated by DCI

This contribution will discuss the remaining issues on UE-sided model LCM for beam management use case.

- Discussion

- 2.1 Applicable functionality reporting

- FFS on option A and B

- In RAN1 reply LS [2], RAN1 indicated that the applicability report is based A) and/or B), which means A) and B) can be supported/configured at the same time. In last meeting, RAN2 agreed signaling details for option A and FFS for option B. From the perspective of RAN2, we think that both option A and option B can be supported, and how to configure is up to NW implementation.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

- Proposal 1: Both option A and option B can be supported, and how to configure is up to NW implementation.

- In last meeting, RAN2 agreed that if option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- The follow-up issue is that if the non-applicable functionality becomes applicable and the inference configuration for the functionality is provided in step 3, whether the UE can autonomously activate the applicable functionalities upon reporting applicable functionalities via UEAssistanceInformation in step 6 for periodic CSI reporting.

- For example, the network provides three CSI-ReportConfig for functionality#1,#2,#3 for periodic CSI reporting in step 3, the UE reports applicability (#1) and inapplicability (#2 and #3) in step 4 and activates functionality#1 autonomously. After a while, functionality#2 becomes applicable, and UE reports it via UAI in step 6. Whether functionality#2 can be activated autonomously upon the reporting via UAI needs to be clarified.

- Fig 1. Basic flow for option A

- From the perspective of operator, we don’t expect the UE autonomously activate the applicable functionalities upon the subsequent applicable functionalities reporting via UEAssistanceInformation message for periodic CSI reporting.

- On the one hand, UAI is just for information for network in legacy mechanism, and there is no UE autonomous behaviour after transmitting UAI message.

- On the other hand, for non-applicable functionality reported in step 4, the network may release the reserved resources after step 4. That means the inference configuration received in step 3 may be unavailable when the functionality becomes applicable from non-applicable. We think the reasonable solution is that after the UE reports updated applicability functionality, the network can provide the inference configuration via RRCReconfiguration message.

- Proposal 2: If option A is configured in Step 3, when the non-applicable functionality reported in step 4 becomes applicable, and the changes are reported to the network via UEAssistanceInformation message, the UE cannot autonomously activate the applicable functionalities for periodic CSI reporting.

- For option B, the CSI-ReportConifg (i.e. inference configuration) included in RRCReconfiguration is provided based on the applicability/inapplicability reporting in step 4, a smart network cannot provide the inference configuration for non-applicable functionality. Therefore, there will be no situation similar to option A mentioned above.

- FFS on explicit cause

- In last meeting, RAN2 agreed to support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause. We don’t think the network needs to know the explicit cause. And we understand that the functionality applicability/inapplicability is enough for the network to provide the inference configuration or activation/deactivation.

- Based on the previous agreements, UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE), model availability in device and configuration provided in step 3 (e.g. inference configuration, or inference related parameters).

- For model availability, some companies proposed that if the UE indicates the cause “model non-availability”, the model transfer can be performed from network to the UE. However, for BM use case where one-sided model is used, we think the model transfer directly from OTT server to the UE is sufficient in this release.

- For NW-side additional condition, if NW-side additional condition of the model in UE side is different from which is provided by network, we understand that it means there is no available model for the NW-side additional condition provided by network. Therefore, the case is similar to the model non-availability analyzed above.

- For UE-side additional conditions (e.g. memory, battery), even if the UE reports to the network, the network cannot be aware of the changes for UE-side additional conditions, such as when the memory or battery can be satisfied for AI operation or not. Thus, it is not clear what benefits it brings to the network.

- Based on the above analysis, there is no need to report the explicit cause within applicability/inapplicability reporting.

- Proposal 3: The explicit cause is not needed within the applicability/inapplicability reporting.

- 2.2 Data collection

- In previous meeting, there was a FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration.

- In our understanding, although RAN2 has no agreements on the details of data collection configuration, it is provided to the UE by RRCReconfiguration message since the UE stays in RRC_CONNECTED state. As specified in TS 38.331 [3], if the UE cannot comply with RRCReconfiguration, the UE shall continue using the configuration used prior to the reception of RRCReconfiguration message or perform the actions upon going to RRC_IDLE or initiate the connection re-establishment procedure.

- 5.3.5.8 Reconfiguration Failure - 5.3.5.8.2 Inability to comply with RRCReconfiguration The UE shall: - <omitted> 1> else if RRCReconfiguration is received via NR (i.e., NR standalone, NE-DC, or NR-DC): - <omitted> 2> else if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over the SRB1 or if the upper layers indicate that the nas-Container is invalid: NOTE 0a: The compliance also covers the SCG configuration carried within octet strings e.g. field mrdc SecondaryCellGroupConfig. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded SCG configuration or with the combination of (parts of) the MCG and SCG configurations. NOTE 0b: The compliance also covers the V2X sidelink configuration carried within an octet string, e.g. field sl ConfigDedicatedEUTRA. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded V2X sidelink configuration. 3> if the RRCReconfiguration message was received as part of ConditionalReconfiguration; or, 3> if the RRCReconfiguration message was received as part of ltm-Config: 4> continue using the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected; 3> else: 4> continue using the configuration used prior to the reception of RRCReconfiguration message; 3> if AS security has not been activated: 4> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other' 3> else if AS security has been activated but SRB2 and at least one DRB or multicast MRB or, for IAB and NCR, SRB2, have not been setup: 4> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'; 3> else: 4> initiate the connection re-establishment procedure as specified in 5.3.7, upon which the reconfiguration procedure ends; - <omitted>

- For data collection for model training, we think it is a separate procedure from other UE behaviors in connected mode. If the UE cannot perform data collection based on received configuration, there is no need to initiate the connection re-establishment procedure or perform the actions upon going to RRC_IDLE, and the other UE behaviors should not be affected and should be performed as usual.

- Proposal 4: When the UE can’t perform data collection for model training based on received configuration, other UE behaviors in connected mode should not be affected.

- However, we think the network needs to know when the UE cannot perform data collection, e.g. re-configure the UE. Therefore, an indication is needed from UE to network to indicate that the UE cannot perform data collection based on received configuration. Furthermore, the indication can be included in RRCReconfigurationComplete message.

- Proposal 5: An indication is introduced in RRCReconfigurationComplete message when UE can’t perform data collection based on received configuration.

# 3	Conclusion

Here are the observations and proposals for LCM for UE-sided model for beam management.

Applicable functionality reporting

- Proposal 1: Both option A and option B can be supported, and how to configure is up to NW implementation.

- Proposal 2: If option A is configured in Step 3, when the non-applicable functionality reported in step 4 becomes applicable, and the changes are reported to the network via UEAssistanceInformation message, the UE cannot autonomously activate the applicable functionalities for periodic CSI reporting.

- Proposal 3: The explicit cause is not needed within the applicability/inapplicability reporting.

Data collection

- Proposal 4: When the UE can’t perform data collection for model training based on received configuration, other UE behaviors in connected mode should not be affected.

- Proposal 5: An indication is introduced in RRCReconfigurationComplete message when UE can’t perform data collection based on received configuration.

# 4	Reference

- Draft_R2_129_Meeting_Report_v3

- R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model

- TS 38.331 NR; Radio Resource Control (RRC) Protocol Specification