3GPP TSG-RAN WG2 Meeting #129bis        	                       R2-2502730

Wuhan, China, 7th-11th April 2025

Agenda item:	8.1.3

Source:	CMCC

Title:	Discussion on NW side data collection

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed the NW side data collection and achieved the following agreements.

All agreements for NW side data collection

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

- Low power bit indication is supported

- Data availability indication is supported.  FFS when this would be triggered

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- FFS how to handle idle/inactive and RLF cases and whether we have a unified.

This contribution will discuss the remaining issues for NW side data collection.

- Discussion

- 2.1 NW side data collection for beam management

- Issue#1: FFS on data availability indication reporting

- RAN2 has agreed that when UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. In last meeting, RAN2 discussed when to report to the network, after or before the buffer is full, but had no consensus.

- Some companies proposed that if the UE can report before the buffer is full, the network could retrieve the collected data by the UE. In this way, the buffer can always maintain available space for data collection, thereby ensuring the continuity of the collected data.

- However, we don’t think this is a valid motivation. In our understanding, even if the UE reports before the buffer is full, the network may not retrieve the data immediately upon receiving the UE reporting, since the latency requirements on data collection for model training is relaxed. Consequently, it is possible that when the network requests the UE to report collected data, the UE's buffer is already full, and the UE has stopped collecting data. In this case, the data collection process will still be interrupted and the collected data is discontinuous.

- Observation 1: Considering the latency requirements on data collection for model training is relaxed, the network may not retrieve the data immediately upon receiving the data availability indication. Therefore, if the network delays retrieval, the data collection may be stopped because the buffer is full.

- On the other hand, given the UE’s power constraints, continuous data collection cannot be maintained all the time for a certain UE, thus the data interruption is unavoidable. However, the network can collect data from multiple UEs. And the network can aggregate the data from multiple UEs to construct a time-continuous dataset.

- Observation 2: Given the UE's power constraints, continuous data collection is not feasible and interruption is unavoidable, but the network can aggregate data from multiple UEs to construct a time-continuous dataset.

- Based on the above analysis, we think the data availability indication can be reported to the network after the buffer is full. And this will not introduce additional impacts on UE power consumption since the measurement is stopped.

- Proposal 1: The data availability indication can be reported to the network after the buffer is full.

- In legacy logged MDT, the UE can indicate the available measurements via UE-measurementAvailable [2] in msg5, e.g. RRCSetupComplete, RRCResumeComplete, RRCReestablishComplete message.

- For NW data collection for AI/ML, considering the UE keeps in connected mode, msg5 is not applicable for data availability indication reporting. We think UAI can be used to send the data availability indication to the network.

- Proposal 2: The data availability indication can be sent to the network via UEAssistanceInformation massage.

- Issue#2: FFS on time stamp

- In legacy logged MDT reporting, the absolute time info absoluteTimeStamp (48bits, as specified in [2]) when the logged measurement is provided and the relative time info relativeTimeStamp (as specified in [2]) which indicates the time of logging measurement results would be included in UEInformationResponse message.

- In last meeting, RAN2 discussed to used absolute time stamp and relative time stamp for data collection but has no consensus. Some companies indicated that the network can be aware of the time info because the UE is in connected mode, and the continuous samples without exact time info are enough for model training. And this may not be needed for spatial domain. However, we don’t think so considering the following aspects:

- Firstly, RAN2 has agreed to support event-triggered data collection. In this case, the network cannot be aware of when the event is satisfied since it depends on UE’s measurements.

- Secondly, the NW-sided model could be trained centrally by OAM using datasets from multiple gNBs, rather than being trained locally at individual gNBs. In such scenario, OAM would lack awareness of the specific the time information.

- Thirdly, if the UE reports the logged data to a different gNB than the one providing the data collection configuration to the UE, the gNB will not be aware of the configuration and time info of logged data. For example, after a handover procedure, the UE reports the data collected within serving gNB to target gNB, but target gNB does not have the data collection configuration and will miss the time info of logged data.

- Lastly, the cell or beam quality varies at different moments (e.g. due to congestion) rather than remaining static, whether for temporal prediction or spatial prediction. For example, the quality of the same cell or beam will be different at 8 a.m. and 8 p.m..

- Observation 3: The time info is necessary for model training considering the following aspects:

- - For event-triggered data collection, the network cannot be aware of when the event is satisfied

- - For model training at OAM using datasets from multiple gNBs, OAM cannot be aware of the specific the time information

- - For the scenario that UE reports the data collected within serving gNB to target gNB, the target gNB does not have the data collection configuration and cannot cannot be aware of the time info

- - For the cell or beam quality, it can vary at different moments (e.g. at 8 a.m. and 8 p.m.), whether for temporal prediction or spatial prediction

- Based on the above analysis, the time stamp should be included for data collection for model training. The similar principle to logged MDT can be reused, i.e. use absoluteTimeStamp to indicate the absolute time when the data collection configuration is provided, and relativeTimeStamp is used to indicate the time of logged measurement results.

- Proposal 3: The time stamp should be included for training data collection, and the similar principle to logged MDT can be reused, i.e. use absoluteTimeStamp to indicate the absolute time when the data collection configuration is provided, and use relativeTimeStamp to indicate the time of logged measurement results.

- Issue#3: FFS on handing during mobility

- In last meeting, RAN2 agreed that UE retains logged data during handover (HO) and indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message). FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed. And FFS how to handle idle/inactive and RLF cases and whether we have a unified.

- In our understanding, if there are different behaviors for handover, idle/inactive and RLF cases, it may force the network to keep the UE in connected mode. We prefer to have a unified solution to handle handover, idle/inactive and RLF cases.

- Proposal 4: It is proposed to have a unified solution to handle handover, idle/inactive and RLF cases.

- From the perspective of operator, if the UE is configured to collect data, it is not expected to release autonomously by the UE since this can lead to the waste of UE power consumption. Therefore, the UE can retain logged data when entering to idle/inactive mode or when RLF occurs, and then can indicate availability of logged data within RRCSetupComplete, RRCResumeComplete, and RRCreestablishmentComplete message.

- Proposal 5: The UE can retain logged data when entering to idle/inactive mode or when RLF occurs, and then can indicate data availability within RRCSetupComplete, RRCResumeComplete, and RRCreestablishmentComplete message.

- On the other hand, considering the flexibility of network control, we think an indication can be provided to the UE to retain or release the logged data during handover, state transition to idle/inactive mode, or RLF case. Specifically, the indication can be included in HandoverCommand, RRCRelease, or RRCReconfiguration message. And if the indication is not provided, the UE retains the logged data by default.

- Proposal 6: The indication can be provided to the UE to retain or release the logged data within  HandoverCommand, RRCRelease, or RRCReconfiguration message.

- In logged MDT, the UE will start the 48 hour timer when the UE memory reserved for MDT is full, as specified in TS 37.320 [3]. That is, the UE retains the logged data for up to 48 hours after the buffer is full. We think the similar principle can be reused for training data collection.

- Proposal 7: The UE retains the logged data for up to 48 hours after the buffer is full.

- Issue#4: Data type for NW sided model and UE sided model

- During the study phase, RAN1 reply LS [4] provided the data content and typical data size for beam management use case. It can be seen that they are common for UE-sided model and NW-sided model training, but different for inference and monitoring. More details can be further discussed based on RAN1 progress.

| LCM purpose | UE-side/NW-side models | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | UE-side, NW-side | L1-RSRPs and/or beam-IDs | See Note 1 for L1-RSRPs | Relaxed |  |
| Inference | UE-side | Beam prediction results | Small (10s of bits) | Time-critical | RAN1 has agreed to consider L1 signalling for this reporting |
|  | NW-side | L1-RSRPs, and Beam-IDs if needed, for Set B | See Note 1 for L1-RSRPs | Time-critical |  |
| Monitoring | UE-side | Event occurrence and/or calculated performance metrics (from UE to NW) See Note 4 | Small (10s of bits) | Near-real-time |  |
|  | UE-side | L1-RSRP(s) and/or beam-ID(s) See Note 4 | Up to 10 bits, or up to 100 bits, or up to hundreds of bits. See Note 1 for L1-RSRPs | Near-real-time |  |
|  | NW-side | L1-RSRP(s) and/or beam-ID(s) See Note 4 | Up to 10 bits, or up to 100 bits, or up to hundreds of bits. See Note 1 for L1-RSRPs | Near-real-time |  |

- Proposal 8: The data content is common for UE-sided model and NW-sided model training, but different for inference and monitoring. More details can be further discussed based on RAN1 progress.

- Issue#5: FFS on memory size

- RAN2 agreed that UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. In legacy logged MDT and QoE measurement, the minimum AS layer memory size is 64KB [5].

- TS 38.306, Section 4.2.18 Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No. - TS 38.306, Section 6 Features; Condition. AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports".

- For NW data collection, considering there are multiple use cases which needs training data, we think the legacy value is baseline and other larger value is preferred, i.e. the minimum AS layer memory size is at least 64KB.

- Proposal 9: The minimum AS layer memory size for training data logging is at least 64KB.

# 3	Conclusion

Here are the proposals for NW side data collection.

FFS on data availability indication reporting

- Observation 1: Considering the latency requirements on data collection for model training is relaxed, the network may not retrieve the data immediately upon receiving the data availability indication. Therefore, if the network delays retrieval, the data collection may be stopped because the buffer is full.

- Observation 2: Given the UE's power constraints, continuous data collection is not feasible and interruptions are unavoidable, but the network can aggregate data from multiple UEs to construct a time-continuous dataset.

- Proposal 1: The data availability indication can be reported to the network after the buffer is full.

- Proposal 2: The data availability indication can be sent to the network via UEAssistanceInformation massage.

- FFS on time stamp

- Observation 3: The time info is necessary for model training considering the following aspects:

- - For event-triggered data collection, the network cannot be aware of when the event is satisfied

- - For model training at OAM using datasets from multiple gNBs, OAM cannot be aware of the specific the time information

- - For the scenario that UE reports the data collected within serving gNB to target gNB, the target gNB does not have the data collection configuration and cannot cannot be aware of the time info

- - For the cell or beam quality, it can vary at different moments (e.g. at 8 a.m. and 8 p.m.), whether for temporal prediction or spatial prediction

Proposal 3: The time stamp should be included for training data collection, and the similar principle to logged MDT can be reused, i.e. use absoluteTimeStamp to indicate the absolute time when the data collection configuration is provided, and use relativeTimeStamp to indicate the time of logged measurement results.

FFS on handing during mobility

- Proposal 4: It is proposed to have a unified solution to handle handover, idle/inactive and RLF cases.

- Proposal 5: The UE can retain logged data when entering to idle/inactive mode or when RLF occurs, and then can indicate data availability within RRCSetupComplete, RRCResumeComplete, and RRCreestablishmentComplete message.

Proposal 6: The indication can be provided to the UE to retain or release the logged data within  HandoverCommand, RRCRelease, or RRCReconfiguration message.

Proposal 7: The UE retains the logged data for up to 48 hours after the buffer is full.

Data type for NW sided model and UE sided model

- Proposal 8: The data content is common for UE-sided model and NW-sided model training, but different for inference and monitoring. More details can be further discussed based on RAN1 progress.

FFS on memory size

Proposal 9: The minimum AS layer memory size for training data logging is at least 64KB.

# 4	Reference

- Draft_R2_129_Meeting_Report_v3

- TS 38.331 NR; Radio Resource Control (RRC) protocol specification

- TS 37.320 Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2

- R1-2310681 Reply LS on Data Collection Requirements and Assumptions

- TS 38.306 NR; User Equipment (UE) radio access capabilities