3GPP TSG-RAN WG2 Meeting #129b	R2-2502742

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.5.2

Source:	Panasonic

Title:	On-demand SSB SCell operation

Document for:	Discussion/Decision

Introduction

In this contribution, we share our views on configuration of OD-SSB and L3 measurement.

Discussion

  - OD-SSB configuration

Following was agreed in RAN2#127.

Agreements on OD-SSB SCell 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. 2. Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. 3. NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Referring to RAN1 progress, multiple candidate values need to be configured by RRC.

Agreement in RAN1#119 For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer Agreement in RAN1#119 - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives. - Alt A: Same as for Case #1 - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) Agreement in RAN1#120 - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists

- RRC configuration of OD-SSB

For OD-SSB configuration, sCellConfigCommon and sCellConfigDedicated can support multiple candidate values of OD-SSB parameters. In RRC to initial configuration, states (activation/ deactivation) are indicated. An Scell may have multiple OD-SSBs, therefore, OD-SSB configuration ID is useful and from OD-SSB configuration ID, UE can indicate any parameter that RAN1 concludes to support multiple candidate values.

- Activation/Deactivation by RRC

OD-SSB is used for L1/L3 measurements. We consider OD-SSB triggering timing is up to network implementation irrespective of UE situation. Even after the SCell is configured on-demand SSB may also be further triggered and indicated by network, even if the “always-on” SSB in Case 2 is already available. Therefore, independent signaling from Scell activation is also useful. For RRC, RAN2 agreed that no need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. SCell modification indicates activation/deactivation of OD-SSB.

- Activation/Deactivation by MAC

For MAC CE, whether MAC CE has a fixed size includes a bitmap with activation/deactivation and/or flexible size was discussed. On the other hand, RAN1 agreed “Configuration/indication of the number N of on-demand SSB bursts”. When this implicit deactivation is configured, fixed size of MAC CE with activation/deactivation is not suitable since indication of deactivated is not necessary for the OD-SSB with N value. For activation, if MAC CE indicates N values, the field for N values is necessary. Therefore, there would be two options.

Option1: Fixed size MAC CE with activation/deactivation for all candidates OD-SSB ID is applicable for explicit indication. Flexible size MAC CE is applicable for activation of the number N of OD-SSB bursts.

Option2: Flexible size MAC CE is used for activation and deactivation. MAC CE indicates only for the activated or deactivated OD-SSB ID.

If a UE is configured one of explicit indication of OD-SSB and N of OD-SSB burst, for a UE, option 1 has no confusion. However, two type of MAC CE should be specified. Option 2 has merit that MAC CE for OD-SSB can be common for explicit indication of OD-SSB and N of OD-SSB. If N values are indicated by MAC CE, the field of N values can be zero for explicit deactivation. Therefore, we propose Option 2.

Proposal 1: sCellConfigCommon and sCellConfigDedicated support multiple candidate values of OD-SSB parameters.

Proposal 2: Scell modification indicates activation/deactivation of OD-SSB.

Proposal 3: The new MAC-CE for OD-SSB is flexible size and common for explicit indication of OD-SSB and N of OD-SSB burst. MAC CE includes OD SSB configuration ID.

  - L3 RRM

In RAN2#127bis meeting, the L3 RRM framework on OD-SSB measurement has been discussed and two options are listed as follows.

Option1: Based on different measurement configuration when OD-SSB is transmitted. Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted.

In RAN2#129 meeting, following was agreed for SSB adaptation and reply LS with RAN2 agreement [1].

Agreements on SSB adaptation 1. RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell. 2. RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. 3. For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

RAN1 agreed follows.

Agreement RAN1#120 Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

As same as SSB adaptation, we propose to keep SMTC based L3 RRM framework and to introduce additional SMTC. For same frequency case, a SMTC window could cover both always on SSB and OD-SSB. However, longer/dense SMTC window would restrict the network scheduling flexibility and would lead to more UE power consumption for measurement. Therefore, configuration of two SMTCs would be reasonable. It can support different periodicity for always on SSB and OD-SSB. In the current specification, up to 4 SMTCs can be configured. As similar as smtc2, smtc2-LP, smtc3list and smtc4list, additional smtc can be configured for OD-SSB. It could be additional UE capability. When UE has capability to support additional SMTC for OD-SSB, UE measures both always on SSB and OD-SSB. If UE doesn’t have this capability, SMTC is configured for one of among 1) always on SSB, 2) OD-SSB or 3) one SMTC cover both SSBs with longer/dense window.

For different frequency case, different SMTCs are necessary to measure different SSBs. When UE doesn’t have the capability to support additional SMTCs, one of always on SSB or OD-SSB should be measured at one time. Even if additional SMTC is supported, RAN1 assumes UE is not required to measure both AO-SSB and OD-SSB.  When one of SMTC is active in a time, SMTC patterns can be configured for not to be overlapped in time domain.

Proposal 4: For same frequency of always on SSB and OD-SSB, additional SMTC can be configured for OD-SSB depending on the UE capability.

Proposal 5: For different frequency of always on SSB and OD-SSB, additional SMTC can be configured for OD-SSB depending on the UE capability. When one of SMTC is active in a time, SMTC patterns are configured as not to be overlapped in time domain.

Conclusion

- Based on the discussion, the following proposals are highlighted:

Proposal 1: sCellConfigCommon and sCellConfigDedicated support multiple candidate values of OD-SSB parameters.

Proposal 2: Scell modification indicates activation/deactivation of OD-SSB.

Proposal 3: The new MAC-CE for OD-SSB is flexible size and common for explicit indication of OD-SSB and N of OD-SSB burst. MAC CE includes OD SSB configuration ID.

Proposal 4: For same frequency of always on SSB and OD-SSB, additional SMTC can be configured for OD-SSB depending on the UE capability.

Proposal 5: For different frequency of always on SSB and OD-SSB, additional SMTC can be configured for OD-SSB depending on the UE capability. When one of SMTC is active in a time, SMTC patterns are configured as not to be overlapped in time domain.

Reference

[1] R2-2501484 Reply LS on SSB adaptation