3GPP TSG RAN WG2 Meeting #129bis	 R2-2502753

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.2.2

Source: 	ITL

Title: 	Discussion on A-IoT paging

Document for:	Discussion and decision

Introduction

In the previous RAN2 meeting [1], agreements related to A-IoT paging are as follows:

Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

In this contribution, we discussed the multiple service requests for A-IoT.

Discussion

In RAN2#129 meeting, RAN2 discussed the scenario in which a device might receive multiple paging messages related to different service requests, either from the same reader or from different readers. While current agreements state that parallel service requests from the same reader are not supported, unexpected conditions in real deployments may result in simultaneous paging from multiple sources.

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.  FFS device behaviour if multiple requests are received in parallel (if needed).

While the above agreements are assumed in system design, they do not guarantee that multiple paging messages won’t be received in practice. Therefore, the need for device behavior in such cases is considered as FFS.

Device behavior if multiple service requests are received in parallel

In this section, we show our view on whether to distinguish different service request from same reader or different readers.

- Device Simplicity: Advocate for keeping device behavior simple, avoiding the need to distinguish between readers.

- Transaction ID-based Handling: Devices should use transaction ID to determine if a service request is new or part of an ongoing procedure.

- Reader Coordination via Implementation: Reader-level coordination (e.g., time-domain separation) is assumed to prevent concurrent paging.

- No Introduction of Reader ID: Introducing a Reader ID in paging messages is seen as unnecessary and potentially burdensome for low-capability devices.

Parallel service requests are not supported by design, so there is no need for the device to distinguish between different readers. Introducing reader identification would add unnecessary complexity, signaling overhead, and cost, especially for low-complexity A-IoT devices. Devices can rely solely on the transaction ID to determine whether a service request is new or ongoing. Scenarios involving multiple readers sending simultaneous requests are considered rare or avoidable through proper network deployment and implementation strategies (e.g., time-domain coordination, power control, topology planning). Potential interference issues caused by overlapping coverage from different readers should be addressed at the reader/network side, not by adding complexity to the device. Thus, in Release 19, there is no requirement for the device to distinguish whether a paging request comes from the same or a different reader. Unless clear use cases emerge in future releases, support for such differentiation at the device level is considered unnecessary and inefficient.

Proposal 1: Devices should not be required to distinguish between different service requests from the same or different readers

Also, we show our view what would be the device behavior if another (different) service request is received from the same reader while there is one ongoing procedure.

- One procedure at a time (as per RAN2 agreement): The device is expected to handle only one service procedure at a time. Accepting a new request would violate this.

- Implementation simplicity: Especially for low-complexity A-IoT devices, this behavior simplifies logic and state handling.

- Transaction ID-based filtering: The device can simply discard any paging message that carries a different transaction ID while one procedure is active.

- Avoidance of unexpected behaviors: Prevents unintended switches or overlaps in procedures, ensuring robust and deterministic operation.

Therefore, we think that the device maintains single-procedure integrity by discarding any subsequent service requests during an ongoing session

Proposal 2: If device gets a new service request while one procedure is still ongoing, ignore all new requests

For the same service request, we think that A-IoT devices should not be required to distinguish whether the same service request (i.e., with the same transaction ID) originates from the same reader or a different reader. Devices already use transaction ID to identify and manage service requests. If the same transaction ID is reused, it should represent the same service request regardless of reader origin. It is up to the CN and reader-side implementation to ensure proper coordination (e.g., avoiding duplicate paging collisions, controlling reader coverage.

However, the device should determine its behavior based on whether it has already successfully responded to repeated service request.

- If the device has successfully responded to the service request, it should ignore any further paging messages carrying the same transaction ID.

- If the device has not successfully responded (e.g., due to failure or interruption), it may reattempt access upon receiving a repeated paging message with the same transaction ID

Proposal 3: The device should determine its behavior based on whether it has already successfully responded to repeated service request

Transaction ID

In multi-reader scenarios, the same service request (e.g., for proximity or location) may be sent by multiple readers. For example, The core network (CN) distributes a single A-IoT service request (e.g., for device location or inventory scan) to multiple readers. Each reader, unaware of the others, independently pages the same device with the same service request.

In this case, we show our view on our opinion on whether additional reader-specific information (e.g. reader ID) is required.

Transaction ID is derived from the CN correlation ID, which is unique per service request. This ensures that the device can reliably recognize repeated paging messages for the same service, regardless of which reader sends them.

Therefore, we think that the device can determine whether a received service request is new or already handled based solely on the transaction ID. If the device has already successfully responded to a request with that transaction ID, it shall ignore subsequent paging messages carrying the same transaction ID.

Proposal 4: No need to provide Reader ID to the device

Conclusion

In this contribution, we discussed the A-IoT paging. According to discussion in section 2, we have the following proposals:

Proposal 1: Devices should not be required to distinguish between different service requests from the same or different readers

Proposal 2: If device gets a new service request while one procedure is still ongoing, ignore all new requests

Proposal 3: The device should determine its behavior based on whether it has already successfully responded to repeated service request

Proposal 4: No need to provide Reader ID to the device

Reference

RAN2#129 meeting report