3GPP TSG-RAN WG2 Meeting #129bis	R2-2502761

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.5.3

Source:	OPPO

Title:	Consideration on on-demand SIB1

Document for:	Discussion, Decision

Introduction

This paper outlines our further views on on-demand SIB1, which falls under the following objective.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

Discussion

Handling of UEs in RRC_CONNNECTED

In the legacy system, there are two situations where the connected UE needs to acquire SIB1: one is when the UE experiences RLF, and the other is when the UE requires OSI. If searchSpaceSIB1 is not configured for the latter case, the UE relies on the network-dedicated RRC to provide SIB1. Conversely, if searchSpaceSIB1 is configured, the UE is able to monitor the broadcasted SIB1.

[38.331] Clause 5.2.3.1 The UE shall: 1> apply the specified BCCH configuration defined in 9.1.1.1; 1> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 1> if the UE is in RRC_CONNECTED while T311 is running: 2> acquire the MIB, which is scheduled as specified in TS 38.213 [13]; 2> if the UE is unable to acquire the MIB; 3> perform the actions as specified in clause 5.2.2.5; 2> else: 3> perform the actions specified in clause 5.2.2.4.1. 1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or 1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or 1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or 1> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 1> if the UE is in RRC_CONNECTED while T311 is running: 2> if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE: 3> acquire the SIB1, which is scheduled as specified in TS 38.213 [13]; 3> if the UE is unable to acquire the SIB1: 4> perform the actions as specified in clause 5.2.2.5; 3> else: 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2. 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> perform the actions as specified in clause 5.2.2.5.

Since the connected UE might be a Rel-19 NES UE, and the cell it is connected to may not currently broadcast SIB1, RAN2 must determine how this UE gets SIB1. In RAN2#129, RAN2 has agreed to apply the existing principle for the scenarios lacking searchSpaceSIB1, while leaving an FFS regarding scenarios involving searchSpaceSIB1.

Proposal 5: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, rely on the NW implementation to deliver SIB1 as baseline. [Vivo]: NW cannot know when the UE wants to request OSI, then proposal 5 cannot work in the case. [Ericsson]: Want to have more time to think of two options. - We will make conclusion on P5 next meeting.

To address this issue, we understand the following options to be explored, with a preference for Option 1 and Option 2.

Option 1: Utilize the network-dedicated RRC to deliver SIB1.

  - This approach allows for a unified solution for the connected UEs in case T311 is not running.

  - The gNB may need to resend SIB1 for each modification period.

Option 2: Have the UE obtain SIB1 via the Msg-1 based on-demand SIB1 procedure.

  - This approach allows for a unified solution applicable to UEs in all RRC states.

  - The UE cannot perform this procedure if it works on one BWP without RACH configuration.

Option 3: Request OSI without acquiring the NES cell’s SIB1 to verify si-BroadcastStatus.

  - This means the UE can bypass SIB1 checks, but it may lead to increased signalling overhead for unnecessary OSI requests.

For Rel-19 NES UE in RRC_CONNECTED while T311 is not running, if searchSpaceSIB1 is configured, rely on the NW dedicated RRC for SIB1 delivery, or rely on the UE to acquire SIB1 via the on-demand SIB1 procedure used in RRC_IDLE/RRC_INACTIVE.

Open issues in the running CR

Several open issues are raised during the CR discussion, including,

- FFS whether any changes are required for handling preamble backoff during SIB1 request procedure.

- FFS whether any changes are required for RAR handling when an RO for SIB1 request is shared with other RA usages.

- Whether any changes are required for when the UE can select any SSB for SIB1 request in case there are no SSBs measured above rsrp-SIB1ThresholdSSB is available, dependent on RAN1.

For the first issue, we understand that the RO for SIB1 request can be shared with other RA usages. At the time of sending RAR MAC PDU for a given RA-RNTI, the backoff subheader can be included in this RAR MAC PDU, thus the UE requesting SIB1 needs to consider how to set the backoff value for the next RACH attempt for the SIB1 request. The most straightforward way is to use the legacy mechanism to avoid RA collision. Note that the SI request also follows this legacy rule.

1> else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded: 2> if the Random Access Response contains a MAC subPDU with Backoff Indicator: 3> set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI. 2> else: 3> set the PREAMBLE_BACKOFF to 0 ms.

Reuse the legacy preamble backoff mechanism of the SI request for the SIB1 request procedure as well.

For the second issue, some companies observe that Msg2 transmitted by the NES cell may include SIB1 request ACK for the NES UE and RAR feedbacks for other UEs in this NES cell, because the RO for SIB1 request can be shared with other RA usages. As the NES UE lacks the preamble information for other RA usage, due to the TotalNumberOfRA-Preambles not being included in the UL-WUS configuration, the NES UE may not know how to decode the SIB1 request ACK in the MAC PDU.

Conclusion - The parameter “ssb-PeriodicityServingCell” is not included in the UL-WUS configuration for FDD system. - RA-RNTI is not included in the UL-WUS configuration. - Cell barring information is not included in the UL-WUS configuration. - Cell selection information is not included in the UL-WUS configuration. - Accessible UE types is not included in the UL-WUS configuration. - CarrierBandwidth (for MPR calculation) is not included in the UL-WUS configuration. - InitialUplinkBWP is not included in the UL-WUS configuration. - TotalNumberOfRA-Preambles is not included in the UL-WUS configuration. - SIB1-RequestResourceRedCap is not included in the UL-WUS configuration. - SupplementaryUL is not included in the UL-WUS configuration

To resolve this issue, the simplest way is to contain TotalNumberOfRA-Preambles in the WUS configuration.

If R2 tends to solve the issue of RAR handling when an RO for SIB1 request is shared with other RA usages, contain TotalNumberOfRA-Preambles into the WUS configuration.

For the third issue, some companies mention that RAN1 has not decided whether all the SSBs will be configured with OD-SIB1 RA resources. If not, reusing the legacy SSB selection procedure (i.e., select any SSB) is not applicable.

1> else (i.e. for the contention-based Random Access preamble selection): 2> if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available: 3> select an SSB with SS-RSRP above rsrp-ThresholdSSB. 2> else: 3> select any SSB.

We understand this issue but suggest restricting the network configuration to minimize the spec impact, i.e. the UE does not expect one SSB to be present but without the corresponding WUS configuration.

Reuse legacy SSB selection procedure in MAC spec, i.e., UE does not expect one SSB is present but without corresponding WUS configuration.

Another issue is how SUL co-exists with OD-SIB1 request procedure. In the legacy, the UE needs to select SUL or NUL during the RA procedure initialization.

1> if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL: 2> select the SUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the SUL carrier. 1> else: 2> select the NUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the NUL carrier.

As RAN1 has agreed that SupplementaryUL is not included in the UL-WUS configuration and the current RRC running CR does not include rsrp-ThresholdSSB-SUL (for which the main reason is that R1 tends to see the typical case of OD-SIB1 is for small cell, and thus SUL is not the target scenario for OD-SIB1), the UE should always select NUL when the UE requests SIB1 of an NES cell, by assuming network would not configure SUL for the NES cell of OD-SIB1 feature.

For OD-SIB1, based on R1 conclusion, no specific handling for SUL is needed.

Conclusion

We have the following proposals:

Proposal 1	For Rel-19 NES UE in RRC_CONNECTED while T311 is not running, if searchSpaceSIB1 is configured, rely on the NW dedicated RRC for SIB1 delivery, or rely on the UE to acquire SIB1 via the on-demand SIB1 procedure used in RRC_IDLE/RRC_INACTIVE.

Proposal 2	Reuse the legacy preamble backoff mechanism of the SI request for the SIB1 request procedure as well.

Proposal 3	If R2 tends to solve the issue of RAR handling when an RO for SIB1 request is shared with other RA usages, contain TotalNumberOfRA-Preambles into the WUS configuration.

Proposal 4	Reuse legacy SSB selection procedure in MAC spec, i.e., UE does not expect one SSB is present but without corresponding WUS configuration.

Proposal 5	For OD-SIB1, based on R1 conclusion, no specific handling for SUL is needed.

Reference

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR