3GPP TSG-RAN WG2 #129bis	R2-2502766

Wuhan, China, April 7th– 11th, 2025

Agenda Item:	8.5.4

Source:	Ericsson

Title:	Adaptation of common signal/channel transmissions for NES

Document for:	Discussion

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised in [4]. In this paper, we address the third objective given in the revised WID as:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we discuss the following aspects:

- adaptation of SSB signals,

- adaptation of PRACH occasions and related signaling, and

- adaptation of paging.

# 2	Discussion

### 2.1 Adaptation of SSB signals

RAN2 made the following agreement in the last meeting:

- RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

and the following agreement was made in RAN1#120:

Agreement

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

- Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

- FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1)

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

RAN1 has agreed to support DCI signalling for switching SSB burst periodicity. It is up to RAN2 to decide on whether there is a need to introduce signalling via MAC layer, similar to the signalling introduced for OD-SSB, for this switching mechanism.

RAN1 has agreed to support DCI signalling for switching SSB burst periodicity.

We think there is no need introduce multiple signalling for activation/deactivation, so RAN2 can confirm that DCI signalling would be sufficient for switching SSB burst periodicity.

No need to introduce activation/deactivation MAC CE for switching SSB burst periodicity.

In the last RAN2 meeting, it was also agreed to keep SMTC based L3 RRM framework and introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. SMTC is configured per frequency and network may configure multiple SMTC windows to cover SSBs on the same frequency but transmitted with different periodicities. Assuming that a gNB would configure the active (present) SSBs for SCells via ssb-ToMeasure and the associated rate/periodicity via SMTC, which defines the time window during which the UE can measure SSBs that belong to these SCells, the additional SMTC configuration covers adapted SSB transmissions. That is, in addition to the two SSB configurations and the signalling for activation/deactivation, SMTC configuration for a UE needs to be dynamically adapted accordingly.

Dynamic adaptation of SSB transmissions in SCells cells needs to be accompanied by corresponding adaptation of SMTC.

Introduce mechanism for SMTC adaptation.

### 2.2 Adaptation of PRACH occasions and related signaling

The network can activate time domain RA adaptations for UEs in RRC connected or idle/inactive modes dynamically. As agreed in the last meeting, this would be backwards compatible as legacy UEs will use legacy RA resources per legacy configuration.

Activation of additional PRACH occasions dynamically is supported for UEs in connected and idle/inactive modes.

Dynamic activation of additional RA resources can be achieved via system information update. However, this would be a slow mechanism, and it requires to wake up UEs unnecessarily. For idle/inactive UEs, activation can be indicated at paging occasions (POs), in paging DCI, i.e., DCI 1_0 with P-RNTI, and/or occasions where SIB1 transmissions are made with the assumption that UEs that receive a paging message or acquire SIB1 prior to triggering RA can utilize those additional RA occasions.

For UEs in idle/inactive mode additional RA occasions can be activated at paging and/or SIB1 transmission occasions.

If paging DCI supports an activation mechanism for additional RA resources for both idle/inactive and connected mode UEs, this simplifies the mechanism and improves efficiency as same signaling would be used.

A unified signalling method for activating additional RA resources for both idle/inactive and connected UEs simplifies the dynamic control and improves efficiency by utilizing same signalling for both cases.

However, it may be beneficial to separate the indication for the activation of additional RA resources for UEs in idle/inactive mode and UEs in connected mode. For UEs in connected mode another random-access event that needs to be considered is the one triggered by the PDCCH order. The network may send the PDCCH order to a UE in connected mode when there is pending data in the DL, but the UE is out-of-sync. It may be beneficial if the network activates additional random-access occasions so that the UE is in sync again quickly. In that case, it would be better if additional random-access resources were activated for that UE with a dedicated indication. This may be achieved with an indication introduced in the dedicated PDCCH order signaling for that UE, e.g., the message scrambled with C-RNTI.

For UEs in connected mode, additional RA occasions can be activated at paging and/or, SIB1 transmission occasions, and/or when PDCCH order is transmitted.

For UEs in idle/inactive or connected mode, it should also be possible to activate additional RA occasions at any time. This would simply require introducing a bit/flag along with the configuration of additional RA occasions so that network can active those occasions via system information update. This is of course not as dynamic as DCI signaling.

Additional RA occasions can be activated at any time via system information update

Assuming that adaptive RA configuration includes multiple RA transmission occasions, a Rel-19 NES UE may choose any occasion to transmit a PRACH preamble. RAN2 agreed on introducing adaptive paging occasions and as discussed above we think that it would be beneficial for UEs in idle/inactive mode if additional RA occasions are activated at paging occasions. This is since it is very likely that the paged UE, if it is camping in that cell, will trigger the random-access procedure. When paging occasions are adapted and thus become more condensed, it is possible that multiple UEs are paged within a short period of time. These UEs would then trigger the random-access procedure almost simultaneously assuming that a UE typically initiates network access via the closest possible RA occasion. To avoid collusion in such cases it would be beneficial if gNB assigns a particular instance of the activated adaptive RA resources when paging a UE so that the load due to random access can be balanced. Please see the figure below.

It would be beneficial if gNB assigns a particular instance of the activated adaptive RA resources when paging a UE to balance the load.

…

Adaptive RA resources

gNB assigns the RA resources to the paged UE

Time

SSB transmission

Paging frames/occasions

Figure 1: Assignment of adaptive RA resources

Note that such assignment, i.e., regarding which additional RA resource should be used, does not impact the performance from latency standpoint considering that the UE is paged ahead of its otherwise legacy paging occasion, when configured with adaptive paging occasions.

An activated adaptive RA resource can be assigned to an adaptive paging frame/occasion.

### 2.3 Adaptation of paging

In this section we discuss the remaining aspects concerning the signalling overhead associated with N and Ns parameters, and the aspects concerning the co-existence of PEI and paging adaptations.

### 2.3.1 Signalling overhead and extended values of N and Ns parameters

Below, we evaluate the message size when Ns for Rel-19 UEs is set to 8 and compare the outcome with the message size of legacy configuration as baseline. In the following example, for the sake of evaluation, we assume a SIB1 configuration with hardly anything but paging configuration in all scenarios.

- Legacy baseline scenario: T = 128, N = T/16, Ns = 4 (i.e., every 16th RF is a PF, and each PF is associated with 4 POs), and four instances of firstPDCCH-MonitoringOccasionOfPO parameter is configured for SCS = 15 kHz (i.e., corresponds to the case sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO). This scenario is illustrated in Figure 2.

Figure 2: Legacy baseline scenario (T = 128, N = T/16, Ns = 4, and SCS =15 kHz).

- Option b) scenario: T = 128, N = T/16, for legacy UEs Ns = 4 and for Rel-19 UEs Ns = 8 (i.e., every 16th RF is a PF, and each PF is associated with 4 POs for legacy UEs and 8 POs for Rel-19 UEs). We consider two cases with SCS = 15 kHz (i.e., corresponds to the case sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO) and SCS = 480 kHz (i.e., corresponds to the case sCS480KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO). This scenario is illustrated in Figure 3.

Figure 3:  Option b) scenario (T = 128, N = T/16, Ns for legacy = 4, Ns for Rel-19 UEs = 8 and SCS =15 kHz).

In Table 1, we provide the message sizes for the scenarios above. We assumed that the firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs are defined in the same way as in legacy (i.e., the range of firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs is given in symbols).

Table 1: Message sizes for different paging configurations (symbol level configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs)

|  | Message size [octets] |
| --- | --- |
| Legacy T = 128, N = T/16, Ns = 4, SCS = 15 kHz (baseline) | 28 |
| Option b) T = 128, N = T/16, Ns_leg = 4, Ns_r19 = 8, SCS = 15 kHz | 41 |
| Option b) T = 128, N = T/16, Ns_leg = 4, Ns_r19 = 8, SCS = 480 kHz | 50 |

According to the results above in Table 1, option b) with SCS = 15 kHz and option b) with SCS = 480 kHz have 13 and 22 octets larger message size than the legacy (baseline) configuration, respectively. This accounts for approximately 46% and 79% increase in message size. One can observe that option b) introduces significant signalling overhead even for the case where Ns for Rel-19 UEs is set to 8, according to the latest RAN2 agreements.

Option b) introduces significant signalling overhead (~46% larger message size than the respective legacy configuration with T = 128, N = T/16, Ns for legacy = 4, SCS = 15 kHz) even for the case where Ns for Rel-19 UEs is set to 8.

Based on the results in Table 1 above, we think that it would be beneficial to reduce the impact of option b) on the message size. This can be simply achieved by defining the range of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level instead of symbol level as currently specified for legacy UEs.

Note that we are not proposing to change the meaning of firstPDCCH-MonitoringOccasionOfPO, but its resolution only (i.e., the unit of firstPDCCH-MonitoringOccasionOfPO parameter would still be monitoring occasions).  This would allow us to define the range of the firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs to be the same as of the firstPDCCH-MonitoringOccasionOfPO parameters for legacy UEs but with fewer bits.

In Table 2, we summarize the evaluation for message sizes for two scenarios with option b) in which firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs are defined at the slot level.

Table 2: Message size for different paging configuration scenarios (slot level configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs)

|  | Message size [octets] |
| --- | --- |
| Option b) T = 128, N = T/16, Ns_leg = 4, Ns_r19 = 8, SCS = 15 kHz | 37 |
| Option b) T = 128, N = T/16, Ns_leg = 4, Ns_r19 = 8, SCS = 480 kHz | 46 |

Based on the results in Table 2 above, the configuration of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level results in 4 octets smaller message size than the configuration of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at symbol level, which corresponds to approximately 31% reduction of message size.

The configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level results in 4 octets (~31%) smaller message size than the configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at symbol level.

For Rel-19 NES UEs, firstPDCCH-MonitoringOccasionOfPO is configured at slot level.

Assuming that firstPDCCH-MonitoringOccasionOfPO parameter is configured at slot level for Rel-19 NES UEs, the “FFS on firstPDCCH-MonitoringOccasionOfPO for paging adaptations” from [5], can be addressed in the ASN.1 as follows:

firstPDCCH-MonitoringOccasionOfPOExt-r19   CHOICE {

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZoneThirtySecondT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..319),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZoneThirtySecondT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..639),

sCS480KHZquarterT-SCS120KHZoneSixteenthT-SCS60KHZoneThirtySecondT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1279),

sCS480KHZoneEighthT-SCS120KHZoneThirtySecondT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2559),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..5119)

sCS480KHZoneThirtySecondT  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..10239)

}                                                                                OPTIONAL  -- Need R

Regarding the “FFS on field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions” captured in [5], we observe that for all possible values of ssb-periodicityServingCell in the field description of pagingAdaptationNAndPagingFrameOffset IE, N can also be set to oneThirtySecondT value.

For all possible values of ssb-periodicityServingCell in the field description of pagingAdaptationNAndPagingFrameOffset IE, the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) can be set to oneThirtySecondT value.

In [6], the ASN.1 change related to the “FFS on field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions” can be captured as highlighted below:

pagingAdaptationNAndPagingFrameOffset Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]). A value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on. If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 2 or 3 (as specified in TS 38.213 [13]): - for ssb-periodicityServingCell of 5 or 10 ms, N can be set to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 20 ms, N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 40 ms, N can be set to one of {quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 80 ms, N can be set to one of {oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 160 ms, N can be set to {oneSixteenthT, oneThirtySecondT} If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 1 (as specified in TS 38.213 [13]), N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT } If pagingSearchSpace is not set to zero, N can be configured to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT}

### 2.3.2 Paging adaptations and PEI

When it comes to PEI configuration parameters, let us recall the following from the TS 38.331:

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...

}

PEI-Config field descriptions

payloadSizeDCI-2-7 Payload size of PEI DCI, i.e., DCI format 2_7. The size is no larger than the payload size of paging DCI which has maximum of 41 bits and 43 bits for licensed and unlicensed spectrums, respectively.

pei-FrameOffset Offset, in number of frames from the start of a reference frame for PEI-O to the start of a first paging frame of the paging frames associated with the PEI-O, see TS 38.213 [13], clause 10.4A.

po-NumPerPEI The number of PO(s) associated with one PEI monitoring occasion. It is a factor of the total PO number in a paging cycle, i.e N x Ns, as specified in TS 38.304 [20]. The maximum number of PF associated with one PEI monitoring occasion is 2. The number of PO mapping to one PEI should be multiple of Ns when po-NumPerPEI is larger than Ns.

maxPEI-perPF-r17                        INTEGER ::= 4       -- Maximum number of PEI occasion per paging frame

Furthermore, let us recall the following from Section 7.21 in TS 38.304:

“If one PEI-O is associated with POs of two PFs, the two PFs are consecutive PFs calculated by the parameters PF_offset, T, Ns, and N.”

According to the current specifications, max # of paging occasions per PF is four, max # of PFs associated with one PEI monitoring occasions is two, and if one PEI occasion is associated with POs of two PFs, the two PFs are consecutive PFs.

Current specifications need to be updated in order to support co-existence of PEI and paging adaptations, i.e. Ns = 8.

For Rel-19 PEI, increase the maximum number of paging occasions per PF to 8.

Regarding the following agreement from the last meeting: “Introduce a separate PEI configuration”, even though it confirms that a separate configuration is introduced for Rel-19 NES-capable UEs, it does not capture how PEI transmissions are performed. We interpret this as separate PEI transmission is intended for these UEs. Therefore, from UE perspective, i.e., legacy/Rel-19 NES UEs, different PEI occasions may be used to indicate legacy POs and additional Rel-19 NES POs. This is shown in the example in Figure 4 according to which UEs assigned to legacy POs monitor for PEI in legacy PEI occasions, while UEs assigned to additional POs monitor for additional PEI occasions. Note that each UE (legacy/Rel-19 NES) monitors only one PEI occasion (legacy or Rel-19 NES), regardless of whether some legacy and Rel-19 additional paging occasions overlap. This is also in line with the agreement from RAN2#125bis:

“From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.”

The separation of additional Rel-19 PEI transmission occasions from legacy ones can be achieved by configuring PEI-PO-Offset and/or pei-SearchSpace differently (including firstPDCCH-MonitoringOccasionOfPEI-O) for the legacy and Rel-19 configuration.

Figure 4: An example how different PEI occasions may be used to address legacy POs and the additional Rel-19 POs.

Rel-19 PEI independently addresses the additional Rel-19 POs, including those overlapping with legacy POs. Rel-19 UEs only need to decode the Rel-19 PEI, even when some Rel-19 POs overlap with legacy POs.

Separation of additional Rel-19 PEI transmission occasions from legacy ones can be achieved through different PEI-PO-Offset and/or pei-SearchSpace configurations, including firstPDCCH-MonitoringOccasionOfPEI-O.

### 2.3.3 Rel-19 PEI configuration parameters

Related to “FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?” that was raised in [5], let us recall the pei-ConfigBWP-r17 IE.

pei-ConfigBWP-r17      SEQUENCE {

pei-SearchSpace-r17                 SearchSpaceId,

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                       SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                        SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

}                                                                                           OPTIONAL     -- Cond InitialBWP-Paging

pei-ConfigBWP Provides the configuration for PEI reception in this BWP. If the field is absent, the UE does not receive PEI in this BWP. For the initialDownlinkBWP-RedCap not including CD-SSB and the entire CORESET#0, an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing monitors PEI in the initialDownlinkBWP that includes CORESET#0, if the initialDownlinkBWP is configured with pei-ConfigBWP.

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Paging | This field is optionally present, Need R, if this BWP is the initialDownlinkBWP or initialDownlinkBWP-RedCap including CD-SSB and the entire CORESET#0, and pei-Config is configured in DownlinkConfigCommonSIB. Otherwise, this field is absent. |

It is in this structure that pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O are configured. Without these parameters, the UE will not be able to decode the legacy PEI. For Rel-19 PEI, one way to separate the occasions for legacy PEI transmission from occasions for Rel-19 PEI transmission is exactly through different configurations of these parameters. Therefore, we think that it is also necessary to introduce a pei-ConfigBWP-r19 for configuring the corresponding parameters for Rel-19 PEI.

To support separation of additional Rel-19 PEI transmission occasions from legacy ones, introduce pei-ConfigBWP-r19 for configuration of Rel-19 pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O.

In summary, for Rel-19 PEI configuration, we think the following parameters are needed. Their value ranges align with those of the Rel-17 PEI counterparts, except for firstPDCCH-MonitoringOccasionOfPEI-O-R19, where we think that the appropriate range should be in line with ongoing discussions on condensed POs and firstPDCC-MonitoringOccasionOfPO.

- po-NumPerPEI-r19

- payloadSizeDCI-2-7-r19

- pei-FrameOffset-r19

- subgroupsNumPerPO-r19

- subgroupsNumForUEID-r19

- firstPDCCH-MonitoringOccasionOfPEI-O-R19 [range TBD, depending on ongoing discussions on firstPDCCH-MonitoringOccasionOfPO]

- pei-SearchSpace-R19

- nrofCandidates-PEI-R19

The value ranges of the above parameters align with those of the Rel-17 PEI counterparts, except for firstPDCCH-MonitoringOccasionOfPEI-O-R19, where we think that the appropriate range should be at the slot level, i.e., in line with ongoing discussions on condensed POs and firstPDCCH-MonitoringOccasionOfPO.

When it comes to the parameter lastUsedCellOnly-r17 we don’t think there is need for a new counterpart, and the Rel-17 parameter applies to both legacy and Rel-19 configurations.

For Rel-19 PEI configuration, introduce the following parameters:
-po-NumPerPEI-r19, 
-payloadSizeDCI-2-7-r19, 
-pei-FrameOffset-r19,
-subgroupsNumPerPO-r19,
-subgroupsNumForUEID-r19,
-pei-SearchSpace-R19,
-nrofCandidates-PEI-R19, 
firstPDCCH-MonitoringOccasionOfPEI-O-R19

For Rel-19 PEI configuration, use the same value ranges as for legacy PEI for the following parameters:
-po-NumPerPEI-r19, 
-payloadSizeDCI-2-7-r19, 
-pei-FrameOffset-r19,
-subgroupsNumPerPO-r19,
-subgroupsNumForUEID-r19,
-pei-SearchSpace-R19,
-nrofCandidates-PEI-R19

For Rel-19 NES UEs, firstPDCCH-MonitoringOccasionOfPEI-O is configured at slot level with the same value range as for legacy PEI.

# 3	Conclusion

In this contribution we discuss the objective on the adaptation of common signal/channel transmissions. Based on the discussion in the previous sections we made the following observations:

Observation 1	RAN1 has agreed to support DCI signalling for switching SSB burst periodicity.

Observation 2	Dynamic adaptation of SSB transmissions in SCells cells needs to be accompanied by corresponding adaptation of SMTC.

Observation 3	Activation of additional PRACH occasions dynamically is supported for UEs in connected and idle/inactive modes.

Observation 4	A unified signalling method for activating additional RA resources for both idle/inactive and connected UEs simplifies the dynamic control and improves efficiency by utilizing same signalling for both cases.

Observation 5	It would be beneficial if gNB assigns a particular instance of the activated adaptive RA resources when paging a UE to balance the load.

Observation 6	Option b) introduces significant signalling overhead (~46% larger message size than the respective legacy configuration with T = 128, N = T/16, Ns for legacy = 4, SCS = 15 kHz) even for the case where Ns for Rel-19 UEs is set to 8.

Observation 7	The configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level results in 4 octets (~31%) smaller message size than the configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at symbol level.

Observation 8	According to the current specifications, max # of paging occasions per PF is four, max # of PFs associated with one PEI monitoring occasions is two, and if one PEI occasion is associated with POs of two PFs, the two PFs are consecutive PFs.

Observation 9	Current specifications need to be updated in order to support co-existence of PEI and paging adaptations, i.e. Ns = 8.

Observation 10	Separation of additional Rel-19 PEI transmission occasions from legacy ones can be achieved through different PEI-PO-Offset and/or pei-SearchSpace configurations, including firstPDCCH-MonitoringOccasionOfPEI-O.

Based on the discussion in the previous section, we propose the following:

Proposal 1	No need to introduce activation/deactivation MAC CE for switching SSB burst periodicity.

Proposal 2	Introduce mechanism for SMTC adaptation.

Proposal 3	For UEs in idle/inactive mode additional RA occasions can be activated at paging and/or SIB1 transmission occasions.

Proposal 4	For UEs in connected mode, additional RA occasions can be activated at paging and/or, SIB1 transmission occasions, and/or when PDCCH order is transmitted.

Proposal 5	Additional RA occasions can be activated at any time via system information update

Proposal 6	An activated adaptive RA resource can be assigned to an adaptive paging frame/occasion.

Proposal 7	For Rel-19 NES UEs, firstPDCCH-MonitoringOccasionOfPO is configured at slot level.

Proposal 8	For all possible values of ssb-periodicityServingCell in the field description of pagingAdaptationNAndPagingFrameOffset IE, the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) can be set to oneThirtySecondT value.

Proposal 9	For Rel-19 PEI, increase the maximum number of paging occasions per PF to 8.

Proposal 10	Rel-19 PEI independently addresses the additional Rel-19 POs, including those overlapping with legacy POs. Rel-19 UEs only need to decode the Rel-19 PEI, even when some Rel-19 POs overlap with legacy POs.

Proposal 11	To support separation of additional Rel-19 PEI transmission occasions from legacy ones, introduce pei-ConfigBWP-r19 for configuration of Rel-19 pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O.

Proposal 12	For Rel-19 PEI configuration, introduce the following parameters: -po-NumPerPEI-r19,  -payloadSizeDCI-2-7-r19,  -pei-FrameOffset-r19, -subgroupsNumPerPO-r19, -subgroupsNumForUEID-r19, -pei-SearchSpace-R19, -nrofCandidates-PEI-R19,  firstPDCCH-MonitoringOccasionOfPEI-O-R19

Proposal 13	For Rel-19 PEI configuration, use the same value ranges as for legacy PEI for the following parameters: -po-NumPerPEI-r19,  -payloadSizeDCI-2-7-r19,  -pei-FrameOffset-r19, -subgroupsNumPerPO-r19, -subgroupsNumForUEID-r19, -pei-SearchSpace-R19, -nrofCandidates-PEI-R19

Proposal 14	For Rel-19 NES UEs, firstPDCCH-MonitoringOccasionOfPEI-O is configured at slot level with the same value range as for legacy PEI.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

3GPP TS 38.304 V18.1.0 User Equipment (UE) procedures in Idle mode and in RRC Inactive state

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN#105, October 2024.

R2-2502150, “Comments to 38.331 CR for NES”, Ericsson, 2025 April 7-11, Wuhan, China.

R2-2502173, “Running RRC CR for enhancements for network energy efficiency”, Ericsson, 2025 April 7-11, Wuhan, China.