3GPP TSG-RAN WG2 Meeting #129bis	R2-2502779

Wuhan, China, Apr. 7th- 11th, 2025

Agenda item:	7.0.2.19

Source:	Samsung, OPPO, Xiaomi, Huawei, HiSilicon

Title:	Discussion on prerequisite of Rel-17/18 SL relay capability

WID/SID:	NR_SL_relay_enh-Core

Document for:	Discussion and Decision

Introduction

During reviewing of capabilities, the prerequisites of Rel-17/18 SL relay are missing. This contribution tries to address this issue.

Discussion

Way of specification changes

Since Rel17, the prerequisite for SL relay features are missing. On one hand, from technical point of view, it is beneficial to have a clear indication on the prerequisite of each SL relay capability regardless of which release. On the other hand, the changes starting from Rel17 may have more impact (not only for the specification, but also for the implementations, especially for Rel17). Meanwhile, at this moment, it may be not suitable to update TR38.822 for Rel17. In this sense, it is better to have a general discussion on how to carry out specification changes.

Proposal 1: RAN2 is kindly asked to discuss whether to carry out specification change from Rel17.

Rel-17 SL relay capability

In TR38.822, the following feature list is given for Rel-17 SL relay. However, the prerequisite are missing.

- 31-1 (i.e., band combination for relay discovery for the case of L2 and L3 relay) as the prerequisite

31-1 is required for the following features since those features require the relay discovery at both relay UE side and remote UE side:

- 31-3: Basic NR L2 sidelink relay UE operation

- 31-4: Basic NR L2 sidelink remote UE operation

- 31-8: Support L3 sidelink relay UE operation

- 31-9: Support L3 sidelink remote UE operation

Proposal 2: RAN2 is kindly asked to agree that feature 31-1 can be considered as the prerequisite of 31-3/31-4/31-8/31-9.

- 31-5

To support 31-5 (i.e., handover to idle/inactive relay UE), 31-1/4 can be considered as the prerequisite. Since 31-1 is the prerequisite of 31-4, the prerequisite of 31-5 can be 31-4 only.

Proposal 3: RAN2 is kindly asked to agree that feature 31-4 can be considered as the prerequisite of 31-5.

- 31-6

31-6 represents the support of simultaneous transmission/reception of PC5 data (Relay discovery) and Uu UL/DL. How to understand this capability is unclear. One possible understanding is that this capability is not related to the remote UE basic operation (i.e., 31-4), and handover to idle/inactive relay UE (31-5). Specifically, even without such capability, the UE can perform the operation of Uu link and the operations of PC5 link separately (e.g., by TDM way) to support 31-4 and 31-5. Another understanding is that the 31-6 is the prerequisite of 31-4 and 31-5, which requires the UE perform Uu and PC5 operation at the same time.

Proposal 4: RAN2 is kindly asked to discuss whether 31-6 is the prerequisite of 31-4/31-5.

For clear illustration of the above proposals, the prerequisite of each Rel17 feature is added in the following table.

- 6.2.7	NR_SL_Relay

Table 6.2.7-1: Layer-2 and Layer-3 feature list for NR_SL_Relay

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_SL_Relay-Core | 31-1 | Band combination list supporting transmission and reception of relay discovery | Defines the supported band combinations of NR sidelink relay discovery message transmission and reception by the UE. This parameter is used by the remote UE and relay UE, and for the case of L2 and L3 relay. |  | supportedBandCombinationListSidelinkRelayDiscovery-r17 | RF-Parameters | No | No |  | Optional with capability signalling |
|  | 31-2 | Band combination list supporting transmission and reception of non-relay discovery | Defines the supported band combinations of NR sidelink non-relay discovery message transmission and reception by the UE. |  | supportedBandCombinationListSidelinkNonRelayDiscovery-r17 | RF-Parameters | No | No |  | Optional with capability signalling |
|  | 31-3 | Basic NR L2 sidelink relay UE operation | Indicates whether basic NR L2 sidelink relay UE operation is supported by the UE. | 31-1 | relayUE-Operation-L2-r17 | SidelinkParametersNR-r16 | No | No |  | Optional with capability signalling |
|  | 31-4 | Basic NR L2 sidelink remote UE operation | Indicate whether basic NR L2 sidelink remote UE operation is supported by the UE. | 31-1 | remoteUE-Operation-L2-r17 | SidelinkParametersNR-r16 | No | No |  | Optional with capability signalling |
|  | 31-5 | Remote UE performs handover to idle/inactive relay UE | Indicate whether L2 sidelink remote UE supports direct to indirect path switch with target relay in RRC_IDLE or RRC_INACTIVE state. | 31-4 | remoteUE-PathSwitchToIdleInactiveRelay-r17 | SidelinkParametersNR-r16 | No | No |  | Optional with capability signalling |
|  | 31-6 | UE supports simultaneous transmission/reception of PC5 data (Relay discovery) and Uu uplink/downlink respectively | Indicates, for a particular Uu band combination, the PC5 Relay discovery band combination(s) on which the UE supports simultaneous transmission/reception of PC5 data (Relay discovery) and Uu uplink/downlink respectively. |  | supportedBandCombListPerBC-SL-RelayDiscovery-r17 | BandCombination-v1700 | No | No |  | Optional with capability signalling |
|  | 31-7 | UE supports simultaneous transmission/reception of PC5 data (non-Relay discovery) and Uu uplink/downlink respectively | Indicates, for a particular Uu band combination, the PC5 non-Relay discovery band combination(s) on which the UE supports simultaneous transmission/reception of PC5 data (non-Relay discovery) and Uu uplink/downlink respectively. |  | supportedBandCombListPerBC-SL-NonRelayDiscovery-r17 | BandCombination-v1700 | No | No |  | Optional with capability signalling |
|  | 31-8 | Support L3 sidelink relay UE operation | It is optional for UE to support L3 sidelink relay UE operation | 31-1 | n/a | n/a | n/a | n/a |  | Optional without capability signalling |
|  | 31-9 | Support L3 sidelink remote UE operation | It is optional for UE to support L3 sidelink remote UE operation | 31-1 | n/a | n/a | n/a | n/a |  | Optional without capability signalling |

Rel-18 SL relay capability

L2 U2N related capabilities

- 51-4

To support 51-4 (i.e., NR U2N L2 sidelink remote UE path switch operation), some Rel-17 capabilities should be the prerequisite, e.g., 31-1(i.e., Band combination list supporting transmission and reception of relay discovery), and 31-4(i.e., Basic NR L2 sidelink remote UE operation).

Proposal 5: RAN2 is kindly asked to agree that 31-1/31-4 are prerequisite of 51-4.

In Rel-17, a separate capability 31-5 (i.e., Remote UE performs handover to idle/inactive relay UE) is given. The question is whether 31-5 can be considered as the prerequisite of 51-4. Three possible understandings can be considered:

- Understanding 1 (It is the prerequisite): if a remote UE supports the Rel18 I2I path switch, it already supports Rel-17 D2I path switching. Since in Rel-18, there is no separate UE capability for idle/inactive, it means that the UE supports both of connected case and idle/inactive case. Thus, 31-5 can be considered as the prerequisite of 51-4

- Understanding 2 (Need new capability, and it is not prerequisite): this depends on whether we support the case that the remote UE only supports I2I path switch to CONNECTED relay UE in Rel-18. If the answer is yes, we may need a separate capability of supporting I2I path switch to Idle/Inactive relay UE, and as a result, 31-5 is not the prerequisite of 51-4.

- Understanding 3 (It is not the prerequisite): since Rel-18 does not have separate UE capability for idle/inactive for I2I path switch, 31-5 and 51-4 can be used jointly to determine the UE supports I2I path switch to idle/inactive relay UE. In this sense, it can be understood that 51-4 is the capability for I2I path switch to connected relay UE, and 31-5 is not the prerequisite of 51-4.

Proposal 6: RAN2 is kindly asked to discuss whether 31-5 is the prerequisite of 51-4.

- 51-5

To support 51-5 (i.e., NR L2 multipath remote UE operation using PC5 connection), 31-1 (i.e., BC for relay discovery), and 31-4(i.e., Basic L2 remote operation).

Proposal 7: RAN2 is kindly asked to agree that 31-1/31-4 are prerequisite of 51-5.

- 31-6 as requisite of 51-4/51-5

Similar to Rel-17, it is unclear on whether 31-6 (i.e., UE supports simultaneous transmission/reception of PC5 data (Relay discovery) and Uu uplink/downlink respectively) is the prerequisite of 51-4/51-5. The reason is that in path switch case or multi-path case, the Uu link and PC5 link can work separately (e.g., TDM way). So, it is better to have some discussions in RAN2.

Proposal 8: RAN2 is kindly asked to discuss whether 31-6 is the prerequisite of 51-4/51-5.

- 51-5 (i.e., NR L2 multipath remote UE operation using PC5 connection) as the prerequisite

The feature of 51-5 is required for the following features since all features need remote UE operation using PC5 connection:

- 51-8: indirect path addition/change to idle or inactive relay UE

- 51-9: PDCP duplication with more than one Uu RLC

- 51-15: PDCP duplication over split DRB in L2 multi-path relay

- 51-16: PDCP duplication over split SRB1/2 in L2 multi-path relay

- 51-19: UL transmission via both direct path and indirect path for split DRB

Proposal 9: RAN2 is kindly asked to agree that 51-5 is prerequisite of 51-8/51-9/51-15/51-16/51-19.

- 51-13/14

To support 51-13(i.e, CA-based PDCP duplication over DRB) and 51-14 (i.e., CA-based PDCP duplication over SRB1/2), 51-9 can be considered as the prerequisite since CA-based PDCP duplication requires that the remote UE transmit data over more than one Uu RLC for a DRB.

Proposal 10: RAN2 is kindly asked to agree that 51-9 is the prerequisite of 51-13/14.

- 51-1 (i.e., Band combination list supporting transmission and reception of U2U relay discovery) as prerequisite

51-1 indicates the band combination for U2U relay discovery. So, it should be prerequisite for U2U related capabilities, i.e.,

- 51-2: Basic NR U2U L2 sidelink relay UE operation

- 51-3: Basic NR U2U L2 sidelink remote UE operation

- 51-11: L3 sidelink U2U relay UE operation

- 51-12: L3 sidelink U2U remote UE operation

Proposal 11: RAN2 is kindly asked to agree that 51-1 are the prerequisite of 51-2/51-3/51-11/51-12.

- 51-18

To support 51-18 (i.e., L2 PC5-RRC trigger), the prerequisite for remote UE and relay UE can be different:

- Remote UE: 51-5 can be considered as the prerequisite

- Relay UE: 31-3 can be considered as the prerequisite.

Proposal 12: RAN2 is kindly asked to agree that 51-5 and 31-3 are the prerequisite of 51-18, respectively, for remote UE and relay UE.

For clear illustration of the above proposals, the prerequisite of each Rel18 feature is added in the following table.

- 7.2.8	NR_SL_relay_enh

Table 7.2.8-1: Layer-2 and Layer-3 feature list for NR_SL_relay_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_SL_relay_enh-Core | 51-1 | Band combination list supporting transmission and reception of U2U relay discovery | Defines the supported band combinations of NR U2U sidelink relay discovery message transmission and reception by the UE. This parameter is used by the remote UE and relay UE, and for the case of L2 and L3 relay. |  | supportedBandCombinationListSL-U2U-RelayDiscovery-r18 | supportedBandCombinationListSL-U2U-Relay-r18 | No | No |  | Optional with capability signalling |
|  | 51-2 | Basic NR U2U L2 sidelink relay UE operation | Indicates whether L2 U2U sidelink relay UE operation is supported by the UE. | 51-1 | relayUE-U2U-OperationL2-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-3 | Basic NR U2U L2 sidelink remote UE operation | Indicates whether NR L2 sidelink relay UE operation is supported by the UE. | 51-1 | remoteUE-U2U-OperationL2-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-4 | NR U2N L2 sidelink remote UE path switch operation | Indicates whether enhanced NR L2 U2N remote UE operation for intra-gNB path switch and inter-gNB path switch including separate SL-RSRP and SD-RSRP threshold configurations for events X1 and X2 is supported by the UE. | 31-1, 31-4, 31-5 (FFS), 31-6(FFS) | remoteUE-U2N-PathSwitchOperationL2-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-5 | NR L2 multipath remote UE operation using PC5 connection | Indicates whether L2 multi-path remote UE operation using PC5 connection is supported by the UE. | 31-1, 31-4, 31-6(FFS) | multipathRemoteUE-PC5L2-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-6 | NR L2 multipath relay UE operation using non-3GPP connection | Indicates whether L2 multi-path relay UE operation using non-3GPP connection is supported by the UE. |  | multipathRelayUE-N3C-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-7 | NR L2 multipath remote UE operation using non-3GPP connection | Indicates whether L2 multi-path remote UE operation using non-3GPP connection is supported by the UE. |  | multipathRemoteUE-N3C-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-8 | Indirect path addition/change to idle or inactive Relay UE | Indicates whether L2 multi-path remote UE supports indirect path addition or indirect path change with target relay UE in RRC_IDLE or RRC_INACTIVE state. | 51-5 | remoteUE-IndirectPathAddChangeToIdleInactiveRelay-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-9 | PDCP duplication with more than one Uu RLC | Indicates whether L2 multi-path remote UE supports PDCP duplication with more than one RLC entity over Uu interface in L2 multi-path relay. | 51-5 | pdcp-DuplicationMoreThanOneUuRLC-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-10 | UE supports simultaneous transmission/reception of PC5 data (U2U relay discovery) and Uu uplink/downlink respectively | Indicates, for a particular Uu band combination, the PC5 U2U relay discovery band combination(s) on which the UE supports simultaneous transmission/reception of PC5 data (U2U relay discovery) and Uu uplink/downlink respectively. The leading / leftmost bit (bit 0) corresponds to the first band combination included in supportedBandCombinationListSL-U2U-RelayDiscovery-r18, the next bit corresponds to the second band combination included in supportedBandCombinationListSL-U2U-RelayDiscovery-r18 and so on with value 1 indicating simultaneous transmission/reception is supported. |  | supportedBandCombListPerBC-SL-U2U-RelayDiscovery-r18 | BandCombination-v1800 | No | No |  | Optional with capability signalling |
|  | 51-11 | L3 sidelink U2U relay UE operation | It is optional for UE to support L3 sidelink U2U relay UE operation | 51-1 | n/a | n/a | No | No |  | Optional without capability signalling |
|  | 51-12 | L3 sidelink U2U remote UE operation | It is optional for UE to support L3 sidelink U2U relay UE operation as specified in TS 38.331. | 51-1 | n/a | n/a | No | No |  | Optional without capability signalling |
|  | 51-13 | CA-based PDCP duplication over DRB | Indicates whether L2 multi-path remote UE supports CA-based PDCP duplication over DRB using Uu interface in L2 multi-path relay. | 51-9 | pdcp-CADuplicationDirectpath-DRB-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-14 | CA-based PDCP duplication over SRB1/2 | Indicates whether L2 multi-path remote UE supports CA-based PDCP duplication over SRB1/2 using Uu interface in L2 multi-path relay. | 51-9 | pdcp-CADuplicationDirectpath-SRB-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-15 | PDCP duplication over split DRB in L2 multi-path relay | Indicates whether L2 multi-path remote UE supports PDCP duplication over split DRB in L2 multi-path relay. | 51-5 | pdcp-DuplicationMP-SplitDRB-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-16 | PDCP duplication over split SRB1/2 in L2 multi-path relay | Indicates whether L2 multi-path remote UE supports PDCP duplication over split SRB1/2 in L2 multi-path relay. | 51-5 | pdcp-DuplicationMP-SplitSRB-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-17 | recovery from direct path RLF via split SRB1 using either PC5 connection or non-3GPP connection | Indicates whether L2 multi-path remote UE supports recovery from direct path RLF via split SRB1 using either PC5 connection or non-3GPP connection (if supported) in TS 38.331. |  | directpathRLF-RecoveryViaSRB1-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-18 | L2 PC5-RRC trigger | whether L2 sidelink relay UE or L2 sidelink remote UE supports the PC5-RRC trigger in L2 multi-path relay. | 51-5 (remote UE), or 31-3 (relay UE) | n/a | n/a | No | No |  | Optional without capability signalling |
|  | 51-19 | UL transmission via both direct path and indirect path for split DRB | Indicates whether L2 multi-path remote UE supports UL transmission via both direct path and indirect path for split DRB. | 51-5 | splitDRB-WithUL-BothDirectIndirect-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |

Conclusion

In this contribution, we address the prerequisite of Rel-17/Rel-18 SL relay features, and have the following proposals:

Way of specification changes

Proposal 1: (FFS point) RAN2 is kindly asked to discuss whether to carry out specification change from Rel17.

Rel-17 features

Proposal 2: RAN2 is kindly asked to agree that feature 31-1 can be considered as the prerequisite of 31-3/31-4/31-8/31-9.

Proposal 3: RAN2 is kindly asked to agree that feature 31-4 can be considered as the prerequisite of 31-5.

Proposal 4: (FFS point) RAN2 is kindly asked to discuss whether 31-6 is the prerequisite of 31-4/31-5.

Rel-18 features

Proposal 5: RAN2 is kindly asked to agree that 31-1/31-4/31-6 are prerequisite of 51-4.

Proposal 6: (FFS point) RAN2 is kindly asked to discuss whether 31-5 is the prerequisite of 51-4.

Proposal 7: RAN2 is kindly asked to agree that 31-1/31-4 are prerequisite of 51-5.

Proposal 8: (FFS point) RAN2 is kindly asked to discuss whether 31-6 is the prerequisite of 51-4/51-5.

Proposal 9: RAN2 is kindly asked to agree that 51-5 is prerequisite of 51-8/51-9/51-15/51-16/51-19.

Proposal 10: RAN2 is kindly asked to agree that 51-9 is the prerequisite of 51-13/14.

Proposal 11: RAN2 is kindly asked to agree that 51-1 are the prerequisite of 51-2/51-3/51-11/51-12.

Proposal 12: RAN2 is kindly asked to agree that 51-5 and 31-3 are the prerequisite of 51-18, respectively, for remote UE and relay UE.

- References

Annex – TR38.822 (Change from Rel17)

- 6.2.7	NR_SL_Relay

Table 6.2.7-1: Layer-2 and Layer-3 feature list for NR_SL_Relay

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_SL_Relay-Core | 31-1 | Band combination list supporting transmission and reception of relay discovery | Defines the supported band combinations of NR sidelink relay discovery message transmission and reception by the UE. This parameter is used by the remote UE and relay UE, and for the case of L2 and L3 relay. |  | supportedBandCombinationListSidelinkRelayDiscovery-r17 | RF-Parameters | No | No |  | Optional with capability signalling |
|  | 31-2 | Band combination list supporting transmission and reception of non-relay discovery | Defines the supported band combinations of NR sidelink non-relay discovery message transmission and reception by the UE. |  | supportedBandCombinationListSidelinkNonRelayDiscovery-r17 | RF-Parameters | No | No |  | Optional with capability signalling |
|  | 31-3 | Basic NR L2 sidelink relay UE operation | Indicates whether basic NR L2 sidelink relay UE operation is supported by the UE. | 31-1 | relayUE-Operation-L2-r17 | SidelinkParametersNR-r16 | No | No |  | Optional with capability signalling |
|  | 31-4 | Basic NR L2 sidelink remote UE operation | Indicate whether basic NR L2 sidelink remote UE operation is supported by the UE. | 31-1 | remoteUE-Operation-L2-r17 | SidelinkParametersNR-r16 | No | No |  | Optional with capability signalling |
|  | 31-5 | Remote UE performs handover to idle/inactive relay UE | Indicate whether L2 sidelink remote UE supports direct to indirect path switch with target relay in RRC_IDLE or RRC_INACTIVE state. | 31-4 | remoteUE-PathSwitchToIdleInactiveRelay-r17 | SidelinkParametersNR-r16 | No | No |  | Optional with capability signalling |
|  | 31-6 | UE supports simultaneous transmission/reception of PC5 data (Relay discovery) and Uu uplink/downlink respectively | Indicates, for a particular Uu band combination, the PC5 Relay discovery band combination(s) on which the UE supports simultaneous transmission/reception of PC5 data (Relay discovery) and Uu uplink/downlink respectively. |  | supportedBandCombListPerBC-SL-RelayDiscovery-r17 | BandCombination-v1700 | No | No |  | Optional with capability signalling |
|  | 31-7 | UE supports simultaneous transmission/reception of PC5 data (non-Relay discovery) and Uu uplink/downlink respectively | Indicates, for a particular Uu band combination, the PC5 non-Relay discovery band combination(s) on which the UE supports simultaneous transmission/reception of PC5 data (non-Relay discovery) and Uu uplink/downlink respectively. |  | supportedBandCombListPerBC-SL-NonRelayDiscovery-r17 | BandCombination-v1700 | No | No |  | Optional with capability signalling |
|  | 31-8 | Support L3 sidelink relay UE operation | It is optional for UE to support L3 sidelink relay UE operation | 31-1 | n/a | n/a | n/a | n/a |  | Optional without capability signalling |
|  | 31-9 | Support L3 sidelink remote UE operation | It is optional for UE to support L3 sidelink remote UE operation | 31-1 | n/a | n/a | n/a | n/a |  | Optional without capability signalling |

- 7.2.8	NR_SL_relay_enh

Table 7.2.8-1: Layer-2 and Layer-3 feature list for NR_SL_relay_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_SL_relay_enh-Core | 51-1 | Band combination list supporting transmission and reception of U2U relay discovery | Defines the supported band combinations of NR U2U sidelink relay discovery message transmission and reception by the UE. This parameter is used by the remote UE and relay UE, and for the case of L2 and L3 relay. |  | supportedBandCombinationListSL-U2U-RelayDiscovery-r18 | supportedBandCombinationListSL-U2U-Relay-r18 | No | No |  | Optional with capability signalling |
|  | 51-2 | Basic NR U2U L2 sidelink relay UE operation | Indicates whether L2 U2U sidelink relay UE operation is supported by the UE. | 51-1 | relayUE-U2U-OperationL2-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-3 | Basic NR U2U L2 sidelink remote UE operation | Indicates whether NR L2 sidelink relay UE operation is supported by the UE. | 51-1 | remoteUE-U2U-OperationL2-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-4 | NR U2N L2 sidelink remote UE path switch operation | Indicates whether enhanced NR L2 U2N remote UE operation for intra-gNB path switch and inter-gNB path switch including separate SL-RSRP and SD-RSRP threshold configurations for events X1 and X2 is supported by the UE. | 31-5 (FFS), 31-6(FFS) | remoteUE-U2N-PathSwitchOperationL2-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-5 | NR L2 multipath remote UE operation using PC5 connection | Indicates whether L2 multi-path remote UE operation using PC5 connection is supported by the UE. | 31-4, 31-6 | multipathRemoteUE-PC5L2-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-6 | NR L2 multipath relay UE operation using non-3GPP connection | Indicates whether L2 multi-path relay UE operation using non-3GPP connection is supported by the UE. |  | multipathRelayUE-N3C-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-7 | NR L2 multipath remote UE operation using non-3GPP connection | Indicates whether L2 multi-path remote UE operation using non-3GPP connection is supported by the UE. |  | multipathRemoteUE-N3C-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-8 | Indirect path addition/change to idle or inactive Relay UE | Indicates whether L2 multi-path remote UE supports indirect path addition or indirect path change with target relay UE in RRC_IDLE or RRC_INACTIVE state. | 51-5 | remoteUE-IndirectPathAddChangeToIdleInactiveRelay-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-9 | PDCP duplication with more than one Uu RLC | Indicates whether L2 multi-path remote UE supports PDCP duplication with more than one RLC entity over Uu interface in L2 multi-path relay. | 51-5 | pdcp-DuplicationMoreThanOneUuRLC-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-10 | UE supports simultaneous transmission/reception of PC5 data (U2U relay discovery) and Uu uplink/downlink respectively | Indicates, for a particular Uu band combination, the PC5 U2U relay discovery band combination(s) on which the UE supports simultaneous transmission/reception of PC5 data (U2U relay discovery) and Uu uplink/downlink respectively. The leading / leftmost bit (bit 0) corresponds to the first band combination included in supportedBandCombinationListSL-U2U-RelayDiscovery-r18, the next bit corresponds to the second band combination included in supportedBandCombinationListSL-U2U-RelayDiscovery-r18 and so on with value 1 indicating simultaneous transmission/reception is supported. | 51-1 | supportedBandCombListPerBC-SL-U2U-RelayDiscovery-r18 | BandCombination-v1800 | No | No |  | Optional with capability signalling |
|  | 51-11 | L3 sidelink U2U relay UE operation | It is optional for UE to support L3 sidelink U2U relay UE operation | 51-1 | n/a | n/a | No | No |  | Optional without capability signalling |
|  | 51-12 | L3 sidelink U2U remote UE operation | It is optional for UE to support L3 sidelink U2U relay UE operation as specified in TS 38.331. | 51-1 | n/a | n/a | No | No |  | Optional without capability signalling |
|  | 51-13 | CA-based PDCP duplication over DRB | Indicates whether L2 multi-path remote UE supports CA-based PDCP duplication over DRB using Uu interface in L2 multi-path relay. | 51-9 | pdcp-CADuplicationDirectpath-DRB-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-14 | CA-based PDCP duplication over SRB1/2 | Indicates whether L2 multi-path remote UE supports CA-based PDCP duplication over SRB1/2 using Uu interface in L2 multi-path relay. | 51-9 | pdcp-CADuplicationDirectpath-SRB-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-15 | PDCP duplication over split DRB in L2 multi-path relay | Indicates whether L2 multi-path remote UE supports PDCP duplication over split DRB in L2 multi-path relay. | 51-5 | pdcp-DuplicationMP-SplitDRB-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-16 | PDCP duplication over split SRB1/2 in L2 multi-path relay | Indicates whether L2 multi-path remote UE supports PDCP duplication over split SRB1/2 in L2 multi-path relay. | 51-5 | pdcp-DuplicationMP-SplitSRB-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-17 | recovery from direct path RLF via split SRB1 using either PC5 connection or non-3GPP connection | Indicates whether L2 multi-path remote UE supports recovery from direct path RLF via split SRB1 using either PC5 connection or non-3GPP connection (if supported) in TS 38.331. |  | directpathRLF-RecoveryViaSRB1-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-18 | L2 PC5-RRC trigger | whether L2 sidelink relay UE or L2 sidelink remote UE supports the PC5-RRC trigger in L2 multi-path relay. | 51-5 (remote UE), or 31-3 (relay UE) | n/a | n/a | No | No |  | Optional without capability signalling |
|  | 51-19 | UL transmission via both direct path and indirect path for split DRB | Indicates whether L2 multi-path remote UE supports UL transmission via both direct path and indirect path for split DRB. | 51-5 | splitDRB-WithUL-BothDirectIndirect-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |

Annex – TR38.822 (Change from Rel18)

- 7.2.8	NR_SL_relay_enh

Table 7.2.8-1: Layer-2 and Layer-3 feature list for NR_SL_relay_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_SL_relay_enh-Core | 51-1 | Band combination list supporting transmission and reception of U2U relay discovery | Defines the supported band combinations of NR U2U sidelink relay discovery message transmission and reception by the UE. This parameter is used by the remote UE and relay UE, and for the case of L2 and L3 relay. |  | supportedBandCombinationListSL-U2U-RelayDiscovery-r18 | supportedBandCombinationListSL-U2U-Relay-r18 | No | No |  | Optional with capability signalling |
|  | 51-2 | Basic NR U2U L2 sidelink relay UE operation | Indicates whether L2 U2U sidelink relay UE operation is supported by the UE. | 51-1 | relayUE-U2U-OperationL2-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-3 | Basic NR U2U L2 sidelink remote UE operation | Indicates whether NR L2 sidelink relay UE operation is supported by the UE. | 51-1 | remoteUE-U2U-OperationL2-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-4 | NR U2N L2 sidelink remote UE path switch operation | Indicates whether enhanced NR L2 U2N remote UE operation for intra-gNB path switch and inter-gNB path switch including separate SL-RSRP and SD-RSRP threshold configurations for events X1 and X2 is supported by the UE. | 31-1, 31-4, 31-5 (FFS), 31-6(FFS) | remoteUE-U2N-PathSwitchOperationL2-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-5 | NR L2 multipath remote UE operation using PC5 connection | Indicates whether L2 multi-path remote UE operation using PC5 connection is supported by the UE. | 31-1, 31-4, 31-6 | multipathRemoteUE-PC5L2-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-6 | NR L2 multipath relay UE operation using non-3GPP connection | Indicates whether L2 multi-path relay UE operation using non-3GPP connection is supported by the UE. |  | multipathRelayUE-N3C-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-7 | NR L2 multipath remote UE operation using non-3GPP connection | Indicates whether L2 multi-path remote UE operation using non-3GPP connection is supported by the UE. |  | multipathRemoteUE-N3C-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-8 | Indirect path addition/change to idle or inactive Relay UE | Indicates whether L2 multi-path remote UE supports indirect path addition or indirect path change with target relay UE in RRC_IDLE or RRC_INACTIVE state. | 51-5 | remoteUE-IndirectPathAddChangeToIdleInactiveRelay-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-9 | PDCP duplication with more than one Uu RLC | Indicates whether L2 multi-path remote UE supports PDCP duplication with more than one RLC entity over Uu interface in L2 multi-path relay. | 51-5 | pdcp-DuplicationMoreThanOneUuRLC-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-10 | UE supports simultaneous transmission/reception of PC5 data (U2U relay discovery) and Uu uplink/downlink respectively | Indicates, for a particular Uu band combination, the PC5 U2U relay discovery band combination(s) on which the UE supports simultaneous transmission/reception of PC5 data (U2U relay discovery) and Uu uplink/downlink respectively. The leading / leftmost bit (bit 0) corresponds to the first band combination included in supportedBandCombinationListSL-U2U-RelayDiscovery-r18, the next bit corresponds to the second band combination included in supportedBandCombinationListSL-U2U-RelayDiscovery-r18 and so on with value 1 indicating simultaneous transmission/reception is supported. | 51-1 | supportedBandCombListPerBC-SL-U2U-RelayDiscovery-r18 | BandCombination-v1800 | No | No |  | Optional with capability signalling |
|  | 51-11 | L3 sidelink U2U relay UE operation | It is optional for UE to support L3 sidelink U2U relay UE operation | 51-1 | n/a | n/a | No | No |  | Optional without capability signalling |
|  | 51-12 | L3 sidelink U2U remote UE operation | It is optional for UE to support L3 sidelink U2U relay UE operation as specified in TS 38.331. | 51-1 | n/a | n/a | No | No |  | Optional without capability signalling |
|  | 51-13 | CA-based PDCP duplication over DRB | Indicates whether L2 multi-path remote UE supports CA-based PDCP duplication over DRB using Uu interface in L2 multi-path relay. | 51-9 | pdcp-CADuplicationDirectpath-DRB-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-14 | CA-based PDCP duplication over SRB1/2 | Indicates whether L2 multi-path remote UE supports CA-based PDCP duplication over SRB1/2 using Uu interface in L2 multi-path relay. | 51-9 | pdcp-CADuplicationDirectpath-SRB-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-15 | PDCP duplication over split DRB in L2 multi-path relay | Indicates whether L2 multi-path remote UE supports PDCP duplication over split DRB in L2 multi-path relay. | 51-5 | pdcp-DuplicationMP-SplitDRB-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-16 | PDCP duplication over split SRB1/2 in L2 multi-path relay | Indicates whether L2 multi-path remote UE supports PDCP duplication over split SRB1/2 in L2 multi-path relay. | 51-5 | pdcp-DuplicationMP-SplitSRB-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-17 | recovery from direct path RLF via split SRB1 using either PC5 connection or non-3GPP connection | Indicates whether L2 multi-path remote UE supports recovery from direct path RLF via split SRB1 using either PC5 connection or non-3GPP connection (if supported) in TS 38.331. |  | directpathRLF-RecoveryViaSRB1-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |
|  | 51-18 | L2 PC5-RRC trigger | whether L2 sidelink relay UE or L2 sidelink remote UE supports the PC5-RRC trigger in L2 multi-path relay. | 51-5 (remote UE), or 31-3 (relay UE) | n/a | n/a | No | No |  | Optional without capability signalling |
|  | 51-19 | UL transmission via both direct path and indirect path for split DRB | Indicates whether L2 multi-path remote UE supports UL transmission via both direct path and indirect path for split DRB. | 51-5 | splitDRB-WithUL-BothDirectIndirect-r18 | RelayParameters-r17 | No | No |  | Optional with capability signalling |

Annex – TS38.306

- 4.2.16.1.1	Sidelink General Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| accessStratumReleaseSidelink-r16 Indicates the access stratum release for NR sidelink communication the UE supports as specified in TS 38.331 [9]. | UE | Yes | No | No |
| multipathRelayUE-N3C-r18 Indicates whether L2 multi-path relay UE operation using non-3GPP connection is supported by the UE. | UE | No | No | No |
| multipathRemoteUE-N3C-r18 Indicates whether L2 multi-path remote UE operation using non-3GPP connection is supported by the UE. | UE | No | No | No |
| multipathRemoteUE-PC5L2-r18 Indicates whether L2 multi-path remote UE operation using PC5 connection is supported by the UE. A UE supporting this feature shall also indicate support of supportedBandCombinationListSL-RelayDiscovery-r17, remoteUE-Operation-L2-r17, and supportedBandCombListPerBC-SL-RelayDiscovery-r17(FFS). | UE | No | No | No |
| pdcp-DuplicationMoreThanOneUuRLC-r18 Indicates whether L2 multi-path remote UE supports PDCP duplication with more than one RLC entity over Uu interface in L2 multi-path relay. A UE supporting this feature shall also indicate support of multipathRemoteUE-PC5L2-r18. | UE | No | No | No |
| pdcp-CADuplicationDirectpath-DRB-r18 Indicates whether L2 multi-path remote UE supports CA-based PDCP duplication over DRB using Uu interface in L2 multi-path relay. A UE supporting this feature shall also indicate support of pdcp-DuplicationMoreThanOneUuRLC-r18. | UE | No | No | No |
| pdcp-CADuplicationDirectpath-SRB-r18 Indicates whether L2 multi-path remote UE supports CA-based PDCP duplication over SRB1/2 using Uu interface in L2 multi-path relay. A UE supporting this feature shall also indicate support of pdcp-DuplicationMoreThanOneUuRLC-r18. | UE | No | No | No |
| pdcp-DuplicationMP-SplitDRB-r18 Indicates whether L2 multi-path remote UE supports PDCP duplication over split DRB in L2 multi-path relay. A UE supporting this feature shall also indicate support of multipathRemoteUE-PC5L2-r18. | UE | No | No | No |
| pdcp-DuplicationMP-SplitSRB-r18 Indicates whether L2 multi-path remote UE supports PDCP duplication over split SRB1/2 in L2 multi-path relay. A UE supporting this feature shall also indicate support of multipathRemoteUE-PC5L2-r18. | UE | No | No | No |
| directpathRLF-RecoveryViaSRB1-r18 Indicates whether L2 multi-path remote UE supports recovery from direct path RLF via split SRB1 using either PC5 connection or non-3GPP connection (if supported) in TS 38.331 [9]. | UE | No | No | No |
| posSIB-ForwardingSupported-r18 Indicates whether the UE, when operating as an NR L2 sidelink relay UE, supports forwarding of posSIBs. The UE capable of operation as an NR L2 sidelink relay UE shall set this field to supported if it is capable of obtaining posSIBs. | UE | CY | No | No |
| relayUE-Operation-L2-r17 Indicates whether NR L2 sidelink relay UE operation is supported by the UE. A UE supporting this feature shall also indicate support of supportedBandCombinationListSL-RelayDiscovery-r17. | UE | No | No | No |
| relayUE-U2U-OperationL2-r18 Indicates whether L2 U2U sidelink relay UE operation is supported by the UE. A UE supporting this feature shall also indicate support of supportedBandCombinationListSL-U2U-RelayDiscovery-r18. | UE | No | No | No |
| remoteUE-IndirectPathAddChangeToIdleInactiveRelay-r18 Indicates whether L2 multi-path remote UE supports indirect path addition or indirect path change with target relay UE in RRC_IDLE or RRC_INACTIVE state. A UE supporting this feature shall also indicate support of multipathRemoteUE-PC5L2-r18. | UE | No | No | No |
| remoteUE-Operation-L2-r17 Indicates whether NR L2 sidelink remote UE operation is supported by the UE. A UE supporting this feature shall also indicate support of supportedBandCombinationListSL-RelayDiscovery-r17. | UE | No | No | No |
| remoteUE-PathSwitchToIdleInactiveRelay-r17 Indicates whether L2 sidelink remote UE supports direct to indirect path switch with target relay in RRC_IDLE or RRC_INACTIVE state. A UE supporting this feature shall also indicate support of remoteUE-Operation-L2-r17. | UE | No | No | No |
| remoteUE-U2N-PathSwitchOperationL2-r18 Indicates whether enhanced NR L2 U2N remote UE operation for intra-gNB path switch and inter-gNB path switch including separate SL-RSRP and SD-RSRP threshold configurations for events X1 and X2 is supported by the UE. A UE supporting this feature shall also indicate support of supportedBandCombinationListSL-RelayDiscovery-r17, remoteUE-Operation-L2-r17, remoteUE-PathSwitchToIdleInactiveRelay-r17 (FFS), and supportedBandCombListPerBC-SL-RelayDiscovery-r17(FFS). | UE | No | No | No |
| remoteUE-U2U-OperationL2-r18 Indicates whether L2 U2U sidelink remote UE operation is supported by the UE. A UE supporting this feature shall also indicate support of supportedBandCombinationListSL-U2U-RelayDiscovery-r18. | UE | No | No | No |
| sfn-DFN-OffsetSupported-r18 Indicates whether the UE, when operating as an NR L2 sidelink relay UE, supports indication of the offset between SFN and DFN timelines. | UE | No | No | No |
| sl-PRS-CommonProcCapabilityPerUE-r18 Indicates the common SL-PRS processing capability, and comprises the following parameters: - maxNumOfActiveSL-PRS-Resources-r18: Maximum number of active SL PRS resources across all configured RPs across all bands in a slot assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE; - maxNumOfSlotswithActiveSL-PRS-Resources-r18: Maximum number of slots with active SL PRS resources across all configured RPs across all bands assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE. A UE supporting this feature shall also support sl-PRS-CommonProcCapabilityPerBand-r18. | UE | No | No | No |
| splitDRB-WithUL-BothDirectIndirect-r18 Indicates whether L2 multi-path remote UE supports UL transmission via both direct path and indirect path for split DRB. A UE supporting this feature shall also indicate support of multipathRemoteUE-PC5L2-r18. | UE | No | No | No |