3GPP TSG-RAN WG2 Meeting #129bis	R2-2502790

Wuhan, China, 7 - 11 April, 2025

Title: 	Discussion on MRO for CHO with candidate SCGs

Source: 	Huawei, HiSilicon

Agenda item:	8.10.2.2

Document for:	Discussion/Decision

- Introduction

RAN3 has agreed on an outgoing LS [1] to RAN2. And RAN2 has made some progress in [1][2].

In this paper, we mainly analyze possible RAN2 impacts for MRO for CHO with candidate SCGs.

- Discussion

In the RAN3 LS [1], it mentions the following:

(1) Regarding MRO for CHO with candidate SCG(s):

RAN3 concluded that the following information can be reported by the UE in RLF Report for failure reason detection and optimization:

- The type of the first fulfilled execution condition (e.g. CPAC or CHO). Fulfilment means all events of the type are met.

- Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

- The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

We also compare the above requirements with the previous RAN2 meeting agreements (listed in section 5).

Bullet (1): The type of the first fulfilled execution condition (e.g. CPAC or CHO). Fulfilment means all events of the type are met.

At RAN2#127bis, the following agreement was made, and we think RAN2 and RAN3 are already aligned. Details can be discussed later during stage-3 CR discussion.

- UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS.

Bullet (2): Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered.

At RAN2#127, RAN2 agree the following:

We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

For Bullet (2), we think the applicable use cases are: (a) when both CHO condition and associated CPC condition are fulfilled; (b) when CHO is fulfilled but CPC conditions are not fulfilled.

Bullet (3):Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered.

At RAN2#127, RAN2 agree the following:

We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

For Bullet (3), we think the applicable use cases are: (a) when both CHO condition and associated CPC condition are fulfilled; (b) when CPC is fulfilled but CHO conditions are not fulfilled.

Bullet (4): The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

At RAN2#127, RAN2 agree the following:

We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

For Bullet (4), we think it is only applicable for the case when both CHO condition and associated CPC condition are fulfilled.

In [2], we checked the latest text regarding inclusion of identifier of candidate PCell(s) and identifier of candidate PSCell(s), and in section5.3.10.5, it captures the following:

if the UE has both executive conditions and if both conditions are fulfilled, the UE include identifiers of candidate PCell(s) and candidate PSCell(s)

We think the current running CR is missing the cases where either CHO or CPC condition is fulfilled, so we proposed:

Proposal 1: It is proposed RAN2 to agreed that when CHO is fulfilled but CPC conditions are not fulfilled, in RLF report, UE reports identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered.

Proposal 2: It is proposed RAN2 to agreed that when CPC is fulfilled but CHO conditions are not fulfilled, in RLF report, UE reports identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered.

- Conclusion

Based on the discussions above, we have the following proposals:

Proposal 1: It is proposed RAN2 to agreed that when CHO is fulfilled but CPC conditions are not fulfilled, in RLF report, UE reports identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered.

Proposal 2: It is proposed RAN2 to agreed that when CPC is fulfilled but CHO conditions are not fulfilled, in RLF report, UE reports identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered.

- Reference

[1] R3-247908, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT, Source: RAN3, To: RAN2

[2] R2-25xxxxx, Draft Running CR based on R2-2500918 - V08_Rapp, Ericsson

- RAN2 meeting agreements on MRO for CHO with candidate SCGs

RAN2#128

Agreements - RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements. - Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

RAN2#127bis

Agreements 1: Continue the work of MRO for LTM 1st priority. 2: Focus CHO with candidate SCGs (focus on agreed scenarios by RAN3) with 2nd priority 3: For Rel-18 leftovers, regarding SON for SDT, focus on RSRP and data volume in SON reports. Other enhancements (including the additional failure cause) shall be paused unless time permits. 4: Close the stage-2 work on MRO for MR-DC SCG failure. 5: Pause subsequent CPAC and wait if any for RAN3 LS. 6: Pause and wait if any for RAN3 LS for SON/MDT for Slicing and NTN. 7: Send an LS to RAN3 that RAN2 want to prioritize RAN3-based solutions on P5 and P6. 8: Pause and wait for RAN3 LS on MHI Enhancement for SCG Deactivation/Activation.

Agreements - UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS. - Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

RAN2#127

UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

RAN2#126

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

RAN2#125bis

RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.