3GPP TSG-RAN WG2 Meeting #129bis	R2-2502800

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.6.4

Source:	Samsung

Title:	Remaining issues of Conditional Intra-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In RAN2#127bis, 128 and 129 meetings, RAN2 made following agreements to support the Conditional intra-CU LTM, and RAN2 prepared the MAC/RRC running CRs [1][2] based on these agreements.

Agreements on C-LTM in RAN2#127bis

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Agreements on C-LTM in RAN2#128:

1.	The triggering condition of conditional LTM can be based on L3 measurement.

2.	CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

3.	The L1 execution condition of a candidate cell is associated to only one triggering event.

4.	For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

5.	To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

6.	The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

7.	For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

8.	The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

9.	The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

10.	Candidate cell TA is maintained by a new timer.

11.	For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

12.	For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

Agreements on C-LTM RAN2#129:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

In this contribution, we will further address remaining issues to support the conditional intra-CU LTM in detail.

Discussion

Issue 1: Beam(s) used for L3 based C-LTM

Based on the RAN2 agreement in RAN2#129 meeting, beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition in case of L1 based C-LTM but there was no agreement how to handle the L3 based C-LTM.

From our understanding, it is straightforward that the principle of the L1 based C-LTM to select the beam for RACH-less LTM with CG could be applied L3 based C-LTM case as well. It means that the C- LTM based on L3 measurements (condA3/condA5 events) can be triggered by the evaluation per candidate cell (i.e., with consolidation of multiple beams of the candidate cell), then UE could select the beam for RACH-less LTM with CG based on the any beam meets L3 based C-LTM execution condition.

Proposal 1: For L3 based RACH less C-LTM, UE select the beam for CG resource selection based on RSRP threshold similar to RACH less HO or CG SDT.

Issue 2: TAT handling for C-LTM

During RAN2#129 meeting, RAN2 made a lot of agreements regarding the TAT for C-LTM as below.

- CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell.

- If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid.

- If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM.

- The CLTM TAT is per candidate configuration.

- FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

RAN2 discussed following two issues but couldn’t reach the conclusion.

- Issue 1: How to handle the TAT for the target cell after C-LTM execution

- Issue 2: How to handle TATs for other candidate cells after C-LTM execution

For the first issue, we summarized the candidate options and RAN2 impact.

  - Option 1: Stop the TA timer associated with early TA for the target cell

    - Target cell has no information for the remaining duration of the new early TA timer exactly, and there could be some TA update in case of cell switch physically.

    - When UE stop the early TA timer for the target cell?

  - When UE send the first MAC PDU transmission using the configured grant to the target cell, UE stop the early TA timer for the candidate cell

    - When UE start the TA timer (i.e. PTAG TA timer) for the target cell?

  - When UE send the first MAC PDU transmission using the configured grant to the target cell, UE stop the early TA timer for the candidate cell

  - Option 2: C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell.

    - There are no critical reason to stop the running C-LTM TAT (for early sync) and the running C-LTM TAT is still valid based on the definition.

    - How to start the TAT for the target cell should be defined on top of this running CLTM TAT

  - Alt 2-1: The running C-LTM TAT could be act as the legacy TAT for the target cell.

  - -Alt 2-2: Define that UE can perform UL transmission when the TAT for the target cell is running

Based on above observation, the expected UE operation of each options is described:

Option 1: 1> when a conditional LTM cell switch procedure is triggered for a LTM candidate cell as in clause 5.y.3 or indicated by upper layer, and the ltm-Candidate-TimeAlignmentTimer associated with the LTM candidate cell is running: 2> apply the stored TA value associated with the LTM candidate cell for the PTAG as specified in clause 6.1.3.4x; 2> stop ltm-Candidate-TimeAlignmentTimer; 2> start the timeAlignmentTimer associated with the PTAG. Option 2-1: 1> when a conditional LTM cell switch procedure is triggered for a LTM candidate cell as in clause 5.y.3 or indicated by upper layer, and the ltm-Candidate-TimeAlignmentTimer associated with the LTM candidate cell is running: 2> apply the stored TA value associated with the LTM candidate cell for the PTAG as specified in clause 6.1.3.4x; 2> ltmCandidateTimeAlignmentTimer is considered as the timeAlignmentTimer associated with the PTAG. Option 2.2 Furthermore, when the timeAlignmentTimer(s) associated with all PTAG(s) and ltm-Candidate-TimeAlignmentTimer for SpCell is not running, CG-SDT procedure is not ongoing and Positioning SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell.

We think not stopping the running TAT for the target cell is simple and less specification impact.

Proposal 2: C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, RAN2 clarify the running TAT to act as the timeAlignmentTimer associated with PTAG.

For the second issue, the expected candidate solutions are similar with the first issue.When UE is connected in the source cell, UE could have multiple C-LTM TA values with running C-LTM TATs (for early UL sync). After UE triggers the C-LTM based on the execution condition, UE try to switch to the target cell by RACH-less CLTM. The easiest way to handle the TATs for other candidate cell is deleting the values but Rel-19 C-LTM support the simultaneous C-LTM, it could be helpful to keep the running C-LTM TAT for subsequent RACH-less C-LTM.

Proposal 3: C-LTM TAT for other candidate cell is not stopped upon C-LTM cell switch execution to the target cell if it is running.

Issue 3: RRC Others

As indicated by WID, the C-LTM is limited to the non-DC case. Thus, the C-LTM for SCG is not supported in Rel-19.

Proposal 4: RAN2 is kindly asked to confirm that C-LTM for SCG is not supported in Rel-19.

Even for Intra-CU CLTM, when C-LTM is configured, it is not clear whether all the LTM candidates need to be always C-LTM candidates also. Allowing a mix of LTM candidates and CLTM candidates will increase the flexibility of network deployment. For e.g. some candidate DUs in a CU can support only R18 LTM while some DUs can support C-LTM along with R18 LTM. This may also help the UE to reduce unnecessary evaluations, and will be also similar to the legacy Layer3 conditional mobility where every neighbour cell needn’t be a CHO candidate. On the other hand, with the current RRC structure and procedures for C-LTM in the running CR, this may need some additional ways to distinguish between CLTM candidate cells and candidates which are only LTM candidate cells in the ltm-Config, either by using CLTM-ExecutionConditions IE or by using a new flag. So RAN2 is requested to discuss it first.

Proposal 5: RAN2 is kindly asked to confirm if it is possible to have candidates supporting only LTM when CLTM is configured.

In RAN2#129 meeting, RAN2 made following agreement regarding the LTM failure recovery. It covers the case where the UE performs LTM failure recovery, if the LTM cell switch triggered due to C-LTM fails.

Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

As C-LTM can be triggered by RRC (i.e. triggered by L3 measurement) also, we think we need to add the case in the specification. See the below TP.

##### 5.3.5.8.3 T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure) ##### -- skipped 3> if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution triggered by an indication from lower layers or upon fulfilment of CLTM evaluation conditions based on L3 measurements as described in 5.3.5.18.6: 4> revert back to the UE configuration used in the source Pcell except for the PDCP state variables for SRB(s) associated to the MCG; 3> else: 4> revert back to the UE configuration used in the source Pcell; ##### 5.3.7.3 Actions following cell selection while T311 is running 1> if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers or upon fulfilment of CLTM evaluation conditions based on L3 measurements as specified in clause 5.3.5.18.6; and 1> if attemptLTM-Switch is configured; and 1> if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG: 2> perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

Proposal 6: Update the RRC running CR to support CLTM fast failure recovery triggered by L3 measurements. See the TP above.

Issue 4: RACH-less C-LTM for the cells in the same TAG

In CLTM, the UE can derive the TA value of one candidate cell (e.g., Cell A) based on the LTM candidate Timing Advance Command MAC CE. The RACH-less cell switch toward Cell A can be performed as long as the TAT for such cell is still running when the CLTM execution is triggered. However, for other cells belonging to the same TAG as Cell A (e.g., Cell B/C…), the UE cannot perform RACH-less cell switch. The reason is that the UE cannot know that Cell B/C is in the same TAG as Cell A.

Actually, if the UE can know the TAG information of each candidate cell, it has more chance to perform the RACH-less CLTM, i.e., it can perform RACH-less CLTM for the cells in the same TAG as long as the TA value of one cell is derived.

This is different from Rel18 design. In Rel18, the UE performs RACH-less cell switch when receiving the TA value via the cell switch command MAC CE. Thus, it needn’t to reuse the received TA value for other cells. However, in Rel19 CLTM, the UE determines the cell switch by itself even the TA value is not derived. If the UE can know more applicable cells for one received TA value, it can perform RACH-less cell switch to more cell, and speed up the cell switch procedure.

Proposal 7: RAN2 is kindly asked to discuss whether to support the RACH-less CLTM for the cells in the same TAG.

If the above proposal is agreeable, the gNB can inform the TAG information of each candidate cell to the UE when configuring the candidate cells. Moreover, such TAG information should be uniquely assigned for each candidate cell.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: For L3 based RACH less C-LTM, UE select the beam for CG resource selection based on RSRP threshold similar to RACH less HO or CG SDT.

Proposal 2: C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell, RAN2 clarify the running TAT to act as the timeAlignmentTimer associated with PTAG.

Proposal 3: C-LTM TAT for other candidate cell is not stopped upon C-LTM cell switch execution to the target cell if it is running.

Proposal 4: RAN2 is kindly asked to confirm that C-LTM for SCG is not supported in Rel-19.

Proposal 5: RAN2 is kindly asked to confirm if it is possible to have candidates supporting only LTM when CLTM is configured.

Proposal 6: Update the RRC running CR to support CLTM fast failure recovery triggered by L3 measurements. See the TP above.

Proposal 7: RAN2 is kindly asked to discuss whether to support the RACH-less CLTM for the cells in the same TAG.

References

[1]	R2-250xxxx		Running MAC CR for enhanced mobility Ph4	vivo	38.321	18.4.0		NR_Mob_Ph4-Core	Rel-19.

[2]	R2-250xxxx		Rapporteur RRC for inter-CU and conditional LTM	Ericsson	38.331	18.4.0		NR_Mob_Ph4-Core	Rel-19.