3GPP TSG-RAN WG2 Meeting #129bis										       R2-2502802 Wuhan, China, Feb. 7th – 11th, 2025

Source				:  CEWiT

Agenda item		:  8.1.2.3

Title	:  Discussion on Functionality-based LCM for Positioning Use Case

Document for		:  Discussion and Decision

# Introduction

In Rel 18 the application of AI/ML techniques to NR air interface has been studied and captured in TR 38.843 [1]. Based on this normative phase is agreed in Rel 19 with following objective.

Provide specification support for the following aspects:

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

- Beam management – DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In this contribution, LCM-related aspects of UE-sided models for positioning use cases are proposed to continue the discussion in RAN 2.

# Discussion

The following agreements were made during RAN2#125bis, RAN2#126, RAN2#127, RAN2#127bis, RAN2#128, and RAN2#129 meetings on the LCM for the positioning use case.

RAN2#125bis [2]

Agreements

For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs.

For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3.

Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

Support reactive reporting of UE-sided applicable functionality. The NW configures AI/ML functionalities via RRC/LPP message. FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality.

FFS how the two approaches will be specified and whether we can combine them into one procedure. FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed).

RAN2#126 [3]

Agreements for NW-side model

RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

Agreements for UE-side model

The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and 	understanding of the functionality.

Wait for RAN1 for associate ID discussion.

At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements.

RAN2#127 [4]

Agreements on definitions

Supported functionalities refer to functionalities that UE can 	indicate by using UE capability information (via RRC/LPP signalling).

Applicable functionalities refer to functionalities that the UE is ready to apply for inference.

Activated functionalities refers to functionalities already enabled for performing inference.

RAN2#127bis [5]

Agreements

The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline:

Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message.

Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side.

Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).

Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message.

Step 5: The LMF requests the inferred location information using the LPP request location information message.

Step 6: UE reports the inferred location using LPP provide location information message.

Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress).

Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress).

RAN2#128 [6]

Agreements

For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline.

Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF.

Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS.

As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause.  FFS if other fallback options are considered.

RAN2#129 [7]

Agreements

1: Introduce AI/ML positioning Case 1 as a new positioning method.

2: Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR.

3: FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

4: The content of error cause is discussed while drafting stage3 CRs.

5: As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

6: UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed.

This document focuses on case 1 – UE-based positioning with UE-side model, direct AI/ML positioning.

  - Training Phase

For the UE-sided training, the UE/OTT server/LMF performs the model training and monitoring. The OTT server can be within the UE. The OTT server is a server for training the models and may have repositories to store the training data and the trained model. It may have model monitoring functionality too. It shall provide subscriptions to a large number of UEs for training the data collected by all the subscribed UEs. These subscriptions can be managed in a network non-transparent manner or network transparent manner. In the case of the network non-transparent manner, the UE list is provided to the OTT by the network through the network function-based interface.

The UE-sided training is shown in Figure 1. The UE may perform the training based on the available data or the UE may request the LMF to provide the measurements to enable the data collection for the model training. This request may be a supplementary location service (LCS) request. The LMF may initiate the LPP capability exchange with the UE and accordingly configure the measurement operation such as positioning reference signal (PRS) measurements to the UE using the LPP ProvideAssistanceData or RequestLocationInformation message. The UE performs the channel measurements and accordingly trains the AI/ML model.

The LMF-sided training is shown in Figure 2. The LMF may perform the training for the UE-sided model. The LMF may initiate the LPP capability exchange with the UE and accordingly configure the measurement operation such as positioning reference signal (PRS) measurements to the UE using the LPP RequestLocationInformation message. The UE performs the channel measurements and sends the measurements to the LMF using the LPP ProvideLocationInformation message. The LMF trains the AI/ML model based on the UE measurements.

Figure 1: AI/ML Model Training at the UE

Figure 2: AI/ML Model Training at the LMF

For the ground truth label information at the UE, the UE may measure the labels to the data like the location of the UE (the absolute or relative to a predefined reference location, distance, angle in both horizontal and vertical directions, the LOS condition flag (Yes/No), and/or the time stamp measurement, etc.). If the UE is not able to estimate the labels or is not configured to estimate the labels, then upon the request from the UE (MO-LR), the LMF can estimate the labels and provide them to the UE as shown in Figure 3.

Figure 3: Ground Truth Label Collection at the UE from the LMF

Proposal 1: For the UE-sided inference model, RAN2 shall consider two approaches for the model training:

Option 1: UE-side model training

Option 2: LMF-side model training

Proposal 2: The UE shall request the ground truth label information from the LMF by invoking the MO-LR request.

# Conclusion

This contribution provides the following proposals for functionality-based LCM of the UE-sided model for the positioning use case.

Proposal 1: For the UE-sided inference model, RAN2 shall consider two approaches for the model training:

Option 1: UE-side model training

Option 2: LMF-side model training

Proposal 2: The UE shall request the ground truth label information from the LMF by invoking the MO-LR request.

# Reference

[1] TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”

[2] RAN2#125bis Chair Notes

[3] RAN2#126 Chair Notes

[4] RAN2#127 Chair Notes

[5] RAN2#127bis Chair Notes

[6] RAN2#128 Chair Notes

[7] RAN2#129 Chair Notes