3GPP TSG RAN WG2 Meeting #129bis	R2-2502805

Wuhan, China, Apr. 7th - 11th, 2025

Agenda Item:	8.9.2

Source:	ETRI

Title:		Discussion on Store and Forward satellite operation

Document for:	Discussion and Decision

1	Introduction

Store & Forward operation is a new feature to be supported in Rel-19 IoT NTN [1].

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In this paper, we would like to discuss RAN2 impacts for the support of S&F satellite operation and provide our view on them.

2	Discussion

2.1	Time information of transition from normal mode to S&F mode

In the previous meeting, RAN2 discussed the time information of transition from normal mode to S&F mode as follows.

Proposal 3 The time information can also be broadcasted to indicate when the current satellite operation mode will transit from normal mode to S&F operation mode. - Xiaomi thinks this is not needed as the UE should check if the cell is barred or not. - Google thinks this is useful to trigger cell reselection if it does not support S&F. Xiaomi thinks that if the cell still allows such UEs to access the cell then there is no need to trigger reselection - HW thinks that R19 UEs would not need this, T-service is sufficient. - CATT thinks this is not needed - ZTE thinks that an indication of the duration of normal mode (on top of the absolute time indication of S&F mode -> normal mode) would be more useful. Apple agrees. - CMCC thinks it’s better to use T-service - Nokia thinks this indication is not essential. Samsung agrees and also thinks we should not link this to T-service Can come back to this proposal

As shown above, RAN2 has not decided whether the time information of transition from normal mode to S&F mode is needed or not. From our perspective, the time information of transition from normal mode to S&F mode is needed because it can have beneficial effect on Rel-19 UEs that do not support S&F operation.

When the satellite operation mode changes from normal mode to S&F mode, the service link remains available but the feeder link becomes unavailable. Therefore, Rel-19 UEs that do not support S&F operation need to try to find a cell operating in normal mode just before the transition. In order for the try, the UEs are expected to initiate neighbour cell measurements and the time information of transition from normal mode to S&F mode can be used for the initiation. In addition, this time information is common to all the UEs and thus it needs to be broadcast.

If a satellite will transit from normal mode to S&F mode, time information of the transition is broadcast.

Rel-19 UEs supporting S&F operation need to identify whether a cell operating in normal mode can support S&F operation. For Rel-19 UEs supporting S&F operation, such identification may improve cell selection and reselection procedure. For example, when multiple cells broadcast their time information of transition from normal mode to S&F mode, a Rel-19 UE supporting S&F operation may prioritize a cell with a longer normal mode operation time for (re-)selection.

If the time information on the transition from normal mode to S&F mode is broadcast, it indicates that the current cell supports S&F operation and operates in normal mode until the time.

The time information of transition from normal mode to S&F mode is not essential information for Rel-19 UEs and we cannot find any significant reason for the time information to be in SIB1. SIB31 is better than SIB1 to conserve the capacity of SIB1.

The time information on the transition from normal mode to S&F mode is broadcast in SIB31.

There can be two options for the representation of the time information of transition from normal mode to S&F mode:

Option 1: absolute UTC time,

Option 2: the time duration of S&F mode.

Even with Option 2, the absolute time in Option 1 can be obtained. In the previous meeting [2], RAN2 agreed to use absolute UTC time for the introduced time information of transition from S&F mode to normal mode. Consequently, the absolute time in Option 1 can be obtained with Option 2 and the agreed time information. For example, if transition of (normal  S&F) occurs before transition of (S&F  normal), “the time of (normal  S&F)” = “the time of (S&F  normal)” – “the time duration in Option 2”.

However, we prefer Option 1. This is because it aligns with the absolute UTC time representation of “the time of (S&F  normal)” agreed in the previous meeting [2]. In addition, the time duration in Option 2 can be obtained by the time difference of “the time of (S&F  normal)” and “the time of (normal  S&F)”.

Use absolute UTC time to broadcast the time information on the transition from normal mode to S&F mode.

In order for Option 1 can be used for deriving the time duration of S&F mode, i.e., Option 2, not only “the time of (normal  S&F)” but also “the time of (S&F  normal)” need to be broadcast. The time duration of S&F mode may be used to improve cell selection and reselection procedure. For example, when a Rel-19 UE supporting S&F operation performs cell selection and reselection procedure and there are multiple cells that are operating in S&F mode, the UE may prioritize a cell with a shorter time duration of S&F mode among the cells.

NW broadcasts both time information of transitions from normal to S&F mode and vice versa.

One of remaining issues is whether we introduce a new parameter or reuse t-Service to indicate the time of transition from normal mode to S&F mode. We would like to point out that the current t-Service cannot be used for the indication.

Figure 1. Example S&F operation scenario in a quasi-Earth-fixed cell

An example S&F operation scenario is illustrated in Figure 1 and the details are as follows.

Initially, the cell is served by satellite #1

t1: feeder link with gateway #1 becomes unavailable

t2: feeder link with gateway #2 becomes available

t3: the serving satellite changed from satellite #1 to satellite #2

According to the current specifications, t-Service is described as “Time information on when an NTN cell is going to stop serving the area it is currently covering.”. Therefore, t-Service = t3 in Figure 1.

In case of a satellite with transparent payload, the serving cell is changed if feeder link is changed, which results in t-Service = the time of feeder link change. On the other hand, in case of regenerative payload, the serving cell is not changed even if feeder link is changed. The S&F operation is supported by regenerative payload with split MME or full CN during [t1, t2). Therefore, t-Service is not set to t1 or t2. The serving cell is changed at t3 because a new satellite, i.e., satellite #2, becomes the serving satellite, which results in t-Service = t3.

There may be an S&F operation scenario that t-Service is different from the time feeder link become unavailable.

As shown in Figure 1, t-Service may be different from “the time of (normal  S&F)”. Therefore, a new parameter is needed to indicate “the time of (normal  S&F)” other than t-Service.

t-Service may be different from the time of transition from normal mode to S&F mode.

The time of transition from normal mode to S&F mode is carried by a new parameter other than t-Service.

2.2	Release code

Another remaining issue relevant to S&F satelltie operation is where a new release code is needed when a cell is operating in S&F mode and a Rel-19 UE is released to idle mode. A new release code enables a Rel-19 UE to have a different UE behaviour under conditions where the UE released to idle mode by a cell operating in S&F mode. An example of the different UE behaviour is to stop monitoring paging message. However, a Rel-19 UE can identify such conditions by the presence of “S&F operation” indication in SIB1. Therefore, from our perspective, a new release code is not needed.

A new release code for S&F mode operation is not needed.

3	Conclusion

In this paper, the following observations are made:

Observation 1	There may be an S&F operation scenario that t-Service is different from the time feeder link become unavailable.

Observation 2	t-Service may be different from the time of transition from normal mode to S&F mode.

In this paper, we propose the followings:

Proposal 1	If a satellite will transit from normal mode to S&F mode, time information of the transition is broadcast.

Proposal 2	If the time information on the transition from normal mode to S&F mode is broadcast, it indicates that the current cell supports S&F operation and operates in normal mode until the time.

Proposal 3	The time information on the transition from normal mode to S&F mode is broadcast in SIB31.

Proposal 4	Use absolute UTC time to broadcast the time information on the transition from normal mode to S&F mode.

Proposal 5	NW broadcasts both time information of transitions from normal to S&F mode and vice versa.

Proposal 6	The time of transition from normal mode to S&F mode is carried by a new parameter other than t-Service.

Proposal 7	A new release code for S&F mode operation is not needed.

References

RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek, RAN #106, Dec. 2024.

R2-2501333, Report from session on NR NTN and IoT NTN, Session Chair (ZTE Corporation), RAN2 #129, Feb. 2025.