3GPP TSG-RAN WG2 Meeting #129bis		R2-2502807

Wuhan, China, Apr 7 – 11, 2025	Revision of R2-2501213

Agenda Item:	8.10.2.2

Source: 	LG Electronics

Title:  	MRO for CHO and Associated CPAC

Document for:	Discussion

Introduction

In this contribution, we focus on near failure case in the Rel-18 DCCA (i.e. CHO+CPAC) mobility MRO based on the last agreements.

Discussion

Based on a flow which we analysed CHO+CPAC flow shown in below:

Figure 1. CHO+CPC Logging Flow

This flow chart shows logging cases that can occur after the UE is configured with CHO+CPC. According to the flow chart, we can see that in all cases where CHO+CPC logging is required, except for clear success case, the CHO+CPC information can be reported to the network via an RLF report, SCG failure information or SHR (Successful Handover Report).

In other words, although PSCell change can be performed together, CPC is an additional condition based on CHO and the configuration is the same as CHO+SCG, so there is no need to consider SPR (Successful PSCell change or addition Report) for new logging information of the CHO+CPC.

Proposal 1: For CHO with associated CPA/CPC, the UE reports information for the near failure case to the network via SHR not SPR.

According to the previous agreements from RAN2#128 meeting, the UE performs logging of the SHR with the same contents as logging of the RLF-report. That is, the UE does not log the actual time when the first execution condition was satisfied.

However, since the UE does not log the actual time when the first execution condition was satisfied, the network cannot accurately predict the satisfaction times of the first and second execution conditions even if the network receives this time gap information. Without knowing this actual time, the network may not clearly identify the cause when the UE fails to perform mobility at the expected time for CHO+CPAC, and thus cannot appropriately update the execution condition configuration.

Thus, we propose the following for the SHR logging to resolve the above issue:

Proposal 2. For CHO with associated CPA/CPC, the UE should include the satisfaction time of the first execution condition in the SHR as part of the near failure information.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1: For CHO with associated CPA/CPC, the UE reports information for the near failure case to the network via SHR not SPR.

Proposal 2. For CHO with associated CPA/CPC, the UE should include the satisfaction time of the first execution condition in the SHR as part of the near failure information.